3GPP TSG-RAN WG4 Meeting # 104-bis-e 											        R4-2217232

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.6.6

Source:	vivo

Title:						WF on eDRX and RRM measurement relaxations requirements for Redcap UE

Document for:	Approval

Topic #1: Extended DRX enhancements

Sub-topic 1-1 Maintenance on eDRX requirements for Redcap

Issue 1-1-1: Issue when UE transitions from measurements within PTW and to measurements outside PTW or vice versa during one measurement period

- Proposals

  - Option 1: When UE transitions from measurements within PTW and to measurements outside PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement requirements before or after the transition.

  - Option 2: Not necessary to have option 1

Agreements: Option 1

Topic #2: RRM measurement relaxations

Sub-topic 2-1 Remaining issues on RRM measurment relaxation for Redcap

Issue 2-1-1:  Clarification on RRM relaxation applying conditions

- Proposals

  - Option 1: For the issue 2-1-3 in [R4-2215162], option 3 can be considered and the wording of option 3 could be updated.

    - Note: option 3 is “If the UE is configured with and has fulfilled the stationary and not-at-cell-edge criteria in sections 4.2B.2.10.3 and 4.2B.2.11.3 and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells”.

  - Option 2: RAN4 to proceed along option 1 for Issue 2-1-3, i.e. UE shall not relax measurements on any of the neighbour cells in case UE has failed to meet the S criterion.

  - Option 3: If the UE is configured with and has fulfilled multiple relaxation criteria that allows the UE to not measure for 4 hours and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells.

  - Option 4: For option 3, if the UE has satisfied not at cell edge criterion but not criterion S then the NW may need to adapt the threshold to make them aligned

Agreements: Option 3.

- Issue 2-1-2:  RRM measurement relaxation for SDT at inactive state

- Proposals

  - Option 1: RRM relaxation of neighbour cell measurements is allowed for SDT at inactive state and depends on whether configured relaxation criteria are met, e.g. whether single stationary criterion is met, single low mobility criterion is met, or stationary not-at-cell-edge criteria are met.

  - Option 2: option 1 is not necessary

  - Option 3: Up to UE implementation

Agreements: No

Topic #3 Others

Sub-topic 3-1 On offset to transmit CD-SSB and NCD-SSB at different times

Issue 3-1-1: Issue when NCD-SSB time offset = 5ms

- Proposals

  - Option 1: RAN4 to consider sharing mechanism to define the measurement requirement when the NCD-SSB offset configured with 5ms. (xiaomi)

  - Option 2: RAN4 to consider the scenario NCD-SSB time offset = 5ms and CD-SSB in a different BWP to NCD-SSB with MG for CD-SSB being applied. RAN4 to not further treat the scenario with NCD-SSB offset = 5ms, as there is no specification impact. Network can select appropriate MG configuration based on gap offset and MGTA to preclude UE having to drop MG assisted CD-SSB inter-frequency measurements. (Nokia)

  - Option 3: In RedCap, RAN4 to define UE behaviour when the MG and the SMTC meets the proximity condition with the time distance = 4ms; When the SMTC for intra-frequency layer is fully-partially overlapping with the MG due to NCD-SSB offset, UE is required to perform intra-frequency measurement and drop the configured MG. (Ericsson)

  - Option 4: Not necessary to consider this scenario (Huawei Apple Xiaomi oppo)

Agreements: No

Reference