3GPP TSG-RAN WG4 Meeting #105                                                                   R4-2218098

E-meeting, 14th – 18th November, 2022

Agenda item:				5.1

Source:				CableLabs, Charter Communications, Nokia

Title:			Justification for corrections for 100MHz CBW inclusion for n46, n96 and n102 in 38.104

Document for:			Approval

# Introduction

RAN4 agreed on adding the 100 MHz bandwidth for NR-U in Rel-17 and agreed on the 100 MHz NR-U channel raster and unwanted emissions [1]-[2]. The 100 MHz bandwidth has been added in TS 38.101-1. The 100 MHz bandwidth channel rasters in bands n46, n96 and n102 have been added in TS 38.104 via agreed CR R4-2208903 Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1. However, the 100 MHz bandwidth is not added in TS 38.104 V17.7.0 (2022-09) Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR2-1. As noted, high-lighted just before, there is also a mistake in the table header of the specification which is addressed with the same CR proposed. Further, the unwanted emission limit for the 100 MHz NR-U bandwidth is not address in TS 38.104 clause 6.6.4.2.4A Basic limits for Local Area and Medium Range BS for band n46, n96 and n102 (Category A and B). In CR R4-2218124 we propose to address above two missing items.

# Conclusion

Three changes for TS 38.104 V17.7.0 (2022-09) are proposed in CR R4-2218124.

- Adding 100 MHz NR-U bandwidth in Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR2-1.

- Correcting the table headers of Table 5.3.5-1 and Table 5.3.5-2 in TS 38.104 V17.7.0 (2022-09).

- Adding 100 MHz NR-U bandwidth unwanted emission limit in clause 6.6.4.2.4A Basic limits for Local Area and Medium Range BS for bands n46, n96 and n102 (Category A and B).

# References

[1] R4-2206447, “WF on adding 100 MHz channel BW in NR-U bands n46 and n96,” Qualcomm, RAN4 #102-e, Mar. 2022.

[2] R4-220xxxx, “Email discussion summary for [102-e][113] NR_bands_R17_BWs,” Ericsson, Mar. 2022.

[3] R4-22010254, “Email discussion summary for [103-e][119] NR_bands_R17_BWs,” Ericsson, May 2022.