3GPP TSG-RAN WG4 Meeting#105      	R4-2218278

Toulouse. France, 14 – 18 Nov, 2022

Title:	Reply LS to RAN5 LS on ModifiedMPR-Behaviour clarification for different power classes

Source:	Nokia

Agenda item:	10.3.2

Document for:	Approval

# 1	Introduction

This contribution discusses RAN5 LS on ModifiedMPR-Behaviour clarification for different power classes and proposes reply LS to RAN5.

# 2	Discussion

When defining test specification for FR2 MPR enhancements, RAN5 faced some concerns as listed below:

- For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability?

Proposed answer: RAN4 has agreed that if REL15 UE supports modifiedMPR-Behaviour bit 0 UE capability defined in REL16 specification it shall support latest REL16 requirements bit relates to unless new modified MPR-Behaviour bit is defined for same requirement the bit 0 relates to. See RAN4 CR [1].

- For Rel-15 PC2 and 4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: Yes it is.

- For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? Also, is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

Proposed answer: It is optional.

- For Rel-16 PC2, 4 and 5 UEs, is the PC3 MPR as defined in 38.101-2 v16.2.0 applicable? Also, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: Same as answer in c, optional.

- Is any kind of Rel-16 UE supposed to support MPR as defined in 38.101-2 version v16.11.0?

Proposed answer: No, see answer in a

- For Rel-17 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE signals modifiedMPR-Behaviour bit 0=true?

Proposed answer: Rel-17 38.101-2 Annex-H is not correct. It should state This bit SHALL be set to 1 instead of MAY and the REL17 UE supports MPR defined in REL17 specification. There is a RAN4 CR on this [2]

- For Rel-17 PC3 UE, what is the MPR requirement if the UE signals modifiedMPR-Behaviour bit 0=false?

Proposed answer: Rel-17 UE is not allowed to signal modifiedMPR-Behaviour bit 0=false

# 3	Conclusion

We ask RAN4 to approve the LS in Annex and agree the CRs [1].

# 4	References

[1] R4-2218280	CR R16 ModifiedMPR	Nokia

[2] R4-2218279	CR R17 ModifiedMPR	Nokia

# Annex: Draft LS

3GPP TSG-RAN WG4 Meeting#104-e

E-meeting, 15 – 26 Aug, 2022

Title:	Reply LS on ModifiedMPR-Behaviour clarification for different power classes

Response to:

Release:	Release 16

Work Item:	NR_RF_FR2_req_enh-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:	N/A

Contact Person:

#### Name:	Petri Vasenkari

E-mail Address:petri.j.vasenkari@nokia.com@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

[1] R4-2218280	CR R16 ModifiedMPR	Nokia

[2] R4-2218279	CR R17 ModifiedMPR	Nokia

1. Answers to RAN5 questions

- For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability?

Proposed answer: RAN4 has agreed that if REL15 UE supports modifiedMPR-Behaviour bit 0 UE capability defined in REL16 specification it shall support latest REL16 requirements bit relates to unless new modified MPR-Behaviour bit is defined for same requirement the bit 0 relates to. See RAN4 CR [1].

- For Rel-15 PC2 and 4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: Yes it is.

- For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? Also, is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

Proposed answer: It is optional.

- For Rel-16 PC2, 4 and 5 UEs, is the PC3 MPR as defined in 38.101-2 v16.2.0 applicable? Also, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: Same as answer in c, optional.

- Is any kind of Rel-16 UE supposed to support MPR as defined in 38.101-2 version v16.11.0?

Proposed answer: No, see answer in a

- For Rel-17 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE signals modifiedMPR-Behaviour bit 0=true?

Proposed answer: Rel-17 38.101-2 Annex-H is not correct. It should state This bit SHALL be set to 1 instead of MAY and the REL17 UE supports MPR defined in REL17 specification. There is a RAN4 CR on this [2]

- For Rel-17 PC3 UE, what is the MPR requirement if the UE signals modifiedMPR-Behaviour bit 0=false?

Proposed answer: Rel-17 UE is not allowed to signal modifiedMPR-Behaviour bit 0=false

Additionally, RAN4 has agreed two CRs related to this topic. One for Rel-16 [1] to clarify applicability of  MPR is UE signals modifiedMPR-Behaviour bit 0=true and [2] to make new MPR mandatory for Rel-17 UEs to signal modifiedMPR-Behaviour bit 0=true.

2. Actions:

To RAN5 group.

ACTION: 	RAN4 respectfully requests RAN5 group to discuss the answer and evaluate the RAN4 CRs and provide feedback if any.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN4 Meeting#105	 14h – 18th November 2022	Toulouse

TSG-RAN4 Meeting#106	 27th Feb – 3rd Mar 2023	Athens