3GPP TSG RAN WG4 #105 meeting 		R4-2218297

Toulouse, France, November 14th - 18th, 2022

### Agenda Item:	8.6.4.1

### Source:	Meta

### Title:	On MSD evaluation with new assumptions for MSD improvement

Document for:	Approval

# Introduction

In RAN4 #104-bis-e meeting, RAN4 concluded that lower MSD is feasible based on the feasibility results from interested vendor’s MSD results and approved the WF [1] on study for lower MSD which were discussed about the lower MSD as follow:

### Sub-topic 2-2: Feasibility of MSD improvement #### Issue 2-2-1: Whether it is feasible for MSD improvement <Agreement in GTW_Oct-14>: MSD improvement is feasible. #### Issue 2-2-2: Justification of lower MSD Summary of round 1 discussion The following options have been discussed in 1st round. But no majority view. Below which absolute MSD value, the improved MSD can be reported regardless of the number of thresholds? - Option 1: ≤ 15dB - Option 2: ≤ 18dB - Option 3: up to NW decision - Option 4: FFS # <Agreements> FFS in next RAN4 meeting # Topic #3: Study of signaling for Lower MSD (Agenda 6.6.4.2) ### Sub-topic 3-1: Network behaviour for the lower MSD #### Issue 3-1-1: What’s the supposed NW behaviour for the possible lower MSD capability Option 1: Further clarifications are needed on how network would handle UE with nominal or lower MSD differently before the consideration of UE capability introduction. (R4-2215667 Apple) Option 2: How NW handle the band combination configuration based on the MSD capability reporting is up to NW implementation. (R4-2216435 OPPO) # <Agreements> Further discuss the options in next meeting Sub-topic 3-2: MSD capability #### Issue 3-2-1: Whether to introduce the optional lower MSD UE capability based on the feasibility study of MSD improvement? Option 1: Yes Option 2: No Option 3: Other/FFS <Agreement in GTW_Oct-14>: - Optional lower MSD UE capability conditioned on what is intended for the signalling - FFS on design of signalling, including - Function of signalling - Granularity of signalling #### Issue 3-2-2: Granularity of the optional lower MSD UE capability Option 1: per victim band per MSD type per band combination Option 2: not to differentiate with different MSD types Option 3: Others Option 4: Single difference MSD value as the threshold according to the different MSD sources. But do not define the individual MSD levels for all CA/DC band combinations (Meta) <Agreement in GTW_Oct-19> - Use the following bullet as the starting point for granularity of the optional lower MSD UE capability - per victim band per MSD type per band combination #### Issue 3-2-3: Conditions to trigger the lower MSD reporting #### Issue 3-2-4: How to report the lower MSD capability for a BC with same MSD type but different orders #### Issue 3-2-5: How to report the lower MSD capability for Harmonic/cross band isolation with different test points #### Issue 3-2-6: Dynamic MSD reporting - Recommended WF for issues in sub-topic 3-2 Need further discussion when analyzing the details of the lower MSD capability. ### Sub-topic 3-3: Lower MSD threshold(s) #### Issue 3-3-1: Absolute MSD value/threshold(s) or relative threshold(s) #### Issue 3-3-2: Single value/threshold or multiple thresholds #### Issue 3-3-3: In case of single threshold, the proposed value #### Issue 3-3-4: In case of multiple thresholds, the proposed values #### Issue 3-3-5: Whether same lower MSD threshold(s) for different MSD types #### Issue 3-3-6: Predefined or NW configurable thresholds - Recommended WF for issues in sub-topic 3-3 Need further discussion when analyzing the details of the lower MSD capability. ### Sub-topic 3-4: Applicability of lower MSD capability #### Issue 3-4-1: Applicability of the lower MSD capability for power classes #### Issue 3-4-2: Applicability of Lower MSD capability for higher order combination #### Issue 3-4-3: Commonality of the lower MSD capability All companies are ok with the proposal. <Agreement>: One common capability report scheme should apply for all band combinations rather than only example BC. ### Sub-topic 3-5: Format of lower MSD capability #### Issue 3-5-1: How to report the lower MSD capability Option 1: Consider a joint solution of one bit low MSD indication per BC with additional optional MSD report for different interference types under the per BC indication. Option 2: Bit map and lower MSD classes per source - Recommended WF Need further discussion when analyzing the details of the lower MSD capability. ### Sub-topic 3-6: Reducing signalling overhead #### Issue 3-6-1: Methods to reduce the signaling overhead Option 1: The low MSD capability may be reported by the UE upon network query. The query can be filtered by the set of band combinations, the victim band, or the MSD type Option 2: Others Option 3: Leave it to RAN2 - Recommended WF Need further discussion when analyzing the details of the lower MSD capability. ### Sub-topic 3-7: Spec impact due to lower MSD capability #### Issue 3-7-1: How to reflect the lower MSD in RAN4 spec More companies prefer to have further discussion as it depends on the conclusion of other issues. Option 1: Explicit Lower MSD capability threshold(s) should be defined. Option 2: Others - Recommended WF Need further discussion when analyzing the details of the lower MSD capability.

In this paper, we provide our MSD evaluations based on the different MSD sources for the example CA band combinations.

# MSD evaluation results for the example CA band combinations

## MSD evaluation results by Harmonic problem

For CA_n1-n3-n78, the 2nd harmonic from n3 UL will fall into n78 spectrum in DL slot. Based on the following RF parameters in Table 1, the expected MSD levels are evaluated in Table 2.

Table 1. NR CA_n3A-n78A UE RF FE component isolation parameters

|  | Attenuation Value |
| --- | --- |
| n3 Tx in PA output | 28 |
| n3 PA H2 attenuation | 35/ 40 |
| n3 duplexer H2 attenuation | 32 |
| Harmonic filter | 20/25 |
| HB switch H2 | 110 |
| Diplexer attenuation | 25 |
| Antenna isolation | 10/15 |
| Diplexer pathloss | 0.7 |
| UHB switch attenuation | 0.7 |
| UHB switch | 130/110 (primary/secondary) |
| n78 Rx filter attenuation | 1.5 |
| n78 Rx filter | 110/110 (primary/secondary) |
| n3 PA to n78 LNA isolation | 70/75 |

The Table 2 is shown the expected MSD levels for NR CA_n3A-n78A.

Table 2. Comparison of MSD of NR CA_n3A-n78A with different RF components

| Parameter | Considering different Filter/isolation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Primary |  | Diversity |  |
|  | Value | H2 level | Value | H2 level |
| n3 Tx in PA output | 28 |  | 28 |  |
| n3 PA H2 attenuation | 35 | -7 | 35 | -7 |
| n3 duplexer H2 attenuation | 32 | -39 | 32 | -39 |
| Harmonic filter | 20/25 | -59 /-64 | 20/25 | -59 /-64 |
| HB switch H2 | -110 | -59/ -64 | -110 | -59/ -64 |
| Diplexer attenuation | 25 | -84/ -89 | 25 | -84/ -89 |
| Antenna isolation | 0 | -84/ -89 | 10/15 | -94/ -99 |
| Diplexer pathloss | 0.7 | -84.7/ -89.7 | 0.7 | -94.7/ -99.7 |
| UHB switch attenuation | 0.7 | -85.4/ -90.4 | 0.7 | -95.4/ -100.4 |
| UHB switch | -130 | -85.4/ -90.4 | -110 | -95.2/ -99.9 |
| n78 Rx filter attenuation | 1.5 | -86.9/ -91.9 | 1.5 | -96.7/ -101.4 |
| n78 Rx filter | -110 | -86.9/ -91.8 | -110 | -96.5/ -100.9 |
| n3 PA to n78 LNA isolation | 70/75 | -77/ -82 | 70/75 | -77/ -82 |
| Composite |  | -76.85 – -81.54 |  | - 76.98 – -81.93 |
| MSD (dB) | 18.5dB – 23.4dB |  |  |  |

Observation #1: Antenna isolation, attenuation of harmonic filter and PA H2 performance improvement are not dominant factors for the 2nd harmonic product to reduce the MSD requirements.

Observation #2: The aggressor PA to victim LNA isolation improvement is a dominant factor due to the 2nd Harmonic product to improve MSD levels.

Proposal #1: For CA_n3-n78 band combination which has a 2nd harmonic problem, MSD improvement is feasible in case of an enhancement of the aggressor PA to victim LNA isolation level. However, the expected MSD improvement is small compared to the existing MSD requirements.

## MSD evaluation results by cross band isolations

For CA_n41-n77, expected MSD levels are evaluated in Table 4 based on the common RF parameters are in Table 3.

Table 3 Basic RF parameters for CA_n41A_n77A

| Band n77 PA noise @Band 41 Rx (dBm/Hz) | -115 |
| --- | --- |
| CBW at n77 for UL | 100MHz |
| n77 front end filter Loss (dB) | 4 |
| n41 front end filter Loss (dB) | 4 |
| PA to antenna and LNA to antenna IL (dB) | 4 |
| n41/n77 diplexer isolation (dB) | 10/15 |
| n77 filter attenuation @ B41 RX (dB) | 30 |
| Cross-band isolation (dB) (n77 Tx --> n41 Rx) | 40/45 |
| Rx Antenna isolation (dB) | 10/15 |

Table 4. Comparison of MSD of NR CA_n41A-n77A with different RF components

| Parameter | Primary | Diversity |
| --- | --- | --- |
|  | Value | Value |
| Total noise at PA output (dBm/Hz) | -148 | -148 |
| Antenna isolation (10dB/ 15dB) | -148 | -158 /-163 |
| Rx thermal noise @ LNA input (dBm) | -84 | -88.2 / -92.5 |
| Total Interference @ Rx | -82.4 | -87.0/-91.2 |
| MRC | -88.3 / -91.7 |  |
| REFSENS at n41 (dBm/CBW) | -95.1 |  |
| MSD (dB) | 3.4 – 6.8 dB |  |

Observation #3: For the MSD improvement due to cross band isolation, a larger antenna isolation could improve a required MSD level, but this is difficult to achieve in a small form-factor and not verifiable by conducted tests.

Observation #4: An RB restriction (e.g., restriction of RB length or RB position) also could reduce MSD levels. A specific uplink configuration will improve MSD levels.

Proposal #2: RAN4 can consider specific uplink configuration(s) such as RB length or RB position, or both to reduce the MSD level in case of cross-band isolation.

RAN4 had agreed to use the terminology of “ACRL1” and “ACLR2”. However, this terminology could confuse with UTRA ACLR1 and UTRA ACLR2, unless there is clear indication or explanation. To avoid such an ambiguity, it is necessary to have a separate indication rather than just ACLR1 or ACLR2.

Proposal #3: To avoid confusion, RAN4 needs to define a terminology to use NR_ACLR1 and/or NR_ACLR2 to indicate a cross-band interference source(s), instead of using just ACLR1 and/or ACLR2.

## MSD evaluation results by dual uplink transmission

For CA_n1-n3-n78, the 2nd & 4th IMDs products by 2UL_CA_n1-n3 are falling into n78 DL slot. Also 2nd IMD product by 2UL_CA_n3-n78 is falling into n3 Rx band. Therefore, we consider the specific RF component improvement to enhance the MSD requirements in Table 5 and Table 6.

Table 5 shows the RF component isolation parameters to derive MSD level for CA_n1A-n3A-n78A.

Table 5: UE RF Front-end component parameters

| UE ref. architecture Component | Cascaded Diplexer-Duplexer Architecture w/ single ant. or dual ant. |  |  |  |
| --- | --- | --- | --- | --- |
|  | CA_n1A-n3A-n78A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 110 | 65 | 55 | 45 |
| Triplexer | 110 | 72 | 55 | 45 |
| Diplexer | 112 | 85 | 55 | 45 |
| Duplexer | 95 | 75 | 55 | 45 |
| PA Forward | 27 | 30 | 28 | 27 |
| PA Reversed | 38 | 28 | 33 | 32 |
| LNA | 5 | -5 | -5 | -10 |

Table 6 shows the isolation levels based on the RF component in Table 5.

Table 6: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10/15 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60/70 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | L-L/H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | L-L/H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |
| Rx filter attenuation | 38 | n3 filter rejection over n78 |

Table 7: Comparison of MSD in n3 Rx band of NR CA_n3A-n78A UE with different RF parameters

|  | Thermal | IMD w/ ant. ISO 10dB, PCB 60dB | IMD w/ ant. ISO 10dB, PCB 60dB | IMD w/ ant. ISO 10dB, PCB 70dB | IMD w/ ant. ISO 15dB, PCB 70dB |
| --- | --- | --- | --- | --- | --- |
| Main Path (dBm) | -93 | -57.2 | -57.2 | -59.8 | -59.8 |
| Diversity Path (dBm) | -93 | -67.1 | -71.9 | -69.4 | -74.1 |
| After MRC (dBm) | -96 | -67.5 | -72.0 | -69.8 | -74.2 |
| MSD (dB) |  | 28.5 | 24.0 | 26.2 | 21.8 |

Considering unit-to-unit variation in mass productions, it is necessary that more than 80 dB PCB isolation is supposed to be guaranteed, i.e., > 85 dB PCB isolation. In our view, it is excessive and overly tightens the implementation complexity.

Observation #5: For the MSD improvement by dual uplink transmission, 10 dB or less improvements are feasible by enhancing antenna isolation and PCB isolation. However, 80 dB PCB isolation and 20 dB antenna isolation are quite challenging to achieve in a smart phone form-factor.

Proposal #4: RAN4 could improve the MSD level by enhancing antenna isolation and PCB isolation level. However, the expected MSD improvement is less than 10 dB for smart phone form-factors.

# Conclusions

In this contribution, we provided the expected MSD improvement levels for the different MSD sources and provided our view on how to report the MSD levels, how to apply the MSD thresholds for the different power classes, and how to specify the lower MSD capability in RAN4 specifications.

Based on the above evaluation results and proposals in session 2 and 3, we proposed as follows:

Observation #1: Antenna isolation, attenuation of harmonic filter and PA H2 performance improvement are not dominant factors for the 2nd harmonic product to reduce the MSD requirements.

Observation #2: The aggressor PA to victim LNA isolation improvement is a dominant factor due to the 2nd Harmonic product to improve MSD levels.

Observation #3: For the MSD improvement due to cross band isolation, a larger antenna isolation could improve a required MSD level, but this is difficult to achieve in a small form-factor and not verifiable by conducted tests.

Observation #4: An RB restriction (e.g., restriction of RB length or RB position) also could reduce MSD levels. A specific uplink configuration will improve MSD levels.

Observation #5: For the MSD improvement by dual uplink transmission, 10 dB or less improvements are feasible by enhancing antenna isolation and PCB isolation. However, 80 dB PCB isolation and 20 dB antenna isolation are quite challenging to achieve in a smart phone form-factor.

Proposal #1: For CA_n3-n78 band combination which has a 2nd harmonic problem, MSD improvement is feasible in case of an enhancement of the aggressor PA to victim LNA isolation level. However, the expected MSD improvement is small compared to the existing MSD requirements.

Proposal #2: RAN4 can consider specific uplink configurations such as RB length and RB position, or both to reduce the MSD level in case of cross-band isolation.

Proposal #3: To avoid confusion, RAN4 needs to define a terminology to use NR_ACLR1 and/or NR_ACLR2 to indicate a cross-band interference source(s), instead of using just ACLR1 and/or ACLR2.

Proposal #4: RAN4 can improve the MSD level by enhancing antenna isolation and PCB isolation level. However, the expected MSD improvement is less than 10 dB for smart phone form-factors.

# Reference

[1] R4-2217723, “WF on study for lower MSD,” Huawei

[2] RP-2217779, “Moderator summary for discussion [104-bis-e][127] FR1_enh2_part1,” Moderator (Huawei)

[3] R4-2215379, “Investigation of band combinations for MSD reduction,” Qualcomm

[4] R4-2215666, “Further analyses and views on MSD improvement for inter-band CA/DC,” Apple

[5] R4-2216117, “Analysis on improve MSD,” vivo

[6] R4-2216776, “Further discussion on the feasibility of improving MSD,” Huawei

[7] TR37.863-01-01 v15.4.0

[8] TS38.101-1

[9] TS38.101-3