3GPP RAN WG4 Meeting #105		R4-2218303

Toulouse, France, November 14 – 18, 2022

Agenda item:	12

Source:	Globalstar, Apple

Title:	Motivation for a new WID on Introduction of the Satellite L-/S-Band

WI/SI:

Release:	Rel-18

Document for:	Information

# 1	Introduction

After the RAN#86 meeting, a new WI was approved that aims at supporting 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – where were added as bands n255 and n256, respectively. At the same time there exist other satellite deployments and, as was presented during the RAN4#99 meeting, some deployments use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

This discussion paper presents further information and motivation for introduction of a new NTN band with the DL part on the S-band (2500MHz) and the UL part on the L-band (1600MHz).

# 2	NTN L-/S-band

To provide context for the new NTN band plan discussion, Table 2-1 below presents an overview of global frequency allocations for mobile satellite service (MSS) and regulatory related parameters, such as frequency ranges, direction and ITU regions based on the ITU regulation [2]. For the sake of convenience, we also present the total bandwidth for contiguous satellite frequency ranges.

Table 2-1: Regulatory parameters of the L-band and S-band

| Band | Frequencies (MHz) | Direction | Total BW (MHz) | Regions |
| --- | --- | --- | --- | --- |
| L-band | 1518-1559 | Space to Earth (DL) | 41MHz (DL) | 1, 2, 3 |
|  | 1610-1613.8 | Earth to Space (UL) | 3.8MHz (UL) |  |
|  | 1613.8-1626.5 | Earth to Space (UL) Space to Earth (DL) | 12.7MHz (UL/DL) |  |
|  | 1626.5-1660.5 | Earth to Space (UL) | 34MHz (UL) |  |
|  | 1668-1668.4 | Earth to Space (UL) | 7MHz (UL) |  |
|  | 1668.4-1670 | Earth to Space (UL) |  |  |
|  | 1670-1675 | Earth to Space (UL) |  |  |
| S-band | 1980-2010 | Earth to Space (UL) | 30MHz (UL) | 1, 2, 3 |
|  | 2010-2025 | Earth to Space (UL) | 15MHz (UL) | 2 |
|  | 2160-2170 | Space to Earth (DL) | 10MHz (DL) | 2 |
|  | 2170-2200 | Space to Earth (DL) | 30MHz (DL) | 1, 2, 3 |
|  | 2483.5-2500 | Space to Earth (DL) | 16.5MHz (DL) | 1, 2, 3 |
|  | 2500-2520 | Space to Earth (DL) | 20MHz (DL) | 3 |
|  | 2670-2690 | Earth to Space (UL) | 20MHz (UL) | 3 |

As mentioned in the Introduction part, RAN WG4 added support for the L-band and S-band referred to as NTN bands n255 and n256 respectively (see Table 2-2 below). These two bands cover some of the frequency allocations used by the satellite operators, but it does not cover all of them. In fact, band n255 has both DL and UL parts in the L-band, and band n256 has both parts in the S-band. However, there is another pairing of frequencies used by satellite operators, whereupon the L-band is used for UL and the S-band is used for the DL operation.

Table 2-2: Existing NTN bands as defined in TS 38.101-5.

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

There is another potential pairing involving UL block 1610–1626.5MHz from the L-band and the DL block 2483.5–2500MHz from the S-band. Referring specifically to the UL block, ITU-R regulations allocate 1610-1613.8MHz for the UL operation. At the same time, the 1613.8-1626.5MHz block can be used for the UL operation on the primary basis and the DL operation on the secondary basis.

Proposal:	Introduce support for a new NTN band with 1610 – 1626.5MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the corresponding NTN work item.

# 3	Conclusions

- In this contribution we have provided motivation for adding support for a new NTN band to the 3GPP core specifications. While the 3GPP specifications already have bands n255 and 256 – which support L-band and S-band – there exist deployments that have a mixed L-/S-band pairing, where UL resides on L-band and DL is on the S-band. Thus, we ask 3GPP to approve the corresponding WI to enable this pairing.

Proposal:	Introduce support for a new NTN band with 1610 – 1626.5MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the corresponding NTN work item.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- ITU-R, "Radio regulations articles", edition of 2020