3GPP TSG-RAN WG4 Meeting #105	R4-2218310

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	6.3.6.2.2

Source:	Ericsson

Title:	Simulation results for PDCCH and PBCH demodulation in FR2-2

Document for:	Information

# 1	Introduction

In this paper, we present the simulation results on the PDCCH and PBCH demodulation performance in the frequency range 52.6 GHz to 71 GHz. Following the WF [1], we carried out our simulations using the agreed propagation environments during RAN4 meeting #104-e (see Appendix A.1).

For PDCCH: Test number; CBW/ SCS; CORESET duration; CORESET RB; Interleaver Size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 100/120; 1; 60; 3; 2; 2; TDLA30-200; 1_0; 40; 1x2 Low. 2; 100/120; 1; 60; 2; 6; 4; TDLA30-650; 1_1; 56; 1x2 Low. 3; 100/120; 1; 60; 3; 2; 8; TDLA30-200; 1_1; 56; 2x2 Low. 4; 100/120; 2; 60; 3; 2; 16; TDLA30-650; 1_0; 40; 2x2 Low. 5; 400/480; 1; 60; 2; 6; 4; TDLA10-200; 1_1; 56; 1x2 Low. 6; 400/480; 1; 60; 2; 6; 8; TDLA10-200; 1_1; 56; 1x2 Low. 7; 400/480; 2; 60; 3; 2; 16; TDLA10-200; 1_0; 40; 2x2 Low. For PBCH: Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS) - Proposals - Option 1: TDLA30-200; - Option 3: TDLA30-650; Interested companies are encouraged to provide alignment and impairment results for both options in RAN4#105. Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; Unknown. 1-2; 480kHz/400MHz; TDLA10-200; New; Unknown.

# 2	PDCCH Simulation Results

## 2.1	Simulation setup

As per WF [1], and for the sake of completeness, we carried out simulations using TDLA10/30-200/650 and TDLD10-200 channel models. The simulation parameters are given in the Table below

Test Parameters for 100 MHz/120 kHz (reference from TS 38.101-4 Table 7.3.2.2-1)

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Test parameters for 400 MHz/480 kHz

| Parameter |  | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR2.480-1 |  |
| CCE to REG mapping type |  |  | Interleaved |  |
| REG bundle size |  |  | 6 | 2 |
| Interleaver size |  |  | 2 | 3 |
| Shift index |  |  | [0] |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Every 4th slot |  |
|  | CSI Periodicity | Slots | [640] |  |
| CSI-RS for tracking | CSI-RS offset | Slots | [320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4] |  |
| NZP CSI-RS for beam refinement | CSI-RS periodicity | Slots | [640 for CSI-RS resource 1,2] |  |

Furthermore, we used the following general parameters:

- Numerologies: 120 KHz/100 MHz and 480 KHz/400 MHz.

- Max Doppler frequency/DMRS: f_D = 200 Hz and f_D = 650 Hz, DMRS 1+1

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

## 2.2	Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%.

SCS 120 kHz/CBW 100 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | 1_0 | 40 | TDLA30-200 | 3.52 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA30-650 | 1.14 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA30-200 | -2.38 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA30-650 | -6.29 |

Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [9.8] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

SCS 480 kHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-5 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.27 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -1.5 |
| Test 1-6 | 3 | 2 | 60 | 1 | 8 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | -1.49 |
| Test 1-7 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA10-200 | -6.35 |

Observation 2: For SCS 480 kHZ/CBW 400 MHz and referring to the max DL testable SNR at [2.6] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters, except for AL4 under TDLA10-200.

# 3	PBCH Simulation Results

## 3.1	Simulation setup

As per WF [1], we carried out simulations under TDLA10/30-200/650 and TDLD10-200 channel models. Furthermore, we considered the following table where it is worth noting that we kept same target coding rate for both numerologies, 120 KHz SCS and 480 KHz SCS.

Table A.3.4.2-1: PBCH Reference Channel

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | R.PBCH.5 | R.PBCH.6 |
| SS/PBCH block subcarrier spacing | KHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

## 3.2    Simulation results

In this section, we present the PBCH performance in terms of the probability of miss-detection of the PBCH (Pm-bch), given by

Pm-bch=1-AB

where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs, as function of SNR (dB). The Pm-bch is derived with the assumption that UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms). Therefore, we have 4 transmissions for each MIB as per the following parameters’ table.

Table (TS 38.101-4 7.4.2.2-1): Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | Ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Observation 3: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

The following tables gather the obtained results.

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz | 1x2 Low | TDLA30-200 | -6.8866 | unknown |
|  |  |  | TDLA30-650 | -6.9581 |  |

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-2 | 480 KHz | 1x2 Low | TDLA10-200 | -6.3639 | unknown |

Observation 4: The reference SNRs for 120 KHz and 480 KHz SCS, where the SS/PBCH index is unknown, have been provided. These values confirm their conformance with the max DL testable SNR levels set at [9.8] dB and [2.6] dB, respectively.

# 6	Summary

In this paper, we carried out simulations for PDCCH and PBCH in FR2-2. Here we propose some observations that need further discussions:

Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [9.8] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters.

Observation 2: For SCS 480 kHZ/CBW 400 MHz and referring to the max DL testable SNR at [2.6] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters, except for AL4 under TDLA10-200.

Observation 3: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

Observation 4: The reference SNRs for 120 KHz and 480 KHz SCS, where the SS/PBCH index is unknown, have been provided. These values confirm their conformance with the max DL testable SNR levels set at [9.8] dB and [2.6] dB, respectively.

# References

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

# Appendix	Simulation Setup and Results

## A.1	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |