3GPP TSG-RAN WG4 Meeting # 105	R4-2218322

Toulouse, France, 14 – 18 November 2022

Agenda Item:	6.3.3.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The revised work item for extending current NR operation to 71 GHz was approved at TSG RAN#97-e [1]. One of the objectives of this work item is to define BS conformance testing requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. This topic was discussed at TSG RAN4#104-bis-e and the WF was agreed [2-4].

This contribution provides proposals on the measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz based on the agreed WF in [2-3] to complete the perf part of work item as targeted at TSG RAN#98.

# 2.	Discussion

It was agreed in [2]:

- For FR2-2 follow MU evaluation approach described in TR 37.941, sub-clause 5.2 in principle.

  - Target to generate a list for the components (which can be maintained and which need to be updated) by this meeting

  - TE vendors feedback is encouraged

- Whether additional calibration procedure can be considered subject to companies’ further input, RAN4 shall focus on the MU values based on companies’ individual input.

- MU values of the following components (described in TR 37.941, annexes A to C) needs to be updated:

  - A2-5 Mismatch of receiver chain (i.e. between receiving antenna and measurement receiver (a) / low power receiver (b))

  - A2-19 Uncertainty of the LNA (FR2 only)

  - A2-20 Uncertainty of the mixer, up/down converter (FR2 only)

  - B2-5 Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS)

  - C1-1 Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor)

  - C1-2 Uncertainty of the RF signal generator

  - C1-3 Uncertainty of the network analyser

  - C1-7 RF power measurement equipment (e.g. spectrum analyser, power meter) - low power (UEM, absolute ACLR)

  - C1-8 RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR)

  - C1-9 RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2

  - MUOOBint Additional uncertainty for the OOB interferer signal

  - MUPA Uncertainty due to use of PA

NOTE 1:	This list focuses on CATR.

NOTE 2:	Other MU contributors may still be added to this list in next meeting, if technically justified.

- MU values of other components (described in TR 37.941, annexes A, B and C) can be maintained (i.e., no need to update 37 < f < 43.5 GHz for FR2-2).

Moreover, it was agreed in [3] to use the table below as baseline to collect MU values:

Table 2-1: Standard test equipment measurement uncertainty

| UID | Uncertainty source |  | Measurement uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test equipment |  | 3 < f < 6 (GHz) | 24.25 < f ≤ 29.5 GHz | 37 < f < 43.5 (GHz) | 43.5 < f < 60 (GHz) |
| C1-3 | Uncertainty of the Network Analyzer |  | 0.20 (Note 2) | 0.30 | 0.30 | 0.30 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor) |  | 0.14 to 0.26 (Note 2) | 0.50 | 0.70 |  |
| C1-7 | Unceratinty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) for IF used with mixer |  |  |  |  | 0.60 (Note 3) |
| A2-20 | Uncertainty of the mixer (FR2 only) |  |  |  |  | 2.25 (Note 3, Note 6) |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – low power (UEM absolute ACLR) |  |  | 0.90 | 0.90 |  |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – relative (ACLR) |  |  | 0.75 | 0.90 |  |
| C1-9 | Uncertainty of the RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 |  |  | 1.25 | 1.45 |  |
| C1-2 | Uncertainty of the RF Signal Generator |  | 0.46 (Note 2) | 0.90 | 0.90 | 43.5 < f < 48.2 (GHz) 1.60 (Note 4) |
| MUOOBint | Additional uncertainty for the OOB interferer signal |  |  |  |  | 1.1 (Note 5) |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.30 (Note 7) | 0.40 (Note 7) | 0.40 (Note 3) |
| A2-5b | Mismatch of receiver chain for low power receiver |  |  | 0.51 (Note 8) | 0.51 (Note 8) |  |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) |  |  | 0.30 | 0.40 |  |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  |  |  | 0.00 (Note 9) |
| MUPA | MUPA Uncertainty due to use of PA |  |  |  |  | 0.2 (Note 5) |
|  | OTA modulation quality |  |  | 1% | 1% | 1% (Note 10) |
|  |  | Note 1: Values used for FR2-1 MU evaluation in TR 37.941. Note 2: Values used for FR1 MU evaluation in TR 37.941. Note 3: Values are from FR2-1 Tx spurious emission measurement MU budget table in TR 37.941 Note 4: Value is from 47 GHz band, see discussion note (R4-2115779) Note 5: Value is from Table 14.2.4-1 (TR 37.941) out of band blocking FR2-1, uncertainty of OOB blocking interferer signal which is CW signal and uncertainty from PA. Note 6: The applicability of this value for FR2-2 is FFS. Note 7: Values are from FR2-1 EIRP and other MU table in TR37.941 Note 8: Value is from FR2-1 Tx Off power measurement and other MU table in TR37.941 Note 9: Value is from FR2-1 Tx spurious emission measurement in TR37.941. Note 10: Although this value is different in category (value itself represents test MU not contributor), for FR2-2, there is possible increase of EVM MU due to wider BW. |  |  |  |  |

We have the following proposals for the MU values in the table above:

1.	For the frequency range, currently the frequency band allocations for mobile service within the FR2-2 (52.6 < f < 71 (GHz)) are not clear yet, so we propose not to divide FR2-2 into two sub-ranges at 60 GHz breakpoint as this would create complexity for equipment testing if the operating band cross over 60 GHz.

2.	For UID which have values in the ‘43.5 < f < 60 (GHz)’ column (i.e., C1-3, C1-7, A2-20, C1-2, MUOOBint, A2-5a, MUPA), we propose to use the values also for 60 < f < 70 (GHz), as the values are either already relaxed compared to the ones in the ‘37 < f < 43.5 (GHz)’ column or the same values are used for frequency ranges in FR2-2.

3.	For UID which have values in the ‘43.5 < f < 60 (GHz)’ column (i.e., C1-3, C1-7, A2-20, C1-2, MUOOBint, A2-5a, MUPA), we propose to apply the same step size comparing the values in the ‘24.25 < f ≤ 29.5 GHz‘ and ‘37 < f < 43.5 (GHz)’ columns, based on the general trend of implementation complexity as the target frequency increases in FR2-2.

Our proposals are summarized in the table below (changes compared to the table above are highlighted).

Table 2-1: Standard test equipment measurement uncertainty

| UID | Uncertainty source |  | Measurement uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test equipment |  | 3 < f < 6 (GHz) | 24.25 < f ≤ 29.5 GHz | 37 < f < 43.5 (GHz) | 43.5 < f < 71 (GHz) |
| C1-3 | Uncertainty of the Network Analyzer |  | 0.20 (Note 2) | 0.30 | 0.30 | 0.30 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor) |  | 0.14 to 0.26 (Note 2) | 0.50 | 0.70 | 0.90 |
| C1-7 | Unceratinty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) for IF used with mixer |  |  |  |  | 0.60 (Note 3) |
| A2-20 | Uncertainty of the mixer (FR2 only) |  |  |  |  | 2.25 (Note 3, Note 6) |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – low power (UEM absolute ACLR) |  |  | 0.90 | 0.90 | 0.90 |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – relative (ACLR) |  |  | 0.75 | 0.90 | 1.05 |
| C1-9 | Uncertainty of the RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 |  |  | 1.25 | 1.45 | 1.65 |
| C1-2 | Uncertainty of the RF Signal Generator |  | 0.46 (Note 2) | 0.90 | 0.90 | 43.5 < f < 48.2 (GHz) 1.60 (Note 4) |
| MUOOBint | Additional uncertainty for the OOB interferer signal |  |  |  |  | 1.1 (Note 5) |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.30 (Note 7) | 0.40 (Note 7) | 0.40 (Note 3) |
| A2-5b | Mismatch of receiver chain for low power receiver |  |  | 0.51 (Note 8) | 0.51 (Note 8) | 0.51 |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) |  |  | 0.30 | 0.40 | 0.50 |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  |  |  | 0.00 (Note 9) |
| MUPA | MUPA Uncertainty due to use of PA |  |  |  |  | 0.2 (Note 5) |
|  | OTA modulation quality |  |  | 1% | 1% | 1% (Note 10) |
|  |  | Note 1: Values used for FR2-1 MU evaluation in TR 37.941. Note 2: Values used for FR1 MU evaluation in TR 37.941. Note 3: Values are from FR2-1 Tx spurious emission measurement MU budget table in TR 37.941 Note 4: Value is from 47 GHz band, see discussion note (R4-2115779) Note 5: Value is from Table 14.2.4-1 (TR 37.941) out of band blocking FR2-1, uncertainty of OOB blocking interferer signal which is CW signal and uncertainty from PA. Note 6: The applicability of this value for FR2-2 is FFS. Note 7: Values are from FR2-1 EIRP and other MU table in TR37.941 Note 8: Value is from FR2-1 Tx Off power measurement and other MU table in TR37.941 Note 9: Value is from FR2-1 Tx spurious emission measurement in TR37.941. Note 10: Although this value is different in category (value itself represents test MU not contributor), for FR2-2, there is possible increase of EVM MU due to wider BW. |  |  |  |  |

The MU value for each requirement can then be calculated using the corresponding spreadsheet in TR 37.941 [5]. For example, the MU value for the reference sensitivity level requirement can be calculated as shown in the table below. Note that some of the MU values in the ‘37 < f < 43.5 GHz’ column are copied to the ‘43.5 < f < 60 (GHz)’ column except those MU values in the table above (as agreed in [2]).

| Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | 24.25<f<29.5GHz | 37<f<40GHz | 40<f<71GHz |  |  |  | 24.25<f<29.5GHz | 37<f<40GHz | 40<f<71GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| B2-1a | Misalignment and pointing error of BS | 0.2 | 0.2 | 0.2 | Exp. normal | 2.00 | 1 | 0.10 | 0.10 | 0.10 |
| B2-2 | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.15 | 0.15 | 0.15 |
| C1-2 | Uncertainty of the RF signal generator | 0.9 | 0.9 | 1.6 | Gaussian | 1.00 | 1 | 0.90 | 0.90 | 1.60 |
| B2-3 | RF leakage & dynamic range, test range antenna cable connector terminated | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| B2-4a | QZ ripple experienced by BS | 0.4 | 0.4 | 0.4 | Gaussian | 1.00 | 1 | 0.40 | 0.40 | 0.40 |
| B2-9 | Miscellaneous uncertainty | 0 | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.3 | 0.3 | 0.3 | Gaussian | 1.00 | 1 | 0.30 | 0.30 | 0.30 |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) | 0.43 | 0.57 | 0.57 | U-shaped | 1.41 | 1 | 0.30 | 0.40 | 0.40 |
| B2-6 | Insertion loss of transmitter chain | 0.12 | 0.12 | 0.12 | Rectangular | 1.73 | 1 | 0.07 | 0.07 | 0.07 |
| B2-7 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| B2-8 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | 0.29 | U-shaped | 1.41 | 1 | 0.15 | 0.21 | 0.21 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.52 | 0.52 | 0.52 | Rectangular | 1.73 | 1 | 0.30 | 0.30 | 0.30 |
| B2-11 | Misalignment positioning system | 0 | 0 | 0 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| B2-4b | QZ ripple experienced by calibration antenna | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| B2-10 | Rotary joints | 0 | 0 | 0 | U-shaped | 1.41 | 1 | 0.00 | 0.00 | 0.00 |
| B2-1b | Misalignment and pointing error of calibration antenna | 0 | 0 | 0 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| B2-12 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| B2-13 | Switching uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 1.15 | 1.19 | 1.78 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 2.25 | 2.33 | 3.48 |

# 3.	Conclusion

This contribution has provided proposals on the measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz based on the agreed WF in [2-3] to complete the perf part of work item as targeted at TSG RAN#98. The proposals are summarized in the table below (proposals are highlighted).

Table 2-1: Standard test equipment measurement uncertainty

| UID | Uncertainty source |  | Measurement uncertainty (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Test equipment |  | 3 < f < 6 (GHz) | 24.25 < f ≤ 29.5 GHz | 37 < f < 43.5 (GHz) | 43.5 < f < 71 (GHz) |
| C1-3 | Uncertainty of the Network Analyzer |  | 0.20 (Note 2) | 0.30 | 0.30 | 0.30 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter, power sensor) |  | 0.14 to 0.26 (Note 2) | 0.50 | 0.70 | 0.90 |
| C1-7 | Unceratinty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) for IF used with mixer |  |  |  |  | 0.60 (Note 3) |
| A2-20 | Uncertainty of the mixer (FR2 only) |  |  |  |  | 2.25 (Note 3, Note 6) |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – low power (UEM absolute ACLR) |  |  | 0.90 | 0.90 | 0.90 |
| C1-8 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) – relative (ACLR) |  |  | 0.75 | 0.90 | 1.05 |
| C1-9 | Uncertainty of the RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 |  |  | 1.25 | 1.45 | 1.65 |
| C1-2 | Uncertainty of the RF Signal Generator |  | 0.46 (Note 2) | 0.90 | 0.90 | 43.5 < f < 48.2 (GHz) 1.60 (Note 4) |
| MUOOBint | Additional uncertainty for the OOB interferer signal |  |  |  |  | 1.1 (Note 5) |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.30 (Note 7) | 0.40 (Note 7) | 0.40 (Note 3) |
| A2-5b | Mismatch of receiver chain for low power receiver |  |  | 0.51 (Note 8) | 0.51 (Note 8) | 0.51 |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) |  |  | 0.30 | 0.40 | 0.50 |
| A2-19 | Uncertainty of the LNA (FR2 only) |  |  |  |  | 0.00 (Note 9) |
| MUPA | MUPA Uncertainty due to use of PA |  |  |  |  | 0.2 (Note 5) |
|  | OTA modulation quality |  |  | 1% | 1% | 1% (Note 10) |
|  |  | Note 1: Values used for FR2-1 MU evaluation in TR 37.941. Note 2: Values used for FR1 MU evaluation in TR 37.941. Note 3: Values are from FR2-1 Tx spurious emission measurement MU budget table in TR 37.941 Note 4: Value is from 47 GHz band, see discussion note (R4-2115779) Note 5: Value is from Table 14.2.4-1 (TR 37.941) out of band blocking FR2-1, uncertainty of OOB blocking interferer signal which is CW signal and uncertainty from PA. Note 6: The applicability of this value for FR2-2 is FFS. Note 7: Values are from FR2-1 EIRP and other MU table in TR37.941 Note 8: Value is from FR2-1 Tx Off power measurement and other MU table in TR37.941 Note 9: Value is from FR2-1 Tx spurious emission measurement in TR37.941. Note 10: Although this value is different in category (value itself represents test MU not contributor), for FR2-2, there is possible increase of EVM MU due to wider BW. |  |  |  |  |

# References

[1]	RP-222656, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm.

[2]	R4-2217362, “WF on the list of MU contributors for FR2-2”, Nokia.

[3]	R4-2217363, “WF on the test equipment MU”, Keysight Technologies.

[4]	R4-2217364, “WF on the OTA chambers applicability for FR2-2”, Ericsson.

[5]	3GPP TR 37.941 v17.0.0: “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”.