3GPP TSG-RAN WG4 Meeting # 105	R4-2218343

# Toulouse, France, November 14 – November 18, 2022

Agenda item:	11.1

Source:	Intel Corporation

Title:	Analysis of RAN4 options for FG 6-1a support

Document for:	Discussion

- Introduction

In the previous RAN4 meeting, the criteria for high-level analysis on RAN4 options for FG 6-1a support was agreed as below [1].

Conclusion in RAN4 #10e-bis-e - RAN4 works on the below aspects/criteria for highest-level analysis on options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP - RRM requirements impact (Spec impact) / workload in RAN4 - Mobility performance impact - Throughput impact (Data interruption) - UE power consumption / UE complexity - Options from RP-221911 are further split as below for high-level analysis. - Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-1-3) Using a separate RF chain without interruptions - Option B-1-4) Using a separate RF chain with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps - Option B-2-1) Shared MG or NCSG for RLM/BFD/BM and L3 measurement - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG 6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

- This paper discusses each option with the agreed criteria/aspects and provides our views. We try to selectively take the agreeable arguments in the previous meeting.

- Discussion

## RRM requirements/workload load impacts in RAN4

The potential issues mentioned in the previous meeting are summarized to estimate workload impacts in RAN4.

| Options | RRM requirements /workload load | Summary |
| --- | --- | --- |
| A | Whether to introduce timing tracking requirement based on CSI-RS | 1 minor issue |
| B-1-1 | Specify reuse of existing RLM/BFD/BM requirement | 1 minor issue |
| B-1-2 | Specify reuse of existing RLM/BFD/BM requirement Interruption requirements for UE to switch to wider RF | 2 minor issues |
| B-1-3 | Specify reuse of existing RLM/BFD/BM requirement | 1 minor issue |
| B-1-4 | Specify reuse of existing RLM/BFD/BM requirement Interruption requirements for UE to switch to wider RF | 2 minor issues |
| B-2-1 | Shared gap based RLM, BFD and BM requirement Gap sharing mechanism for L1 and L3 measurement CSSF for measurements within gaps | 5 major issues |
| B-2-2 | Dedicated gap based RLM, BFD and BM requirement CSSF for measurements within gaps | 4 major issues |
| C | Specify reuse of existing NCD-SSB requirement | 1 minor issue |

## Mobility performance impact

We investigate the mobility performance by checking the limiting factor for RLM/BFM/BM measurement. Gap-based options would have limitation due to trade-off between gap availability and data transmission availability in time domain.

| Options | Mobility performance limitation | Summary |
| --- | --- | --- |
| A | No limitation since RS for RLM/BFM/BM is within BWP | No limitation |
| B-1-1 | No limitation since RS for RLM/BFM/BM is within UE RF BW | No limitation |
| B-1-2 | No limitation since RS for RLM/BFM/BM is within UE RF BW | No limitation |
| B-1-3 | No limitation since RS for RLM/BFM/BM is within UE RF path | No limitation |
| B-1-4 | No limitation since RS for RLM/BFM/BM is within UE RF path | No limitation |
| B-2-1 | Trade-off between gap availability and data transmission availability . Further degradation under gap-sharing | Gap availability limitation |
| B-2-2 | Trade-off between gap availability and data transmission availability | Gap availability limitation |
| C | No limitation since RS for RLM/BFM/BM is within BWP | No limitation |

## Throughput impact (Data interruption)

Data throughput performance can be investigated by checking the limiting factor caused by RLM/BFM/BM measurement. Like as mobility performance, gap-based options would have limitation due to trade-off between gap availability and data transmission availability in time domain. The interruption time for activation of wider RF BW / additional RF path in B-1-2 and B-1-4 is assumed to be periodic. The gap consists of interruption time from RF switching and actual gap duration.

| Options | Throughput performance limitation | Summary |
| --- | --- | --- |
| A | No limitation since is within BWP | No limitation |
| B-1-1 | No limitation since is within UE RF BW | No limitation |
| B-1-2 | Interruption time for wider RF BW activation | Interruption time * |
| B-1-3 | No limitation since is within UE RF path | No limitation |
| B-1-4 | Interruption time for activation of additional RF path | Interruption time * |
| B-2-1 | Trade-off between gap availability and data transmission availability | Gap ** (Scheduling restriction) |
| B-2-2 | Trade-off between gap availability and data transmission availability | Gap |
| C | No limitation since RS is within BWP | No limitation |

*) For RF activation only, interruption without scheduling restriction is assumed in B-1-2 / B-1-4.

**) NCSG may avoid data interruption but, it also has limitation when periodic gap is required.

## UE power consumption in RF

In this section, we investigate the power consumption in terms of activated BWP size and activated time. We assume that RF power consumption is proportional to bandwidth size and RF path. Considering {1.5, 3, 3.5, 4, 5.5, 6} of gap length (in msec) and {20, 40, 80, 160} gap repetition period (in msec), the portion of gap interval in time i.e., β in B2 is ranging from 0.01 ~ 0.3. Under 3 msec gap for SMTC window of 2 msec with 20 / 160 msec periodicity, β would be 0.15 / 0.02 respectively. The portion of activated interval in time i.e., α in B1 also would have the same range of β if we assume the similar activation pattern as gap.

| Options | UE power consumption in RF | Summary |
| --- | --- | --- |
| A | Active BWP size | BWP |
| B-1-1 | Active BWP size + α × Additional RF BW where α is the portion of activation interval in time | BWP + α × ∆BW (For B-1-1, α = 1) |
| B-1-2 |  |  |
| B-1-3 | Active BWP size + α × SSB BW in additional RF path where α is the portion of activation interval in time | BWP + α × SSB BW in additional RF path (For B-1-3, α = 1) |
| B-1-4 | Active BWP size + α × SSB BW in additional RF path where α is the portion of activation interval in time | BWP + α × SSB BW in additional RF path |
| B-2-1 | (1 – β) × Active BWP size + β × SSB BW size, where β is the portion of gap interval in time | (1 – β) × BWP + β × SSB BW |
| B-2-2 | (1 – α) × Active BWP size + α × SSB BW size, where α is the portion of gap interval in time | (1 – α) × BWP + α × SSB BW |
| C | Active BWP size | BWP |

Note) α, β is the portion of activated interval, Gap interval, respectively in time domain for B-1/B-2 respectively.

# Summary and Conclusion

The qualitative investigations in Section 2.1 ~ 2.4 can be summarized below.

| Options | RRM Spec Impact | Mobility Impact | Tput Impact | RF Power Consumption |
| --- | --- | --- | --- | --- |
| A | 1 minor issue | No limitation | No limitation | BWP |
| B-1-1 | 1 minor issue | No limitation | No limitation | BWP + ∆BW |
| B-1-2 | 2 minor issues | No limitation | Interruption time | BWP + α × ∆BW |
| B-1-3 | 1 minor issue | No limitation | No limitation | BWP + SSB BW in additional RF path |
| B-1-4 | 2 minor issues | No limitation | Interruption time | BWP + α × SSB BW in additional RF path |
| B-2-1 | 5 major issues | Gap availability limitation | Gap | (1 – β) × BWP + β × SSB BW |
| B-2-2 | 4 major issues | Gap availability limitation | Gap interruption | (1 – α) × BWP + α × SSB BW |
| C | 1 minor issue | No limitation | No limitation | BWP |

Note) α, β is the portion of activated interval, Gap interval, respectively in time domain for B-1/B-2 respectively

Different views on the priority of criterion can give various preferences in each option. In our view, mobility/Tput impacts as well as RRM specification impact are the most important factors and UE power consumption in ‘B-1-2/B-1-1’ may be left as UE vendor’s implementation choice.

Also, we think that RAN4 need to consider the justification of workload for new design when existing reasonable and almost stable alternative solutions are available. Thus, we have preference to option ‘A’ and option ‘C’. We also think that option ‘B-1-2/B-1-1’ is reasonable approach but we have less preference to ‘B-2’ and ‘B-1-3/B-1-4'. It is also desirable to enable the benefits of NCD-SSB for RedCap UE in Rel-17 to non-RedCap UE in Rel-18 through option ’C’. We also note that the RAN plenary task only requested RAN4 to perform a high-level analysis of the options and did not request RAN4 to make a recommendation for which option(s) to specify. Although we have shared our preference in the paper, the priority in this meeting to reach consensus on the outcome of the analysis that can be reported to RAN.

Proposal 1: Report to RAN#98 with summary table and contents in section 3.

Proposal 2: Report to RAN#98 with comparison tables and contents in section 2.1 ~ 2.4 as an annex.

- References

- R4-2217280, “WF on RAN task on BWP operation without restriction”