3GPP TSG-RAN4 Meeting #105	R4-2218555

# Toulouse, France, November 14 – November 18, 2022

Agenda item:	8.6.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Power Class aspect on lower MSD

Document for:	Approval

# Introduction

RAN4#104-bis-e approved WF of [1], where a following was captured.

This contribution shares our views on the above aspect in [1].

# Discussion

MSD minimum requirements for different power classes can be significantly different depending on the order and power class as can be seen in Table 1, where seven band combinations have around 10 dB MSD difference between PC3 and PC2. And the number of these band combinations will increase according to the increase of PC2 capable band combinations.

Observation 1: MSD value is more than 10 dB difference between PC3 and PC2 in some cases and the number of these band combinations will increase as the number of PC2 band combinations increases.

Observation 2: MSD value due to higher order IMD like IMD4 for PC2 can be around 20 dB, which may not be always to be zero dB even if a UE has a lower MSD capability while it would be highly possible for PC3 MSD value for the same UE to have zero or close to zero.

Table 1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3/2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) PC3/PC2 | Delta (dB) |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8/17.8 | 9.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2/18.4 | 10.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26/31.9 | 5.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8/18.5 | 10.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A | N/A |
| CA_n2-n774 | n2 | 1855 | 5 | 25 | 1935 | 26/32.10 | 6.1 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 8/19.10 | 11.1 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A | N/A |
| CA_n5-n774,6 | 5 | 844 | 5 | 25 | 889 | 8.3/18.6 | 10.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3/18.6 | 10.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A | N/A |
| CA_n12-n77 | 12 | 702 | 5 | 20 | 732 | 5.5/11.7 | 6.2 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A | N/A |
| CA_n13-n77 | 13 | 782 | 5 | 20 | 751 | 5.5/20.5 | 15 | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | N/A | N/A |
| CA_n14-n77 | 14 | 795.5 | 5 | 15 | 765.5 | 5.5/11.7 | 6.2 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A | N/A |
| CA_n25-n774 | n25 | 1855 | 5 | 25 | 1935 | 26/32.1 | 6.2 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 8/19.1 | 11.1 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A | N/A |
| CA_n30-n77 | 30 | 2310 | 5 | 25 | 2355 | 8/17.6 | 9.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A | N/A |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | N/A | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11/16.3 | 5.3 | IMD4 |
| CA_n66-n77 | n66 | 1730 | 5 | 25 | 2130 | 31/34.33 | 3.33 | IMD2 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5/11.27 | 6.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A | N/A |
| CA_n71-n776 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5/11.4 | 5.9 | IMD5 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A | N/A |

Regarding MSD due to IMD, it may be likely that the delta per MSD type per BC would be close to constant even if e.g., PCB isolation is improved as can be seen in the below Figure 1, which shows MSD due to IMD2 for CA_n3-n78 for PC3 and PC2, respectively (a row in green in Table 1). And the delta is around 6 dB and this is well aligned with the specification.

Delta = around 6 dB

Figure 1: MSD due to IMD2 for CA_n3-n78

The Table 1 says the delta is also different from BC to BC even if the MSD type is the same. For instance, CA_n25-n77 has 11.2 dB delta for IMD4 while CA_n41-n71 has 5.3 dB delta for IMD4. If we were able to say that PC2 and PC3 have always X dB difference for the same MSD type for any band combination, it would be reasonable to allow UE to report MSD value only for single power class. The reality, however, is not the case. Hence, it is not easy to identify precise MSD information if only MSD value for single power class out of possible power classes is reported.

Observation 3: MSD difference b/w PC3 and PC2 is different from MSD type to type and BC to BC, it is not possible to estimate exact MSD value for all the power classes by reporting MSD value only for single power class.

It’s noted that this contribution addressed MSD due to IMD. Though MSD due to other MSD sources like cross band isolation may show slightly different tendency, but essential aspect is the same, i.e., it’s not possible to estimate lower MSD capability for different power classes for the same UE by one of the lower MSD capability for signal PC.

Proposal 1: Allow UE to report different lower MSD capability according to power class

# Conclusion

This document has made the following observations and proposals:

Observation 1: MSD value is more than 10 dB difference between PC3 and PC2 in some cases and the number of these band combinations will increase as the number of PC2 band combinations increases.

Observation 2: MSD value due to higher order IMD like IMD4 for PC2 can be around 20 dB, which may not be always to be zero dB even if a UE has a lower MSD capability while it would be highly possible for PC3 MSD value for the same UE to have zero or close to zero.

Observation 3: MSD difference b/w PC3 and PC2 is different from MSD type to type and BC to BC, it is not possible to estimate exact MSD value for all the power classes by reporting MSD value only for single power class.

Proposal 1: Allow UE to report different lower MSD capability according to power class

# Reference

- R4-2217723, WF on study for lower MSD, Huawei, HiSilicon, TSG-RAN WG4 #104-bis-e