3GPP TSG-RAN WG4 Meeting #105-e	R4-2218759

Toulouse, France, 14th-18th Nov, 2022

Title:	Reply LS on clarification for ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Response to:	R2-2211023

Release:	Rel-17

Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 would like to thank RAN2 for the LS to inquire the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing different power class parameters.

Regarding RAN2’s request to RAN4 for the responses to the following questions:

- Whether R4 16-8 is applicable to only inter-band CA?

- What is the interaction between R4 16-8 and the existing power class capabilities (i.e. ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClassNRPart-r16 (if R4 16-8 is also applicable to the cases other than inter-band CA) and powerClass/powerClass-v1610)?

After discussion during RAN4#105, RAN4 would like to respectfully provide the following responses for RAN2 consideration.

Response to 1): Yes. It is applicable to inter-band UL CA only.

Response to 2): ue-PowerClassPerBandPerBC-r17 indicates power class the UE supported for the individual band within the band combination when operating according to the band combination, while powerClass, powerClass-v1610 indicates the power class for this band combination, meanwhile the former one shall not be higher than the latter one. In addition, ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 determines the maximum Tx power available in each band, in other words, the power class indicated by ue-PowerClassPerBandPerBC-r17 per band per band combination shall not be higher than the power class indicated by ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700.

In short, the maximum Tx power available in the individual band within the band combination is determined by ue-PowerClassPerBandPerBC-r17 if indicated, otherwise by the minimum one between powerClass, powerClass-v1610 and ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 responses into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106	27Feb -03 Mar 2023         Athens, GR

TSG-RAN WG4 Meeting #106-bis-e	17 - 26 April 2023             E-meeting