3GPP TSG-RAN WG4 Meeting#105	R4-2218827

# Toulouse, France, 14 – 18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ON/OFF time mask for TX switching across two bands with dual-TAG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce time masks for TX switching across two bands with dual-TAG and remove the restriction that UL CA with TX switching is restricted to the collocated scenario with a single TAG. This can be done with changes limited to the switching time masks specified in 38.101-1 for verifiction of switching band combinations supported with capability supportedNumberTAG (optional for NR CA and can also be included for band combinations with switching). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Time masks are specified by observing that for an uplink switching triggered for an uplink transmission starting at T0 on either carrier, the UE is not expected to be scheduled or configured to transmit on any of the carriers for a duration of X ms preceding T0 regardless of the timing advance NTA of each TAG. The general requirements for a non-collocated scenario are subject to the applicable MTTD for inter-band UL CA corresponding to an UL timing difference of one symbol at SCS = 30k. Clause 6.3A.3.3.2: time masks are introduced for verification of the non-collocated case with carriers belonging to different TAGs for the switching option switchedUL. The time masks are intended for functional verification of Tx switching with uplink slot timing difference between the carriers when symbols containing the switching period preceding the time T0 are not scheduled by the test system on either carrier. These time masks do not preclude any other scheduling or configuration on either carrier such that the UE is not expected to transmit on any carrier during OFDM symbols that fully overlaps with the switching period X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tx switching with uplink slot timing difference between the carriers when symbols containing the switching period preceding the time T0 are not scheduled by the test system on either carrier is not verified, which can lead to UE malfunction in the field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The mask modified such that the switching period and any uplink slot timing difference are contained in an integer number of OFDM symbols and that time time T0 starts at a symbol boundary of the slot. The text modified to make clear that the UE is not ‘expected to be scheduled or configured to transmit’ during these symbols. |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

##### 6.3A.3.3.1	General

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3.3 apply.

For inter-band carrier aggregation with two contiguous carriers assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3A.3.1 apply for those carriers.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3A.3.2 apply for those carriers.

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per operating band), the transmit ON/OFF time mask requirements specified in subclause 6.3.3.1 apply for the NR band supporting one component carrier, and for the NR band supporting two contiguous component carriers the requirements specified in subclause 6.3A.3.1 apply.

##### 6.3A.3.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power for CA power class 3 when the capability uplinkTxSwitching-PowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The above requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers in

a single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

In case supportedNumberTAG is supported with two TAGs for a band pair supporting dynamic UL Tx switching, the switching time masks specified in Figure 6.3A.3.3.2-1c and Figure 6.3A.3.3.2-1d apply for the switching option switchedUL with the switching period taken in carrier 1 and carrier 2, respectively. For an uplink switching triggered for an uplink transmission starting at T0 on either carrier, the UE is not expected to be scheduled or configured to transmit on any of the carriers for a duration of X ms preceding T0 regardless of the timing advance TTA of either TAG with TTA as specified in clause 4.3.1 of [6].

For the switching period located in carrier 1 as specified in Figure 6.3A.3.3.2-1c, the UE is not expected to be scheduled or configured to transmit on carrier 1 during n1 ≥ 0 OFDM symbols (OS) containing the switching period and n2 ≥ n1 ≥ 0 OS accommodating the uplink slot timing difference between the uplink carriers in addition. The UE is not expected to transmit on on carrier 1 during any overlap with the switching period preceding T0 on carrier 2 when TTA,1 > TTA,2 and is not expected to transmit on carrier 1 after T0 on carrier 2 when TTA,1 < TTA,2, where TTA,1 and TTA,2 denote the timing advance for the TAG of carrier 1 and carrier 2, respectively.

Figure 6.3A.3.3.2-1c: Time mask for switching between UL carrier 1 and UL Carrier 2 belonging to different TAGs, where the switching period is located in carrier 1

For the switching period located in carrier 2 as specified in Figure 6.3A.3.3.2-1d, the UE is not expected to be scheduled or configured to transmit on carrier 2 after T0 on carrier 1 when TTA,1 > TTA,2 and is not expected to transmit on carrier 2 during any overlap with the switching period preceding T0 on carrier 1 when TTA,1 < TTA,2.

Figure 6.3A.3.3.2-1d: Time mask for switching between UL carrier 1 and UL Carrier 2 belonging to different TAGs, where the switching period is located in carrier 2

The above requirements for UEs supporting supportedNumberTAG with two TAGs apply for an uplink slot timing difference between the carriers up to MTTD in accordance with clause 7.5.4 of [13]. The switching period X is in accordance with the supported uplinkTxSwitchingPeriod for the band pair.

The time masks do not preclude any other scheduling or configuration on either carrier such that the UE is not expected to transmit on any carrier during OFDM symbols for the configured sub-carrier configuration m that fully overlaps with the switching period X.

##### 6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod2T2T is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1 and carrier 2.

The switching periods described in Figure 6.3A.3.3.3-1a and Figure 6.3A.3.3.3-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T.

Figure 6.3A.3.3.3-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.3-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

##### 6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

< end of changes >