3GPP TSG-RAN4 Meeting #105	R4-2218861

# Toulouse, France, November 14th - 18th, 2022

Agenda item:			10.1.2

Source:	vivo

Title:	Further discussion and reply LS on UE power limitation for STxMP in FR2

Document for:	Discussion

Introduction

In RAN4#104, a LS [1] has been received from RAN1 for studies of simultaneous UL transmission across multiple UE panels (STxMP), in which the following two assumptions on power limitation were listed, and some questions were raised.

- Assumption 1: Power limitation per panel for STxMP

- Assumption 2: A total power limitation per UE over all UE panels used for STxMP

Above power limitation includes both total radiated power and EIRP, and scenarios of these assumptions include at least single carrier scenario in FR2.

RAN1 seeks a few answers from RAN4 on the following questions in order to proceed further on the study of UE power control for STxMP.

Question 1: From RAN4 perspective, is Assumption 1 is feasible?

Question 2: From RAN4 perspective, is Assumption 2 is feasible?

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

In this paper, a brief situation is introduced, and two options of LSs were re-submitted.

Discussion

For the past two meetings, although intensive discussion was done, no conclusion was made yet, and some of the key discussion was documented in summary file [2] and [3]. It turns out to be that there are two versions of LS that is difficult to be merged, and both of the two versions were incorporated into [4].

One controversial part is the definition of “panel”. Although there are proposals to introduce tentative “Panel” definition based on earlier RAN1 assumption such as in [6][7], still this cannot be concluded. However, in the meantime, similar assumption was agreed in Multi-RX WI in [8] as following:

Way Forward: Take the following panel assumption for DL reception, noting panel is a logical construct:

‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation: Unit of antenna group to control beam independently - Within a panel, one beam can be selected and used for DL reception. - Across different panels, multiple beams [(each selected per panel)] may be used for DL reception. - ‘Beam’ is assumed to mean spatial filter associated with reception.

So it seems that it is still possible to have a common understanding in RAN4 regarding these definitions. However, if consensus cannot be made, some information change is also helpful compared to no reply.

Proposal: Agree one of the options of the reply LS of last meeting’s outcome, among them:

- Option 1 includes some preliminary answers

- Option 2 is for more RAN1 clarification

Conclusion

In this paper, a brief history has been introduced and the following proposal is made:

Proposal: Agree one of the options of the reply LS of last meeting’s outcome, among them:

- Option 1 includes some preliminary answers

- Option 2 is for more RAN1 clarification

The two options of draft reply LS were also attached in Annex.

- References

[1] R1-2205639, LS on UE power limitation for STxMP in FR2, RAN1#109-e

[2] R4-2214251, Email discussion summary for [104-e][140] NR_reply_LS_UE_RF, Moderator (Apple), RAN4#104-e

[3] R4-2217796, Email discussion summary for [104-bis-e][144] NR_reply_LS_UE_RF, Moderator (Apple), RAN4#104-bis-e

[4] R4-2217758, [Draft] reply LS on UE power limitation for STxMP in FR2, vivo, RAN4#104-bis-e

[5] R4-2212808, Discussion and reply LS on UE power limitation for STxMP in FR2, vivo, RAN4#104-e

[6] R4-2216122, Discussion and reply LS on UE power limitation for STxMP in FR2, vivo, RAN4#104-bis-e

[7] R4-2216783, Reply LS to RAN1 on UE power limitation for STxMP in FR2, Qualcomm, RAN4#104-bis-e

[8] R4-2217731, WF on FR2 UE RF requirements for 2AoA DL Rx, vivo, RAN4#104-bis-e

- Annex A Option 1

3GPP TSG-RAN4 Meeting #105	R4-22xxxxx

# Toulouse, France, November 14th - 18th, 2022

Title:	[Draft] reply LS on UE power limitation for STxMP in FR2

Response to:	(R4-2211516_ R1-2205639) LS on UE power limitation for STxMP in FR2

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on UE power limitation for STxMP in FR2.

Since RAN4 has not defined the concept of “panel” unfortunately, following answers are provided based on earlier RAN1 assumption as below:

‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation:

- Unit of antenna group to control beam independently

  - Within a panel, one beam can be selected and used for UL transmission.

  - Across different panels, multiple beams (each selected per panel) may be used for UL transmission

  - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception

- Unit of antenna group to control its transmission power

- Unit of antenna group to have a common UL timing

Regarding the questions, RAN4 is currently have the following answers:

Question 1: From RAN4 perspective, is Assumption 1 is feasible?

Answer: Yes.

Question 2: From RAN4 perspective, is Assumption 2 is feasible?

Answer: Yes.

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

Answer: RAN4 confirm that existing UE RF requirements are framed so standards compliance implies regulation compliance (clause 6.5x in TS38.101-2).

For any additional limitation like the sum over all panels of the per-panel power limitation for STxMP, would be defined in RAN4 if necessary after the WI started in RAN4.

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

Answer: It is believed that both assumptions are feasible, and both assumptions shall be applied to a same UE. The per-panel power limitation would be defined if necessary in latter stage, and the per-UE power limitation should be applicable at all the time. ‘Limitation’ here applies to regulatory compliance rather than a configured power requirement.

RAN4 can define requirements for STxMP, e.g., per-panel/TCI configured output power once RAN4 begin work on this topic. The WI has not started in RAN4 yet.

Additionally, RAN4 has agreed that ‘Panel’ shall not be explicitly used in a requirement to ensure maximum flexibility for different UE implementations.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above answers into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106                  27th February – 3rd March 2023         Athens, Greece

TSG-RAN WG4 Meeting #106bis-e                  17th April – 26th April 2023         Online

- Annex B Option 2

3GPP TSG-RAN4 Meeting #105	R4-22xxxxx

# Toulouse, France, November 14th - 18th, 2022

Title:	[Draft] reply LS on UE power limitation for STxMP in FR2

Response to:	(R4-2211516_ R1-2205639) LS on UE power limitation for STxMP in FR2

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on UE power limitation for STxMP in FR2.

Before RAN4 can analyze the questions in the LS, RAN4 kindly requests clarifications for terms used:

- Panel:

RAN4 has not defined the concept of “panel.

- Power limitation:

From RAN4 perspective, there are several concepts related to the power radiated by UE:

- Power class: The definition of power class (e.g. TS 38.101-2 clause 6.2.1.x), which is a package composed of below requirements

  - Min peak EIRP (The lower limit of EIRP at Tx beam peak direction);

  - Max EIRP (This is derived from regulatory requirements) and Max TRP;

  - Spherical coverage (The minimum EIRP at the Nth percentile of the distribution of EIRP measured over the full sphere around the UE).

- Configured transmitted power: PCMAX, f, c, which is used in RAN1 spec TS 38.213 power control part, and also applied in TS 38.101-2 clause 6.2.4: “The configured UE maximum output power PCMAX, f, c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215”. It is noted that PCMAX used by RAN1 power control mechanism for FR2 is considered at the virtual antenna connector which is not testable from RAN4 perspective.

- P-max: The parameter p-Max (i.e. p-UE-FR2) similar to FR1 p-UE-FR1 was introduced by RAN2 spec, which is the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. However, P-max for FR2 has not been implemented in RAN4 requirements .

With the RAN1 definition of panel, RAN4 would like to ask which above concept(s)  is(are) necessary to consider for StxMP ‘power limitation’..

RAN4 will discuss whether to define additional limitation or requirement for STxMP once RAN4 begin work on this topic. The WI has not started in RAN4 yet.

Additionally, RAN4 has agreed that ‘Panel’ shall not be explicitly used in a requirement to ensure maximum flexibility for different UE implementations.

2. Actions:

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the above clarifications during their discussion, and provide answers to the follow-up questions.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #106                  27th February – 3rd March 2023         Athens, Greece

TSG-RAN WG4 Meeting #106bis-e                  17th April – 26th April 2023         Online