# 3GPP TSG-RAN WG4 Meeting #105	R4-2218862

# Toulouse, France, 14th - 18th November, 2022

Source: 	vivo

Title: 	Initial simulation results for adjacent channel co-existence evaluation

Agenda Item:	8.18.2.1

Document for:	Discussion

Introduction

In the last meeting, the adjacent channel co-existence scenarios and detailed simulation assumptions were further discussed. The agreements and way-forwards for adjacent channel co-existence simulation of non-overlapping subband full duplex (SBFD) were captured in [1]. In this contribution, we provide our initial simulation results for adjacent channel co-existence evaluation.

Discussion

In this contribution, we only considered the scenarios for SBFD co-existence study with high priority, as shown in Table 1. The Urban Macro scenario is agreed as baseline scenario for SBFD co-existence study with high priority.

Table 1: Scenarios for SBFD co-existence study

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
| FR2 (30GHz) | 3 | Urban Macro -> Urban Macro | High |

The aggressor and victim combinations are listed in Table 2 below.

Table 2: Victim, aggressor and aggressor baseline for SBFD co-ex study

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 |  | High |
| SBFD (DUD) | NR TDD DL | Case 7 | No system in adjacent channel | High |
| SBFD(DU) |  | Case 8 |  | High |

The detailed simulation assumptions for FR1 and FR2 Urban Macro scenarios were given in the Annex. The grid shift assumption is 100% for all the cases of the urban macro deployments. Both SBFD antenna configuration 1 and configuration 2 are considered in the SLS.

The simulation results for SINR and throughput distributions are given for Case 1,2,7,8 for FR1 and FR2 as in Section 2.1~2.4.

Case 1

FR1

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.35 | 26.09 | 0.59 | 4.00 |
| ACI | 1.35 | 26.09 | 0.59 | 4.01 |
| Gain (acl-baseline)/basline | 0.01% | 0.01% | 0.03% | 0.34% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.63 | 25.43 | 0.57 | 3.40 |
| ACI | 0.63 | 25.43 | 0.57 | 3.41 |
| Gain (acl-baseline)/basline | 0.01% | 0.00% | 0.00% | 0.23% |

FR2

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.73 | 21.93 | 0.58 | 3.28 |
| ACI | 0.73 | 21.95 | 0.58 | 3.31 |
| Gain (acl-baseline)/basline | 0.40% | 0.07% | 0.01% | 1.00% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -6.83 | 23.60 | 0.31 | 3.44 |
| ACI | -6.82 | 23.66 | 0.31 | 3.44 |
| Gain (acl-baseline)/basline | -0.02% | 0.25% | 0.00% | 0.01% |

Case 2

FR1

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.83 | 24.89 | 0.31 | 3.55 |
| ACI | -0.83 | 24.89 | 0.31 | 3.55 |
| Gain (acl-baseline)/basline | -0.01% | 0.03% | 0.00% | 0.01% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.29 | 23.47 | 0.60 | 3.45 |
| ACI | 0.29 | 23.47 | 0.60 | 3.45 |
| Gain (acl-baseline)/basline | 0.00% | 0.00% | 0.00% | 0.00% |

FR2

SBFD antenna configuration 1

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.60 | 21.44 | 0.53 | 3.19 |
| ACI | -0.60 | 21.51 | 0.53 | 3.19 |
| Gain (acl-baseline)/basline | -0.04% | 0.30% | 0.08% | 0.07% |

SBFD antenna configuration 2

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -2.52 | 21.87 | 0.38 | 3.44 |
| ACI | -2.52 | 21.87 | 0.38 | 3.44 |
| Gain (acl-baseline)/basline | 0.00% | 0.00% | 0.00% | 0.00% |

Case 7

FR1

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 0.00 | 24.84 | 0.53 | 3.73 |
| ACI | 0.00 | 24.74 | 0.53 | 3.73 |
| Gain (acl-baseline)/basline | 0.44% | -0.40% | -0.02% | -0.01% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -5.41 | 11.38 | 0.20 | 1.55 |
| ACI | -5.48 | 11.14 | 0.20 | 1.53 |
| Gain (acl-baseline)/basline | 1.22% | -2.05% | -0.92% | -1.73% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.50 | 24.81 | 0.38 | 3.64 |
| ACI | -0.50 | 24.77 | 0.38 | 3.64 |
| Gain (acl-baseline)/basline | 0.06% | -0.13% | -0.01% | -0.04% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -2.49 | 11.21 | 0.32 | 1.54 |
| ACI | -2.49 | 11.07 | 0.32 | 1.52 |
| Gain (acl-baseline)/basline | 0.15% | -1.20% | 0.12% | -1.21% |

FR2

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.27 | 22.44 | 0.41 | 3.54 |
| ACI | -3.28 | 22.11 | 0.41 | 3.53 |
| Gain (acl-baseline)/basline | 0.34% | -1.48% | 0.00% | -0.33% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -1.86 | 12.25 | 0.30 | 1.67 |
| ACI | -1.87 | 12.25 | 0.30 | 1.66 |
| Gain (acl-baseline)/basline | 0.86% | -0.04% | 0.00% | -0.33% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.05 | 23.87 | 0.33 | 3.55 |
| ACI | -3.12 | 23.76 | 0.34 | 3.53 |
| Gain (acl-baseline)/basline | 2.55% | -0.46% | 3.74% | -0.48% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -1.23 | 12.28 | 0.42 | 1.67 |
| ACI | -1.23 | 12.27 | 0.42 | 1.67 |
| Gain (acl-baseline)/basline | 0.00% | -0.10% | -0.02% | -0.01% |

Case 8

FR1

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.79 | 24.51 | 0.57 | 3.42 |
| ACI | -0.79 | 24.37 | 0.57 | 3.42 |
| Gain (acl-baseline)/basline | 0.83% | -0.60% | -0.01% | 0.00% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.04 | 11.59 | 0.26 | 1.58 |
| ACI | -3.06 | 11.35 | 0.26 | 1.55 |
| Gain (acl-baseline)/basline | 0.53% | -2.03% | -0.77% | -1.80% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | 1.86 | 24.84 | 0.47 | 3.57 |
| ACI | 1.81 | 24.83 | 0.47 | 3.57 |
| Gain (acl-baseline)/basline | -2.39% | -0.03% | -0.01% | -0.09% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -2.70 | 11.94 | 0.32 | 1.62 |
| ACI | -2.71 | 11.74 | 0.32 | 1.60 |
| Gain (acl-baseline)/basline | 0.13% | -1.72% | -0.32% | -1.61% |

FR2

SBFD antenna configuration 1

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -3.63 | 22.94 | 0.23 | 3.72 |
| ACI | -3.65 | 22.87 | 0.23 | 3.72 |
| Gain (acl-baseline)/basline | 0.81% | -0.32% | -0.27% | 0.01% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -4.78 | 12.24 | 0.20 | 1.67 |
| ACI | -4.78 | 12.25 | 0.20 | 1.67 |
| Gain (acl-baseline)/basline | 0.01% | 0.05% | 0.00% | 0.06% |

SBFD antenna configuration 2

DL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -0.60 | 23.95 | 0.43 | 3.59 |
| ACI | -0.74 | 23.92 | 0.43 | 3.59 |
| Gain (acl-baseline)/basline | 24.31% | -0.13% | -1.04% | 0.00% |

UL

|  | 5% SINR | 50% SINR | 5% thr (Bps/Hz) | 50% thr (Bps/Hz) |
| --- | --- | --- | --- | --- |
| Baseline | -2.28 | 12.25 | 0.33 | 1.66 |
| ACI | -2.28 | 12.26 | 0.33 | 1.66 |
| Gain (acl-baseline)/basline | 0.00% | 0.12% | 0.00% | 0.02% |

Conclusion

This contribution provides initial simulation results of SINR and throughput distributions for adjacent channel co-existence in FR1 and FR2, considering Urban Macro scenario as high priority. The following observations are made:

For Urban Macro scenario

- SBFD (DUD/DU) Aggressor->NR TDD DL Victim, the throughput losses are within 5%, which shows these two systems can co-exist;

- NR TDD DL Aggressor -> SBFD (DUD/DU) Victim, the throughput losses are within 5%, which shows these two systems can co-exist;

- References

[1] R4-2217466, WF for adjacent channel co-existence evaluation of SBFD operation, Samsung, CMCC, RAN4#104bis-e.

- Annex

FR1

Table A.1: Network layout for urban macro in FR1

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Path-loss model | - Macro(Aggressor) → Macro(Victim): - Macro-to-UE: UMa see TR 38.803 - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 |
| BS Tx power | For legacy TDD: 49 dBm For SBFD antenna configuration 1: 46 dBm For SBFD antenna configuration 2: 49 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | (Mg,Ng,M,N,P)=(1,1,1,1,2)，Omni |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| UE number per TRP | For legacy TDD: 1 user For SBFD with {DU}: 2 users For SBFD with {DUD}: 3 users |
| UE distribution | 20% indoor and 80% outdoor |
| O2I penetration loss models | Low loss model – 80% High-loss model – 20% |
| Channel bandwidth | 100 MHz |
| SCS | 30kHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz, 273PRB For SBFD {DUD}: 40MHz + 40MHz < ND, NU, NG > = <105, 53, 5> For SBFD {DU}: 80MHz < ND, NU, NG > = <214, 54, 5> |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DUD} and {DU}: 20MHz |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| UE max TX power in dBm | 23 dBm |
| UE min TX power in dBm | -33 dBm (100 MHz CBW) see TS 38.101-1 [7] |
| Handover margin in dB | 3 dB (Same as FR2) |
| BS mechanical downtilt angle in degrees | 6 degrees |
| Grid shift | 100% |
| Minimum UE-UE (2D) distance | 3m |

Table A.2: FR1 BS antenna element pattern for Urban Macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree horizontal element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

Table A.3: ACLR and ACS for FR1

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 45 dBc |
| BS ACS | 46 dBc |
| UE ACLR | 30 dBc (only ACLR1 is considered) |
| UE ACS | 33 dBc |

FR2

Table B.1: Network layout for urban macro in FR2 (30GHz)

| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| --- | --- |
| Inter-site distance | 200 m |
| BS antenna height | 25 m |
| Path-loss model | - Macro (Aggressor) – Macro (Victim) - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Macro-to-UE(V): Uma + penetration loss see TR 38.803 - UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss see TR 38.803 - UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. |
| UE distribution | 100% outdoor |
| UE antenna height | 1.5m |
| UE number per TRP | For legacy TDD: 1 user For SBFD with {DU}: 2 users For SBFD with {DUD}: 3 users |
| Channel bandwidth | 200MHz |
| SCS | 120kHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz, PRB = 132 PRBs For SBFD {DUD}: 80MHz + 80MHz < ND, NU, NG > = <50, 26, 3> For SBFD {DU}: 160MHz < ND, NU, NG > = <103, 26, 3> |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DUD} and {DU}: 40MHz |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| UE max TX power in dBm | 22.4 dBm |
| UE min TX power in dBm | -40dBm |
| Handover margin in dB | 3 dB |
| BS mechanical downtilt angle in degrees | 10 degrees |
| BS Noise figure in dB | 10 |
| UE Noise figure in dB | 10 |
| BS TX power | For legacy TDD: [30] dBm For SBFD antenna configuration 1: [27] dBm For SBFD antenna configuration 2: [30] dBm |
| BS antenna configuration ((Mg, Ng, M, N, P)) | For legacy TDD: (1, 1, 8, 16, 2) For SBFD antenna configuration 1: (1, 1, 4, 16, 2) For SBFD antenna configuration 2: (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) |
| UE antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) |
| Grid shift | 0%, 100% |
| Minimum UE-UE (2D) distance | 3m |

Table B.2: FR2 BS antenna modelling for urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. Note 4: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 5: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |

Table B.3: FR2 UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees Note 3 Fixed elevation: 90 degrees |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note3: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. Note 4: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation |  |

Table B.4: ACLR and ACS for FR2

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 28 dBc |
| BS ACS | 23.5 dBc |
| UE ACLR | 23 dBc |
| UE ACS | 23 dBc |