# 3GPP TSG-RAN WG4 Meeting #105	R4-2218866

# Toulouse, France, 14th - 18th November, 2022

Source: 	vivo

Title: 	Further discussion on feasibility aspects and RF requirements impact for CA_n5-n28

Agenda Item:	8.2.3.1

Document for:	Approval

Introduction

In the last meeting, we further discussed the feasibility aspects and RF requirements impacts for handheld UE supporting CA_n5-n28. It was agreed both 2 antenna and 3 antenna architectures can be studied [1]. For RF requirements impact, it needs further investigation on whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28. In this contribution, we provide our views on remaining issues for CA_n5-n28.

Discussion

For low band CA_n5-n28, the following agreements were reached in the last meeting [1].

Table 1. Agreements in RAN4#104bis-e for CA_n5-n28

| ## CA_n5-n28 |  |  |
| --- | --- | --- |
| ## Feasibility investigation |  | ## UE RF Requirements |
| ## Two-Antenna | ## Three-Antenna |  |
| Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis GTW agreement: - Both 2 and 3 antenna architectures will be analysed in the study item - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements. |  | Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28 Agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting. |
| Issue 3-3: RF parameters for requirements analysis Agreements: Companies are encouraged to provide the RF parameters when the feasibility study is conducted. |  | Issue 3-4: Other observations and proposals Agreements: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5. - Architecture assumption and multiplexer isolation performance needs to be clarified |

Currently, there are 2UL/2DL DC_28-n5 and 1UL/2DL CA_n5-n28 specified in the specification. Both these two band combinations consider separate antenna architecture as baseline since there is no existing LB-LB quadplexer. From antenna perspective, it was agreed that both 2/3 antenna architectures can be analyzed in the study item. However, if we intend to reuse the requirements defined for DC_28-n5, it is suggested to use separate antenna architecture (3-antenna) as baseline for analysis.

Proposal 1: Use 3-antenna as baseline for 2UL/2DL CA_n5-n28 analysis if RAN4 intends to reuse the requirements defined for DC_28-n5.

From our perspective, 2UL/2DL CA_n5-n28 can still reuse the separate antenna architecture for DC_28-n5. The RF parameters for DC_28-n5 can still be used in the CA analysis. The following RF parameters can be considered for CA_n5-n28 MSD requirements analysis.

Table 2. RF parameters for CA_n5-n28 requirements analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection at n5 UL range | 35 | dB |
| CIM5 of n5 UL relative to wanted level | -70 | dBc |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| PA RXBN noise | -125 | dBm/Hz |
| Thermal noise at RX ANT port | -165.5 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Proposal 2: It is suggested to consider Table 2 for CA_n5-n28 MSD requirements analysis.

Conclusion

This contribution further discusses remaining issues for feasibility aspects and RF requirements for handheld UE supporting CA_n5-n28. The following observations and proposals are made:

Proposal 1: Use 3-antenna as baseline for 2UL/2DL CA_n5-n28 analysis if RAN4 intends to reuse the requirements defined for DC_28-n5.

Proposal 2: It is suggested to consider Table 2 for CA_n5-n28 MSD requirements analysis.

Table 2. RF parameters for CA_n5-n28 requirements analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection at n5 UL range | 35 | dB |
| CIM5 of n5 UL relative to wanted level | -70 | dBc |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| PA RXBN noise | -125 | dBm/Hz |
| Thermal noise at RX ANT port | -165.5 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

- References

[1] R4-2217709, WF on CA_n5-n28, ZTE, RAN4#104bis-e.