3GPP TSG-RAN WG4 Meeting # 105		R4-2219033

Toulouse, France, November 14 – November 18, 2022

Agenda item:			10.1.1

Source:			Samsung

Title:	Discussion on maximum uplink timing difference for Multi-DCI multi-TRP with two TAs

Document for:		Discussion

- Introduction

In RAN4#104-bis-e meeting, LS (R1-2205593) from RAN1 was discussed and a WF was approved in [1]. RAN4 sent reply LS [2] to RAN1 for agreed MRTD/MTTD. There are still FFS parts in MTTD.

In this contribution, we provide our consideration of FFS part and give our proposals.

- Discussion

In last meeting, MRTD and MTTD values are agreed for UE capable of supporting RTD>CP. But for UE not capable of supporting RTD>CP, the MTTD needs further discussed. It is listed in [1] as below:

Issue 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1 Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP+M1 µs for FR1. Where M1 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 33/34.6 µs.

Issue 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2 Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP + M2 µs for FR2. Where M2 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 8/8.5 µs.

For UE not capable of supporting RTD >CP, CP length can be used as MRTD between multiple TRPs.

MTTD = MRTD + M1/M2.

For the components which contribute to M1 or M2, we think M1 or M2 should consider Te, TA adjustment accuracy, TA resolution error.

For multi-TRPs, Te/TA adjustment accuracy/TA resolution error are independent for different TRPs and should not be zeros.

Te is specified in spec:

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | [1.58]*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | [1.35]*64*Tc |
|  |  | 960 | [0.90]*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | [1.13]*64*Tc |
|  |  | 960 | [0.86]*64*Tc |

Te depends on SCS. To cover different SCS of uplink signals, the Te is chosen to be the maximum value in current requirements.

TA adjustment accuracy is specified as ±256 Tc for FR1 and ±128 Tc for FR2-1.

TA resolution error is specified as 16∙64∙Tc2μ, 16*64Tc for FR1 and 256Tc for FR2-1.

Therefore, M1 can be 1.6us, while M2 can be 0.5us.

Proposal 1: For the components which contribute to M1 or M2, M1 or M2 should consider Te, TA adjustment accuracy, TA resolution error. M1 = 1.6us while M2 = 0.5us.

For transient period between 2 UL signals associated with 2 different TAs, we don’t think it should be counted in MTTD. If multi-DCI uplink transmissions are TDM, the scheduling restriction can be introduced.

Proposal 2: Transient period between 2 UL signals shouldn’t be counted in MTTD. If multi-DCI uplink transmissions are TDM, the corresponding scheduling restriction can be introduced.

- Conclusion

In this contribution, we provide our consideration of FFS parts in MTTD and our proposals are:

Proposal 1: For the components which contribute to M1 or M2, M1 or M2 should consider Te, TA adjustment accuracy, TA resolution error. M1 = 1.6us while M2 = 0.5us.

Proposal 2: Transient period between 2 UL signals shouldn’t be counted in MTTD. If multi-DCI uplink transmissions are TDM, the corresponding scheduling restriction can be introduced.

- Reference

[1] R4-2217278, WF on MRTD/MTTD requirement for multi-TRPs with 2 Tas, Apple

[2] R4-2217279, Reply LS on MTTD for multi-DCI multi-TRP with two Tas, Ericsson