3GPP TSG-RAN4 Meeting #105	R4-2219065

# Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2731 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction to CHO requirement for satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.133 V17.7 captures two condtions in time based CHO for SAN, taking intra-frequency as example: - condition [T1-1] occurs earlier than TEvent_DU - condition [T1-1] occurs later than TEvent_DU plus [Tidentify intra with index] or [Tidentify_intra_without_index] Similarly, for location based CHO for SAN, taking intra-frequency as example: - both condition [D1-1] and condition [D1-2] are fulfilled earlier than TEvent_DU - both condition [D1-1] and condition [D1-2] are fulfilled is later than TEvent_DU plus [Tidentify intra with index] or [Tidentify_intra_without_index] The conditions aren’t complete, the time interval [T1-1] occuring after TEvent_DU but before TEvent_DU plus [Tidentify intra with index] or [Tidentify_intra_without_index] isn’t covered. And there are some editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete condtions. Other editorial corrections/updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conditions aren’t complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.2.2.2, other section/clauses are unaffected. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.1C.2.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than [Tidentify intra_SAN with index] or [Tidentify_intra_SAN_without_index] defined in clause [9.2.5C.1] or clause [9.2.6C.2].

For time-based conditional intra-frequency handover:

-	If condition [T1-1] occurs earlier than TEvent_DU +[Tidentify intra with index]  or TEvent_DU +[Tidentify_intra_without_index], then the measurement time delay equal to [Tidentify intra_SAN with index] or [Tidentify_intra_SAN_without_index] assuming UE only performs the measurements wihin the SMTC window of the target cell.

-	If condition [T1-1] occurs later than TEvent_DU +[Tidentify intra with index] or TEvent_DU +[Tidentify_intra_without_index], then the measurement time delay equals to the time from the end of Tevent_DU until condition [T1-1].

For location-based conditional intra-frequency handover:

-	If both condition [D1-1] and condition [D1-2] are fulfilled earlier than TEvent_DU+[Tidentify intra with index] or TEvent_DU +[Tidentify_intra_without_index], then the measurement time delay equal to [Tidentify intra_SAN with index]or [Tidentify_intra_SAN_without_index] assuming UE only performs measurements within the SMTC window of the target cell.

-	If both condition [D1-1] and condition [D1-2] are fulfilled is later than TEvent_DU plus [Tidentify intra with index] or [Tidentify_intra_without_index] for intra-frequency handover, then the measurement time delay equal to the time from the end of Tevent_DU until time when both condition [D1-1] and condition [D1-2] are fulfilled.

For inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than [Tidentify_inter_SAN_with_index] or [Tidentify_inter_SAN_without_index] defined in clause [9.3.4C].

For time-based conditional inter-frequency handover:

-	If condition [T1-1] occurs earlier than TEvent_DU +[Tidentify_inter_SAN_with_index] or TEvent_DU+[Tidentify_inter_SAN _without_index], then the measurement time delay is equal to [Tidentify_inter_SAN_with_index] or [Tidentify_inter_SAN_without_index] assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If condition [T1-1] occurs later than TEvent_DU plus[Tidentify_inter_SAN_with_index] or [Tidentify_inter_SAN_without_index], then the measurement time delay equals to the time from the end of Tevent_DU until condition [T1-1].

For location-based conditional inter-frequency handover,

-	If both condition [D1-1] and condition [D1-2] are fulfilled earlier than TEvent_DU +[Tidentify_inter_SAN_with_index] or TEvent_DU +[Tidentify_inter_SAN_without_ind ex], then the measurement time delay equal to [Tidentify_inter_SAN_with_index] or [Tidentify_inter_SAN_without_index] assuming that the UE uses only the SMTC window of the target inter-frequency carrier for performing the measurements. In this case Ksatellite=1, CSSFinter=1.

-	If both condition [D1-1] and condition [D1-2] are fulfilled later than TEvent_DU + [Tidentify_inter_SAN_with_index] or  TEvent_DU +[Tidentify_inter_SAN_without_index], then the measurement time delay equal to the time from the end of Tevent_DU until time of both condition [D1-1] and condition [D1-2] are fulfilled.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period [Tidentify_intra_without_index] or [Tidentify_intra_with_index] for intra-frequency handover or [Tidentify_inter_without_index] for inter-frequency handover.

<End of change>