3GPP TSG-RAN WG4 Meeting#105      	R4-2219082

Toulouse. France, 14 – 18 Nov, 2022

Source:	Nokia, BT plc

Title:	Discussion on non-contiguous UL CA for NR-U

Agenda item: 	7.28.1

Document for:	Discussion and Approval

# Introduction

The enhancements of NR shared spectrum bands WI, as presented in [1], currently does not include any objectives related to non-contiguous UL CA for NR-U. The WI on NR Intra band CA, as presented in [2], now includes requests to support this type of band combination.

In this contribution we discuss how to best handle these band combination requests.

# Discussion

In the RAN approved WID [2] there are currently the following band combinations requests added.

| Band combination configuration | UpLink configuration | Contact Company | RAN Plenary in which the conbination was requested | Related WI (acronym) |
| --- | --- | --- | --- | --- |
| CA_n46(2A) | CA_n46(2A) | BT plc | RAN#97 | NR_CA_R18_Intra |
| CA_n102(2A) | CA_n102(2A) | BT plc | RAN#97 | NR_CA_R18_Intra |

Both n46 and n102 are shared spectrum bands, meaning they will operate utilizing NR-U. To our understanding non-contiguous UL CA has not been specified for NR-U; however for other NR bands  it is already included in the specifications.

Observation 1:	Non-contiguous UL CA has not been specified for NR-U.

How to best address the operator request for support for non-contiguous UL CA for NR-U is open for discussion. One option can be to add this to the WI for enhancements of NR shared spectrum bands WI.

Proposal 1:	RAN4 shall discuss how to address the request to support non-contiguous UL CA for NR-U.

# Conclusion

This contribution discusses aspects related to for non-contiguous UL CA for NR-U and has the following proposal and observations:

Observation 1:	Non-contiguous UL CA has not been specified for NR-U.

Proposal 1:	RAN4 shall discuss how to address the request to support non-contiguous UL CA for NR-U.

# References

- RP-222174, Enhancements of NR shared spectrum bands, Apple

- RP-222105, WID Band combination table for NR Intra band CA for xCC DL_yCC UL, Ericsson