3GPP TSG-RAN WG4 Meeting # 105 -e	R4-2219089

Electronic Meeting, November 14th – 18th, 2022

Title:	TP to TR 38.717-02-01: Addition of CA_n71-n77

Source:	Nokia, Bell, Telus

Agenda item:	7.1.1.1

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-02-01 to add CA_71A-n77(3A) with UL CA.

************************************* Start of TP*****************************************

## 5.x		CA_n71-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n71+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n71+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n77(3A) | CA_n77(2A) CA_n71A-n77A | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n77.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, the 5th UL harmonic of n71 falls inside the n77 RX band.

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

There is no harmonic mixing occurring between n71 and n77.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements.

#### 5.x.1.4	∆TIB and ∆RIB values

For CA_n66-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n71-n77 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n71-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

#### 5.x.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

There’s nothing new to adding n77(2A) to these requirements. The requirements are already covered in 38.101-1.

Table 5.x.2A.1.1-1: UE Power Class for intraband non-contiguous CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77(2A) | 23 | +2/-3 |

Table 5.x.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports PC3 within an NR TDD or FDD band and supports PC2 within a second NR TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported [powerClassPerBand] if indicated or ue-PowerClass otherwise |  |  |  |  |  |  |  |  |

#### 5.x.2.2		UE co-existence

Table 5.x.2.2-1 gives IMD interference analysis for CA_ n71-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.x.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low+Ch | fy_low+MaxD | fy_high-MaxD | fy_high-Ch |
| --- | --- | --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| DL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| 2nd order IMD products | ¦fy_low+Ch – fx_low¦ | ¦fy_low+MaxD – fx_low¦ | ¦fy_low+Ch + fx_low¦ | ¦fy_high-Ch + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 600 | 6700 | 8300 |
| 3rd order IMD products | ¦2*fx_low + fy_low+Ch¦ | ¦2*fx_high + fy_high-Ch¦ | ¦2*fx_low - fy_low+MaxD¦ | ¦2*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 10000 | 12500 | 2700 | 4800 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low+Ch¦ | ¦3*fx_high + fy_high-Ch¦ | ¦3*fx_low - fy_low+MaxD¦ | ¦3*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 13300 | 16700 | 6000 | 9000 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_low+Ch¦ | ¦2*fx_low - 2*fy_low+MaxD¦ | ¦2*fx_low + 2*fy_low+Ch¦ | ¦2*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 200 | 1200 | 13400 | 16600 |
| Two-tone 5th order IMD products | ¦4*fx_low + fy_low+Ch¦ | ¦4*fx_high + fy_high-Ch¦ | ¦4*fx_low – fy_low+MaxD¦ | ¦4*fx_high – fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 16600 | 20900 | 9300 | 13200 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_low+MaxD¦ | ¦3*fx_high – 2*fy_high-MaxD¦ | ¦2*fx_low + 3*fy_low+Ch¦ | ¦3*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 2100 | 5400 | 16800 | 20800 |

Based on the table 5.x.2.2-1, the 4th order intermodulation products of n77 fall inside the RX band of n71.

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n71-n77 | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 44, 45, 50, 51, 53, 65, 66, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 2, 7, 25, 41, 34, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.x.2.3		REFSENS requirements

Table 5.x.2.3-1 lists the MSD required due to the 4th. IMD for the dual uplink configuration. The MSD value is taken from R4-2217804.

Table 5.x.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n7713 | n71 | N/A | 5 | N/A | 640 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 RBSTART=25 | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 RBSTART=25 | 3800 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

NOTE 13:	For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.

************************************* End of TP*****************************************