3GPP TSG-RAN WG4 Meeting #105	R4-2219147

Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	TP on regulatory aspects

Agenda Item:	8.18.3

Document for:	Approval

- Introduction

At RAN#94-e, a new SI on evolution of NR duplex operation was approved [1], to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. One of the RAN4 objectives is to study on the regulatory aspects.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In [3], we provide a discussion on the regulatory aspects. In this contribution, a text proposal is proposed for TR 38.858.

- Reference

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC

- “RAN1 Chair’s Notes,” 3GPP TSG RAN WG1#109-e, May 9th – May 20th, 2022.

[3]   R4-2212495, “On regulatory aspects”, Huawei, HiSilicon

11	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum

In the present the spectrum is issued with clearly defined duplex mode, i.e. FDD or TDD, and probably SDL or SUL. In 3GPP specifications, it assumes the TDD base stations deployed in the same geographical area and use the same or adjacent operating band, are synchronized. No additional co-existence requirements are covered for unsynchronized operation. To avoid cross-link interference situations, regulatory conditions at national level define the common TDD frame structures for multiple operators’ operation in the same band or administrations ask MNOs to agree on common frame structure for Macro cellular deployments.

For single operator’s TDD network, there may be no limitation on the frame structure and it is up to operator’s choice.

It already possible today to use different TDD frame structure for isolated deployment, e.g. isolated indoor factory, as long as obligation to avoid interference is guaranteed.

SBFD operation would allow simultaneous transmission and reception in different sub-bands within the same carrier. New regulatory requirements might be needed to allow SBFD operation for multiple operators’ deployment.