| 3GPP TR 38.877 V0.1.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR mmWave MB-BS (Release 18 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General	7

4.1	Study item objective	7

4.2	Deployment scenarios	7

5	Feasibility study	7

5.1	General	7

5.2	Wideband RF architectures	7

5.3	Wideband antenna architectures	7

6	Study on RF requirements	8

6.1	Definition of FR2 multi-band BS	8

6.2	Re-using FR1 multi-band methods	8

6.3	Re-using FR1 exceptions	8

6.4	FR2 specific multi-band requirements	8

Annex <A> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is the Technical Report for the Study Item on BS RF requirement evolution dealing with FR2 multi-band BS deployments.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	General

4.1	Study item objective

Study the following aspects for FR2 multi-band BS:

    - Example bands:

      - 26+28 GHz: n258 + n261

      - 28+39 GHz: n257/n261 + n260

      - 26+40 GHz: n258 + n259/n262

      - 28+40 GHz: n257/n261 + n259/n262

    - Investigate the feasibility and performance of wideband RF and antenna architectures covering multiple FR2 bands

    - Investigate if FR1 multi-band methods are re-usable for FR2, and (if so) agree on the appropriate inter-RF BW gaps

    - Investigate if FR1 exceptions are acceptable for FR2

    - Investigate whether a generic solution for all combinations within FR2-1 is possible and/or a solution for all or a part of the frequency range should be targeted

      - Frequency range 24-29 GHz which includes n257/n258/n261

      - Frequency range 37-48 GHz which includes n260/n259/n262

    - Study the definition of FR2 multi-band BS

4.2	Deployment scenarios

The scope of this study encompasses FR2 multi-band BS can support multiple FR2-1 mmWave bands transmission and/or reception through common active RF components. BS that supports multiple bands by means of separate antenna arrays and active components within the same enclosure are not in the scope of this study as they can already be supported by Rel-15 single band requirements and do not need further study.

FR2 BS capable of multi-band operation (whether those in the scope of this study, or BS supporting multiple bands by means of separate antenna arrays and active components within the same enclosure) should support evolution from single FR2-1 band to two FR2-1 bands application. Operators can configure for single band operation at one band initially and upgrade it to two bands sharing a common radio without further hardware investment.

Since a common active RF components are used for FR2 multi-band BS, the receiver and transmitter operate simultaneously would require very large isolation, which make it less possible for unsynchronized operation between bands. Hence inter-band synchronized operation is required for multi-band operation.

Possible FR2 multi-band BS structures and configurations based on combining different receiver/transmitter implementations (multi-band or single band), as well as mapping receive and transmit signals on a shared antenna or separate antennas should be considered in the study.

Scenarios 1), 4), 5) and 6) below should be considered as the target scenarios to be studied in this study while no study needed on the scenario 2) and 3) below:

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware, i.e. only one band can be configured to operate at any time.

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers two bands which have overlapping spectrums, for example, n258+n257, and BS support no overlapping frequency range in the two bands.

5	Feasibility study

5.1	General

5.2	Wideband RF architectures

5.3	Wideband antenna architectures

6	Study on RF requirements

6.1	Definition of FR2 multi-band BS

6.2	Re-using FR1 multi-band methods

6.3	Re-using FR1 exceptions

6.4	FR2 specific multi-band requirements

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4#104-e | R4-2214779 |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4#105 | R4-2219148 |  |  |  | R4-2217502 TP for deployment scenarios | 0.1.0 |