3GPP TSG-RAN WG4 Meeting # 105-e 	R4-2219164

# Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on reply LS on extending the maximum range for NS values

Agenda Item:	7.28

Document for:	Approval

1 Introduction

RAN2 inform RAN4 the progress on extending AdditionalSpectrumEmission values beyond 7 to allow for more NS values and requests RAN4 to provide feedback on some questions [1].

In this contribution, we provide some discussion and our views on these questions and suggest the proposals for a reply LS.

2 Discussion

The first question is if there are concerns with the approach of using ‘7’ as reserved value. Checking with the latest version of TS 38.101-1, it can be found that up to 5 NS values are used for an unlicensed band and up to 6 NS values for a licensed band. The detailed NS values per band can be found in following tables. So the AdditionalSpectrumEmission value ‘7’ is not used and it should be ok to use it as reserved value. Hence we can reply RAN2 that RAN4 does not have concern with the approach.

Proposal 1: the AdditionalSpectrumEmission value ‘7’ is not used in RAN4 specification and i RAN4 does not have concern with the approach of using ‘7’ as reserved value.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
| n102 | NS_58 | 20, 40, 60, 80 | 5945 – 6425 | 10 |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n18 | NS_01 | NS_100 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 | NS_57 |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n85 | NS_01 | NS_06 |  |  |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n89 | NS_01 | NS_100 |  |  |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n97 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n1042 | NS_01 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. |  |  |  |  |  |  |  |  |

The second question is the reasons and intentions for introducing the extended NS values from Rel-17. It is a Rel-18 WI and if there is no urgency to introduce in in Rel-17. The preference is to introduce it in Rel-18. Meanwhile if the urgency is clarified, we are open to introduce it in Rel-17.

Proposal 2: it is proposed to further discussion the need of introducing the extended NS values from Rel-17.

The third question is if the intention on extending the NS values is only for unlicensed bands or in general. It was discussed in RAN4 at 104e meeting and already had agreement that the extension is applicable for unlicensed band [2].

Agreement:

- Send LS to RAN WG2 asking to extend the existing range of NS values up to 32.

  - The extension is applicable for unlicensed band

- Further discuss the better solution to address the issue of running out the NS values.

Furthermore, from the above table on mapping of network signalling label, there is no need for licensed band to extend NS values. There are still some spare values for licensed band and it is not an issue since rel-15 when licensed bands are introduced. For unlicensed band the main motivation is because there are SP, LPI and VLP modes in unlicensed 6GHz, which is not the case for licensed band. Hence we suggest to reply RAN2 that the extending the NS values is only for unlicensed band.

Proposal 3: there is no need for licensed band to extend NS values and the extending the NS values is only for unlicensed band.

3 Conclusion

In this contribution, we provide some discussion and our views on these questions of RAN2 LS R2-2211064.

Proposal 1: the AdditionalSpectrumEmission value ‘7’ is not used in RAN4 specification and i RAN4 does not have concern with the approach of using ‘7’ as reserved value.

Proposal 2: it is proposed to further discussion the need of introducing the extended NS values from Rel-17.

Proposal 3: there is no need for licensed band to extend NS values and the extending the NS values is only for unlicensed band.

References

[1] R2-2211064, “Response to LS on extending the maximum range for NS values”

[2] R4-2214236, “Email discussion summary for [104-e][125] NR_unlic_enh”