# 3GPP TSG-RAN WG4 Meeting # 105                                                            R4-2219213

# Toulouse, France, November 14 – November 18, 2022

Source:	ZTE Corporation, China Unicom

Title:	TP for TR 38.896 to include HPUE single band n8

Agenda item:			7.22.2

Document for:	Approval

1. Introduction

A text proposal for TR 38.896 [1] to add the include HPUE single band n8, based on the analysis in [2][3] and discussion in [4].

2. Reference

[1] R4-2215852	TR 38.xxx v0.0.1 HPUE_NR_FR1_FDD_R18, China Unicom

[2] R4-2215661	MSD analyses for PC2 FDD bands with 2Tx, Apple

[3] R4-2215893	Discussion on PC2 band n8 RSD,  ZTE Corporation, China Unicom

[4] R4-2217769 Email discussion summary for [104-bis-e][116] HPUE_Basket_FDD, Moderator(China Unicom)

2. Text Proposal

---Start of changes---

5.x	NR band n8

5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n8 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

5.x.2	A-MPR requirements

FFS.

5.x.3	Reference sensitivity requirements

For PC2 single band n8, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n8 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | FFS | FFS | FFS | FFS | - | - | FFS | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-2, and it is proposed to adopt the average values:

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

|  | Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-2215893 | n8 | 0.5 | 0.6 | 1.2 | 5.0 | - | - | 6.7 | - | - | - |
| R4-2215661 | n8 | 0 | 0 | 0.9 | 6.2 | - | - | 8.1 | - | - | - |
| Average | n8 | 0.2 | 0.3 | 1.1 | 5.6 | - | - | 7.4 | - | - | - |
|  | NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |

---End of changes---