3GPP TSG-RAN4 Meeting #105	R4-2219277

# Toulouse, France, 14 Nov - 18 Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-4: PDCCH requirement with intra-slot repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections for requirements for FDD/TDD PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined for FDD/TDD PDCCH with intra-slot repetition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1.x (New) 5.3.2.2.x (New) 5.3.3.1.x (New) 5.3.3.2.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

##### 5.3.2.1.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.2.1.x-2, with the addition of test parameters in Table 5.3.2.1.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.1.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.5us, frequency offset = 200Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.1.x-2: Minimum performance for PDCCH with 15kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 2x2, ULA Low | 1 | [2.7] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 1>

<Start of change 2>

##### 5.3.2.2.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.2.2.x-2, with the addition of test parameters in Table 5.3.2.2.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.25us, frequency offset = 300Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.2.x-2: Minimum performance for PDCCH with 30kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-x.1 TDD | TDLA30-10 | 2x2, ULA Low | 1 | [3.5] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 2>

<Start of change 3>

##### 5.3.3.1.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.3.1.x-2, with the addition of test parameters in Table 5.3.3.1.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.3.1.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.5us, frequency offset = 200Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.3.1.x-2: Minimum performance for PDCCH with 15kHz SCS (Note 2)

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 2x4, ULA Low | 1 | [-1.2] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 3>

<Start of change 4>

##### 5.3.3.2.x	Minimum requirements for PDCCH with intra-slot repetition

The performance requirements are specified in Table 5.3.3.2.x-2, with the addition of test parameters in Table 5.3.3.2.x-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.x-1: Tests parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
|  |  | Repetition transmission schemes |  |  | FDM |  |
|  |  | CCE to REG mapping type |  |  | nonInterleaved |  |
|  |  | REG bundle size |  |  | 6 |  |
|  |  | Time offset/Frequency offset of the second TxRP from the first TxRP |  |  | timing offset = -0.25us, frequency offset = 300Hz |  |
|  |  | Frequency domain resource allocation for CORSET |  |  | Frequency non-overlapping |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Note: PDCCH is transmitted from both TRxP #1 and TRxP #2 |  |  |  |  |  |  |

Table 5.3.2.2.x-2: Minimum performance for PDCCH with 30kHz SCS

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-x.1 TDD | TDLA30-10 | 2x4, ULA Low | 1 | [-1.0] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Bandwidth, CORESET parameters, reference channel, Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 4>