3GPP TSG-RAN WG4 Meeting #105                                                    R4-2219278

Toulouse, France, 14 Nov. – 18 Nov. 2022

Agenda Item: 	6.2.6.3.1

Source:	Ericsson

Title:	Update simulation results for UE PDSCH for NTN

Document for:	Information

# 1	Introduction

According to last meeting’s agreements, the delay profile and test case design has been updated.

In this contribution, we will share our simulation results based on the updates. Simulation assumption can be found in the Annex.

# 2	Discussion

## 2.1 Ideal results for agreed test cases

Table 2.1-1: Ideal results for alignment

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | HARQ process | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | Disable HARQ | 70 | -2.1 |
| 1-2 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | HARQ 16 |  | -2.7 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | HARQ 32 |  | -2.7 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC3.5-200 | 1x2, ULA Low | HARQ 16 |  | 4.0 |

## 2.2 Impairment results for agreed test cases

Table 2.2-1 Impairment results for requirement

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | HARQ process | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | Disable HARQ | 70 | -0.1 |
| 1-2 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | HARQ 16 |  | -0.7 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | HARQ 32 |  | -0.7 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC3.5-200 | 1x2, ULA Low | HARQ 16 |  | 2.0 |

Note: DS=5ns, Doppler=200Hz, K-factor=8.05dB

# 3	Summary

In this contribution, we provided our simulation results for NTN PDSCH based on the updated delay profile and test case design.

# References

[1]	R4-2217344 WF for NTN Demodulation requirements - general and PDSCH, Qualcomm Incorporated

# Annex

Simulation assumption

Table 1: Simulation parameters for PDSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Number of SAN antennas/UE antennas |  | 1x2 |
| Carrier frequency |  | 2 GHz |
| Duplex mode |  | FDD |
| Bandwidth (MHz) / Subcarrier spacing (kHz) |  | 10 / 15 |
| Residual frequency offset |  | No FO, 0.1ppm (200Hz) |
| Channel model |  | NTN-TDLA (NLOS) NTN-TDLC (LOS) |
| Channel model k-factor |  | Refer to the test cases |
| MCS |  | 4 (QPSK) 13 (16QAM) |
| Reference measurement channel |  | R.PDSCH.1-1.1 FDD (QPSK) R.PDSCH.1-2.1 FDD (16QAM) |
| HARQ | Maximum number of HARQ transmissions | Refer to the test cases |
| DM-RS | DM-RS configuration type | Type 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Depends on RMC |
|  | Number of DM-RS CDM group(s) without data | 1 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| K_offset | Slot | 8 |