3GPP TSG-RAN WG4 Meeting #105	R4-2219326

Toulouse, France, Nov. 14 – Nov. 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2744 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on cell reselection in Idle mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The spec. specifies that UE shall initiate cell selection procedures for the selected PLMN after 10s. However, considering Rx beam sweeping factor N1 in FR2, the evaluation time Nserv and Tevaluate is much larger than 10s which implies the UE will initiate cell selection for the selected PLMN regardless of UE finishing once serving cell evaluation in FR2. DRX cycle length [s]; Scaling Factor (N1); Nserv [number of DRX cycles]; Tevaluate [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; M1*N1*4; 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 5; M1*N1*4; 5.12 x N1 (8 x N1). 1.28; 4; N1*2; 6.4 x N1 (5 x N1). 2.56; 3; N1*2; 7.68 x N1 (3 x N1). |
| Summary of change: |  | - Introduce the max function between the fixed value 10s and N1 scaling factor with DRX cycles. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may unecessarily and frequently initiate cell selection procedures for the selected PLMN causing severe performance degradation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of Change 1-----------------------------------------------------------

#### 4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE has evaluated according to Table 4.2.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for 10 s, the UE is allowed to further search any new suitable cell using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information till T = max(10s, [K1]*N1*M1*DRX cycles) before initiating cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where:

K1= [16] if DRX cycle is 0.32s, or [8] if DRX cycle is 0.64s,

otherwise K1= [4].

Table 4.2.2.2-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |
| 0.32 | 1 | 8 | 12 | M1*N1*4 |
| 0.64 |  | 5 | 8 | M1*N1*4 |
| 1.28 |  | 4 | 6 | N1*2 |
| 2.56 |  | 3 | 5 | N1*2 |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. |  |  |  |  |

------------------------------------------------------------ End of Change 1---------------------------------------------------------------