3GPP TSG-RAN WG4 Meeting#105                              R4-2219376

Toulouse, France,14th Nov – 18th Nov,2022

Agenda item:	8.25.3

Source: 	ZTE Corporation

Title: 	Further discussion on RRM requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In this contribution, we want to share further views on RRM requirement impacts from RAN4 perspective based on the latest RAN1/RAN2 agreement reached in Oct meeting..

RAN1 agreement in RAN1#110-bis-e meeting

Agreement: The following information can be used to characterize the physical beam(s) supported by NCR-Fwd for access link: - Number of beams supported for access link - FFS: How to define the detailed value (e.g., per beam type) - FFS: Whether the number of beam can be derived by beam layout - Spatial relationship between different beams - FFS: Beam types defined by the beam width (e.g. two types as wide beam and narrow beam type) - FFS: Beam direction defined by the boresight of beam - FFS: Whether/How to deliver this information to gNB - FFS: Coverage area for each beam type - FFS: Beam ID (via explicit or implicit means) Agreement: The following methods are supported for access link beam indication: - Single beam index per indication is supported to indicate one beam. - The indication of multiple beams in one indication is supported - Note: The multiple beams are applied in TDMed over same frequency resource Agreement Confirm the WA that in access link, a DL beam and a UL beam which are correspondent with each other have the same beam index. Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link Conclusion An NCR is not expected to perform forwarding in “OFF” state. Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols - FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd. Agreement Following parameters should be supported to define the time resource: - For a periodic and/or semi-persistent configured time resource, starting time, duration per beam(s) and periodicity is needed. - For aperiodic indication of time resource, starting time and duration per beam(s) is needed. - FFS: The SCS for starting time, duration, and periodicity - FFS: How to define the duration, e.g., via the length of time resource or resource index(es) - FFS: Whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication and the first OFDM symbol in the slot) Agreement If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - Predefined rule is used to define the beam in case there is no indication via the new signalling - FFS: Details of the predefined rule - FFS: Application of predefined rule for other cases - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. - The new signalling, if needed, is an optional NCR capability Agreement For the ON/OFF information indication, at least one of following options is supported to indicate the ON state of NCR - Alt-1: Explicit indication with dedicated field to indicate ON state - Note: At least it’s supported when the beam indication is not applicable - Alt-2: Implicit indication via the beam indication - Alt-3: Indication via the time domain resource indication (i.e., the NCR is assumed to be ON over the indicated time domain resource)

Agreement: For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused. - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports. Agreement: To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement: HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported. Agreement: PUCCH and PUSCH are supported for NCR-MT. Agreement: The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link. Agreement: To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported.

RAN2 agreement in RAN1#119-bis-e meeting

Agreement: RAN2 confirms to use RRC signalling to configure NCR-MT to receive side control information. How the side control information itself is transmitted (i.e. via RRC or DCI or MAC CE) is up to RAN1 (RAN2 may discussion the initial RAN1 decision and revisit if needed).

Agreement: NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support)

Agreement NCR-MT supports SRB0/1/2 and DRB is optional. FFS on maximum number of DRBs.

Agreements RRM functions supported by NCR-MR: - Cell selection is mandatory - Cell reselection, RLM, BFD, BFR are FFS

Discussion

## 2.1. RRM requirements for NCR-MT

First of all, from RRM perspective, as mentioned in the previous section 2.1, the NCR-MT entity should not only received the side control information from gNB, but also need to maintain the control link between DL input of NCR and the parent gNB. Indeed from our understanding, NCR-MT is somehow similar as low cost IAB-MT to receive the limited side control information from parent gNB instead of providing the backhaul transmissions. During the last RAN4 meeting, there were some initial discussions for NCR type and NCR class in RF sessions, the following agreement has been reached. Indeed this is just similar as Rel-16 IAB class definition.

Issue 2-2 NCR type and NCR class:

- Aagreements:

- For NCR-fwd link in Rel-18, at least reuse the Rel-17 repeater type/Class i.e.,

  - Repeater Type: 1-C and 2-O;

  - Repeater class: wide-area and local area (DL, UL), medium range for DL

- FFS for repeater type 1-H and 1-O for FR1 repeater given the beamforming capability assumption;

- The baseline assumption that same type/class applied for NCR-fwd link and NCR-MT

To follow the similar principle for IAB-MT RRM requirement, RRM requirement is not applicable for wide area NCR-MT and only applicable for Local area NCR-MT since its expected propagation channel between gNB and wide area NCR-MT should be stable enough without causing too much radio link failure etc.

Proposal 1: RRM requirement is only applicable for Local area NCR-MT and not applicable for wide area NCR-MT.

In the following section, we provide the initial analysis for RRM requirement of NCR-MT on top of Rel-15 NR UE RRM requirement and its details could be found as following.

During the last RAN2 meeting, RAN2 has reached following agreement which might have the impacts on RAN4 RRM requirement.,however from RAN4 RRM perspective, there are no specific RRM requirement defined for cell selection.

Therefore it is not expected to define any RRM requirement for cell selection of NCR-MT.

Agreement: NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support)

Agreements RRM functions supported by NCR-MR: - Cell selection is mandatory - Cell reselection, RLM, BFD, BFR are FFS

Proposal 2: not not define RRM requirement for cell selection of NCR-MT;

During the last RAN1/2 meeting, the following agreement has been reached for NCR-MT and its uplink transmission, therefore the transmit timing and its timing advance requirement are needed similar as Rel-16 IAB-MT.

Agreement: NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support)

Agreement: PUCCH and PUSCH are supported for NCR-MT.

Agreement: To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. Note: This does not mean all legacy UE sounding procedure will be supported.

Proposal 3: to define transmit timing requirement for NCR-MT and reuse the same requirement as Rel-16 IAB-MT.

In this contribution, we want to share some initial discussions on these requirements for

Table 1. Technical justification for RRM requirement of local area NCR-MT

| Requirements | Detailed RRM Scope | Is requirement needed for Rel-18 NCR- MT? | Technical justifications |
| --- | --- | --- | --- |
| Cell reselection | - Intra-frequency - Inter-frequency | FFS | wait for RAN2 progress |
| Handover |  | FFS | wait for RAN2 progress |
| RRC Connection Mobility Control | - RRC re-establishment - Random access - RRC release with redirection | FFS | wait for RAN2 progress |
| SCell activation/deactivation delays |  | No | For Rel-18 NCR-MT, single carrier operation should be sufficient for the reception of the side control information defined by RAN1/2/3. Don’t see the strong necessity to have Scell configuration and activation. |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No | Similar reason as Scell activation/deactivation of CA. |
| PSCell addition/release delays |  | No | NR-DC is not expected to be supported for Rel-18 NCR-MT. |
| Active BWP switching | - Interruptions at active BWP switching - Active BWP switching delay | No | BWP switching for Rel-18 NCR-MT is not expected to happen frequently and don’t see its strong necessity. |
| TCI switching | TCI switching delay | FFS | For Rel-16 IAB-MT, there are no such kind of TCI switching requirement defined for it. However for Rel-18 NCR-MT, it’s supposed to support the adaptive beamforming in C-link, therefore it might be beneficial to define the corresponding TCI switching delay requirement for NCR-MT. |
| RLM | Requirements for RLM, e.g., based on SSB, CSI-RS or both | FFS | wait for RAN1/2 progress |
| Link recovery procedures | Requirements for SSB-based and CSI-RS based beam failure detection and candidate beam detection | FFS | wait for RAN1/2 progress |
| Measurements requirements (time duration, number of cells, etc.) |  | No | Measurement requirements are not that important to Rel-18 NCR-MT due to its fixed deployment. |
| Measurement accuracy requirements |  | No | Similar as Rel-16 IAB-MT. No requirement is needed different from the normal UE. |
| MT timing related requirements | - MT MRTD/MTTD - MT transmit timing - MT timing advance | Yes | The same timing related requirement as Rel-16 IAB-MT shall be applied to Rel-18 NCR-MT. MT MRTD and MTTD is not applicable for NCR-MT. |

Conclusions

In this contribution, we want to share further views on NCR-MT from the RRM perspective and proposals are made as following:

Proposal 1: RRM requirement is only applicable for Local area NCR-MT and not applicable for wide area NCR-MT.

Proposal 2: not to define RRM requirement for cell selection of NCR-MT;

Proposal 3: to define transmit timing requirement for NCR-MT and reuse the same requirement as Rel-16 IAB-MT.

Proposal 4: to further further the proposal in Table 1 for RRM requirement of NCR-MT.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- RP-222323, TR 38.867 for NR Network-Controlled repeater, RAN#97-e

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.