3GPP TSG-RAN WG4 Meeting # 105	R4-2219506

# Toulouse, France, November 14 – November 18 ,2022

Title: 	Simulation results on FR2-2 CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.2.3

Document for:	Discussion

Background

As per [1], we provide our simulation results for FR2-2 CQI requirements in this paper.

Simulation results

The simulation results for CQI distribution and BLER requirements are captured in Table 2-2 (Without Phase noise modelled) and Table 2-3(With Phase noise) with the parameters specified in Table 2-1.

Table 2-1: Simulation results for FR2-2 CQI test

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | DDDSU S=10D:2G:2U |
| Propagation channel |  |  |  | AWGN |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| MIMO layers |  |  |  | 2 |
| Phase noise |  |  |  | Set 2 in TR 38.808 |

Table 2-2: Simulation results without phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100% | 0 | 0 | 100% |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 87.5% |
| 12 | 10 | 100% | 0 | 0 | 88.9% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 0 | 24.0% |
| 18 | 13 | 100% | 0 | 0 | 53.8% |

Table 2-3: Simulation results with phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 0 | 100% | / | / | 0.07 |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 100% |
| 12 | 10 | 100% | 0 | 0 | 100% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 11% | 100% |
| 18 | 13 | 100% | 0% | 100% | 100% |

Conclusion

In this paper we provide our simulation results for FR2-2 CQI requirements.

Reference

[1]   R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated