3GPP TSG-RAN4 Meeting #105	R4-2219543

# Toulouse, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2763 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on requirements for UE Rx-Tx measurement for PDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - PDC measurement requirements need account for PDC RS resources overlap with measurement gaps or SMTC. - It is FFS in the DRX requirements whether LCM or MAX should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce scaling factor in PDC measurement requirements to account for PDC RS resources overlap with measurement gaps or SMTC. - Use LCM in the DRX requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDC measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on agreed CR R4-2217238 in RAN4#104-bis-e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.12.4	Measurement period requirements

9.12.4.1	PRS Measurement Period

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*fLCM(TPRS, TDRX)Teffect

Where:

NPRSslot is the maximum number of DL PRS resources configured in a slot,

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to [maxNumOfDL-PRS-ResProcessedPerSlot] as specified in [TBD],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

Teffect

Teffect=TPRS

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length in use.

Klayer3 is

Ntotal / Noutside_MG for FR1, and no requirements apply if Noutside_MG=0, and

Ntotal / Navailable for FR2, and no requirements apply if Navailable=0

Where,

-	For a window W of duration max(TPDC-RS,  MGRP_max, TSMTC), where TPDC-RS is periodicity of the target PDC-RS, MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, TSMTC is the periodicity of SMTC for intra-frequency measurement, and starting at the beginning of any PDC-RS resource occasion:

-	Ntotal is the total number of PDC-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of PDC-RS resource occasions that are not overlapped with any measurement gap occasion within the window W, and

-	Navailable is the number of PDC-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

If PRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

9.12.4.2	TRS Measurement Period

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*fLCM(TTRS, TDRX)TTRS

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length in use.

Klayer3 is

Ntotal / Noutside_MG for FR1, and no requirements apply if Noutside_MG=0, and

Ntotal / Navailable for FR2, and no requirements apply if Navailable=0

Where,

-	For a window W of duration max(TPDC-RS,  MGRP_max, TSMTC), where TPDC-RS is periodicity of the target PDC-RS, MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, TSMTC is the periodicity of SMTC for intra-frequency measurement, and starting at the beginning of any PDC-RS resource occasion:

-	Ntotal is the total number of PDC-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of PDC-RS resource occasions that are not overlapped with any measurement gap occasion within the window W, and

-	Navailable is the number of PDC-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

<End of Change 1>