3GPP TSG-RAN WG4 Meeting #105	R4-2219560

Toulouse, France, 14 – 18 November, 2022

Title: 	Reply LS on capability for PRS measurement without MG

Source: 	Huawei, HiSilicon

Agenda item:	10.2.1.2

Document for:	Approval

Introduction

RAN4 received LS [1] from RAN2#119-e related to UE capability for PRS measurement without MG FG 14-3. RAN2 asks RAN4 whether FG 14-3 should be reported also to LMF.

In RAN4 feature lists, the feature 14-3 “PRS measurement without MG” is only indicated as “Need for the gNB to know”. It is unclear whether there is also a need for the LMF to know about UE support for feature 14-3, i.e. should this capability also be captured in LPP specification or not. Some companies in RAN2 think this capability can be used for the LMF to determine which TRPs should be included in the PRS configuration. Besides, RAN2 has already captured its prerequisite feature 27-3-2 “DL PRS measurement outside MG and in a PRS processing window” in the UE capability of LPP. Therefore RAN2 would like to ask RAN4 to clarify whether the LMF also needs to know the UE capability on feature 14-3.

In this paper we will provide our views on the question in the LS and a draft reply.

Discussion

The definition of FG 14-3 is as follows.

Based on RAN4 requirements, UE is not required to measure PRS resources from a non-serving cell outside MG if the expected RTD relative to the serving cell is larger than the threshold supported by the UE, which is indicated via FG 14-3. It is noted that the expected RTD is also calculated by UE based on the assistance data. We think it makes sense for LMF to know UE capability for FG 14-3. As mentioned in [1], LMF may use this information to determine which TRPs should be included in the PRS configuration and generate the assistance data accordingly.

Proposal: RAN4 to clarify that LMF also needs to know the UE capability on feature 14-3.

Conclusions

In this paper we provided our views on the question in the LS [1].

Proposal: RAN4 to clarify that LMF also needs to know the UE capability on feature 14-3.

A draft reply LS is provided in the Annex.

Reference

- R2-2209241, LS to RAN4 on capability for PRS measurement without MG, RAN2

Annex: draft Reply LS on capability for PRS measurement without MG

3GPP TSG-RAN WG4 Meeting #105-e R4-22xxxxx Toulouse, France, 14 – 18 November, 2022 Title: Reply LS on capability for PRS measurement without MG Response to: R2-2209241 Release: Release 17 Work Item: NR_pos_enh-Core Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 would like to thanks RAN2 for the question in LS R2-2209241. Based on discussion in RAN4#105, RAN4 concluded that LMF also needs to know the UE capability on feature 14-3. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #106 Feb 27 – Mar 3, 2023 Athens, Greece RAN WG4 Meeting #106-bis-e Apr 17 – Apr 26, 2023 Online