3GPP TSG-RAN WG4 Meeting #105                                R4-2219617

Toulouse, France, 14 November– 18 November, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF architecture and IMD evaluation for CA_n5-n28

Agenda Item:	8.2.3.1

Document for:	Approval

Introduction

In RAN4#104bis meeting, a WF [1] for CA_n5-n28 was approved with the following outcomes.

Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28

Agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting.

Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis

Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 3-3: RF parameters for requirements analysis

Agreements: Companies are encouraged to provide the RF parameters when the feasibility study is conducted.

Issue 3-4: Other observations and proposals

Agreements: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5.

- Architecture assumption and multiplexer isolation performance needs to be clarified

Referring to the contributions [2, 3, 4], RAN4 has reached some agreements on CA_n5-n28 in Rel-17. Most of them are still valid in Rel-18. In this paper, we’d like to provide our views on RF architecture assumption and MSD due to cross band isolation for CA_n5-n28.

Discussion on RF architecture

Referring to contribution [3], the Quadplexers have to be used for CA_n5-n28 two antennas RF architecture as figure 1 and 45dB Tx-Rx isolation between n5 Tx and n28 Rx is assumed.

Quadplexer A: n28A + n5 => 703~733MHz, 758~788MHz, 824~869MHz, 869~894MHz.

Quadplexer B: n28B + n5 => 718~748MHz, 773~803MHz, 824~869MHz, 869~894MHz.

Switch

Quadplexer A

Quadplexer B

Diplexer

Mid bands

n5+n28 Rx

DUP

Diplexer

Switch

Figure 1 two antennas RF architecture for CA_n5-n28

For 3 antenna RF architecture, the first main path can include n28A + n28B implementations. And the second path can include band n5 duplexer as figure 2. For 3 antenna RF architecture, current general duplexer for both band n28 and n5 can be used. Additionally, 35dB ~ 40dB rejection of band n5 Tx filter can be considered at band n28 Rx range 758~803MHz.

Switch

Duplexer n28B

Diplexer

Mid bands

n5+n8 Rx

DUP

Diplexer

Switch

Duplexer n28A

Switch

Duplexer n5

Diplexer

Mid bands

Figure 2 three antennas RF architecture for CA_n5-n28

Proposal 1: Figure 1 and figure 2 can be considered as two and three antennas RF architecture for CA_n5-n28.

MSD due to cross band isolation for CA_n5-n28

Referring to contribution [3], the n28 MSD evaluation for CA_n5-n28 is shown as below.

Figure 3 the n28 MSD evaluation for CA_n5-n28

Considering the impacts from both band n28 UL and n5 UL, we can specify one more MSD test configurations with the following requirements.

Table 1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 888 | 30 | 2.9 | ACLR2 |

Proposal 2: it’s proposed to specify MSD test configurations due to cross band isolation for CA_n5-n28 in table 1.

Summary

Proposal 1: Figure 1 and figure 2 can be considered as two and three antennas RF architecture for CA_n5-n28.

Switch

Quadplexer A

Quadplexer B

Diplexer

Mid bands

n5+n28 Rx

DUP

Diplexer

Switch

Figure 1 two antennas RF architecture for CA_n5-n28

Switch

Duplexer n28B

Diplexer

Mid bands

n5+n8 Rx

DUP

Diplexer

Switch

Duplexer n28A

Switch

Duplexer n5

Diplexer

Mid bands

Figure 2 three antennas RF architecture for CA_n5-n28

Proposal 2: it’s proposed to specify MSD test configurations due to cross band isolation for CA_n5-n28 in table 1.

Table 1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 888 | 30 | 2.9 | ACLR2 |

- References

[1] R4-2217709, WF on CA_n5-n28, ZTE

[2] R4-2119869, TP to TR 38.717-02-01 Addition of CA_n5-n28, Nokia

[3] R4-2202036, Measurements for CA_n5-n28 MSD, Skyworks Solutions, Inc.

[4] R4-2206134, CR to R17 TS38.101-1 on MSD for CA_n5-n28, Skyworks Solutions Inc.