3GPP TSG-RAN WG4 Meeting #105 	R4-2219729

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues for PUSCH demodulation requirements in FR2-2

Agenda item:	6.3.6.3.1

Document for:	Discussion

Introduction

This paper presents discussion on remaining details for PUSCH demodulation requirements for FR2-2.

Discussion

As part of the PUSCH requirements, AWGN levels are defined as in the table [2].

Table 8.2.1.4.2-2: AWGN power level at the BS input BS type; Sub-carrier spacing (kHz); Channel bandwidth (MHz); AWGN power level. BS type 1-O (Note 4); 15; 5; -86.5 - ΔOTAREFSENS dBm / 4.5 MHz. 10; -83.3 - ΔOTAREFSENS dBm / 9.36 MHz. 20; -80.2 - ΔOTAREFSENS dBm / 19.08 MHz. 30; 10; -83.6 - ΔOTAREFSENS dBm / 8.64 MHz. 20; -80.4 - ΔOTAREFSENS dBm / 18.36 MHz. 40; -77.2 - ΔOTAREFSENS dBm / 38.16 MHz. 100; -73.1 - ΔOTAREFSENS dBm / 98.28 MHz. BS type 2-O (Note 5); 60; 50; EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52 MHz. 100; EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz. 120; 50; EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 46.08 MHz. 100; EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz. 200; EISREFSENS_50M + ΔFR2_REFSENS + 21 dBm / 190.08 MHz. NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. NOTE 4: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. NOTE 5: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level..

Update AWGN power level considering the following cases:

  - For 120 kHz kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

  - For 480 kHz kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

Another points where no discussion was held is regarding PTRS ports configuration. It is important that we ge the same configuration specially for the 2x2 cases. Since it is not expected that PTRS would not be used for MIMO, and PN and FO would be shared between all TX/RX branches, there is no gain expected in using 2 PTRS ports.

Adopt 1 PTRS port for PUSCH demodulation requirements.

Conclusion

In the paper, the following Proposals were made:

- Update AWGN power level considering the following cases:

  - For 120 kHz kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

  - For 480 kHz kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

Adopt 1 PTRS port for PUSCH demodulation requirements.

- References

- R4-2217374, WF on PRACH demodulation requirements for FR2-2, Samsung

- TS 38.141-2, NR; Base Station (BS) conformance testing, Part 2: Radiated conformance testing