3GPP TSG-RAN WG4 Meeting # 105                	R4-2219735

Toulouse, France, November 14 – 18, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PRACH simulation results for demodulation requirements for the extension to 71 GHz

Agenda item: 	6.3.6.3.3

Document for: 	Discussion

Introduction

This paper presents PRACH simulation results for the demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Results

Table 1 Simulation parameters

| Parameter | Value |
| --- | --- |
| Channel model | AWGN and TDLA 30-650 for 120 kHz AWGN and TDLA 10-650 for 480 kHz |
| PRACH formats | A2, B4, C2 |
| SCS | 120, 480 |
| PRACH sequence length LRA | 139 for 120 and 480 kHz SCS 571 for 120 and 480 SCS 1151 for 120 kHz SCS |
| Carrier frequency | 71 GHz |
| Logical sequence index | 0 |
| V | 1 |
| NCS | -SCS=120, LRA=139, NCS=69 -SCS=120, LRA=571, NCS=285 -SCS=120, LRA=1151, NCS=575 -SCS=480, LRA=139, NCS=69 -SCS=480, LRA=571, NCS=285 |

Table 2 Time error tolerance used in simulations

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA30-650 | 120 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | 68 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | 59 |

Table 3 PRACH simulation results

|  |  |  |  |  |  |  | SNR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS kHz | Prach sequence length | Ncs | Propagation condition | Frequency Offset (Hz) | Time estimation tolerance | Format | ideal | impair |
| 120 | 139 | 69 | AWGN | 0 | 70 ns | A2 | -14.33 | -11.03 |
|  |  |  |  |  |  | B4 | -18.66 | -15.36 |
|  |  |  |  |  |  | C2 | -14.31 | -11.01 |
|  |  |  | TDLA30-650 | 7100 | 120 ns | A2 | -6.22 | -2.92 |
|  |  |  |  |  |  | B4 | -9.77 | -6.47 |
|  |  |  |  |  |  | C2 | -6.32 | -3.02 |
|  | 571 | 285 | AWGN | 0 | 70 ns | A2 | -20.57 | -17.27 |
|  |  |  |  |  |  | B4 | -24.87 | -21.57 |
|  |  |  |  |  |  | C2 | -20.58 | -17.28 |
|  |  |  | TDLA30-650 | 7100 | 120 ns | A2 | -13.87 | -10.57 |
|  |  |  |  |  |  | B4 | -16.83 | -13.53 |
|  |  |  |  |  |  | C2 | -13.76 | -10.46 |
|  | 1151 | 575 | AWGN | 0 | 70 ns | A2 | -23.63 | -20.33 |
|  |  |  |  |  |  | B4 | -27.87 | -24.57 |
|  |  |  |  |  |  | C2 | -23.61 | -20.31 |
|  |  |  | TDLA30-650 | 7100 | 120 ns | A2 | -16.99 | -13.69 |
|  |  |  |  |  |  | B4 | -20.00 | -16.7 |
|  |  |  |  |  |  | C2 | -16.95 | -13.65 |
| 480 | 139 | 69 | AWGN | 0 | 18 ns | A2 | -14.27 | -10.97 |
|  |  |  |  |  |  | B4 | -18.66 | -15.36 |
|  |  |  |  |  |  | C2 | -14.31 | -11.01 |
|  |  |  | TDLA10-650 | 7100 | 68 ns | A2 | -6.72 | -3.42 |
|  |  |  |  |  |  | B4 | -11.39 | -8.09 |
|  |  |  |  |  |  | C2 | -6.75 | -3.45 |
|  | 571 | 285 | AWGN | 0 | 18 ns | A2 | -20.58 | -17.28 |
|  |  |  |  |  |  | B4 | -24.83 | -21.53 |
|  |  |  |  |  |  | C2 | -20.52 | -17.22 |
|  |  |  | TDLA10-650 | 7100 | 68 ns | A2 | -13.99 | -10.69 |
|  |  |  |  |  |  | B4 | -18.20 | -14.9 |
|  |  |  |  |  |  | C2 | -14.04 | -10.74 |

References

- R4-2217371, WF on general aspects for FR2-2 demodulation requirements, Huawei,HiSilicon

- R4-2217374, WF on PRACH demodulation requirements for FR2-2, Samsung

- R4-2212679, PRACH simulation results for demodulation requirements for the extension to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation