3GPP TSG-RAN WG4 Meeting #105                                                             R4-2219757

# Electronic Meeting, Oct 10 - 19, 2022

Source:	ZTE Corporation

Title:	TP for TR 37.718-00-00 on delta TIB and RIB template for SUL combinations

Agenda Item:	7.13.2

Document for:	Approval

- Introduction

In [1], new templates for delta TIB and RIB due to inter-band CA/DC band combinations have been agreed to introduce in Rel-18. To unify the template as the cases in CA/DC configurations, it is suggested to apply the delta TIB / RIB template for SUL configurations in Rel-18.

This TP is to apply the delta TIB template for SUL configurations in TR 37.718-00-00 [2].

- Reference

- R4-2213600, Simplification of templates for delta TIB and RIB for band combinations, ZTE

- TR 37.718-00-00 v0.2.0 Rel-18 Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP), Huawei

Text Proposal

----- Start of TP -----

### 5.1.6	∆TIB and ∆RIB values

For CA_n1_SUL_n78-n81, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n8-n78.

Table 5.1.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1_SUL_n78-n81 | 0.3 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.1.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1_SUL_n78-n81 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

<< Unchanged sections omitted >>

### 5.3.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.3.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n98 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.3.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n98 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

<< Unchanged sections omitted >>

### 5.4.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.4.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n98 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.4.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n98 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

<< Unchanged sections omitted >>

### 5.5.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n95, the TIB,c and RIB,c values are given in the tables below.

Table 5.5.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n95 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.5.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n41_SUL_n79-n95 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

<< Unchanged sections omitted >>

### 5.6.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n95, the TIB,c and RIB,c values are given in the tables below.

Table 5.6.6-1: ΔTIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n95 | 0.3 | 0.8 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

Table 5.6.6-2: ΔRIB,c due to SUL band combination (three bands)

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n79_SUL_n41-n95 | - | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

<< Unchanged sections omitted >>

5.Y.6	∆TIB and ∆RIB values

Table 5.Y.6-1: ΔTIB,c due to SUL band combination

| SUL Band combination | ΔTIB,c for NR bands / SUL band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for SUL_n41-n81 the band order from left to right is n41 and n81. |  |  |

Table 5.Y.6-2: ΔRIB,c due to SUL band combination

| SUL Band combination | ΔRIB,c for NR bands / SUL band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| SUL_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for SUL_n41-n80 the band order from left to right is n41 and n80. |  |  |

----- End of TP -----