3GPP TSG-RAN WG4 Meeting #105	 R4-2219773

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	DL-PRS measurements with reduced samples capability (R1-2207940, R2-2208797)

Agenda item:	10.2.1.1

Document for:	Discussion

Introduction

For the design of DL-PRS measurement, RAN2#119-e provided the LS on the UE features for DL-PRS measurements with reduced samples capability [1]:

RAN2 observes that the UE capability of reduced samples of DL-PRS measurement has different granularities in different RRC states according to the RAN1 and RAN4 UE features list. To be specific, the RAN1’s UE feature 27-3-1: M-sample measurements in RRC_CONNECTED is a per band capability: 27-3-1; M-sample measurements in RRC_CONNECTED; The capability to support reporting a measurement based on measuring M=1 or 2 samples (instances) of a DL PRS resource set; If the UE does not provide the capability, the UE is assumed to support M=4 only; per band. RAN4’s UE feature 14-2: PRS measurement for reduced sample in RRC_INACTIVE state is a per UE capability: 14-2; PRS measurement for reduced sample in RRC_inactive state; Capability of supporting reduced number of samples (M=1, 2) for PRS measurement in RRC_inactive state; 27-17; The reduced number of samples (M=1,2) for PRS measurement in RRC_inactive state cannot be supported. The UE is assumed to support M=4 only.; Per UE. Therefore, RAN2 requires a clarification on whether a unified design in terms of the granularity of the UE capability for reduced samples of PRS measurement in the UE feature 27-3-1 from RAN1 and the UE feature 14-2 from RAN4 is needed, i.e., whether to change the type of 27-3-1 to per UE (RAN1’s responsibility) or to change the type of 14-2 to per band (RAN4’s responsibility).

The RAN2 LS [1] was discussed in RAN1#110 [2]. RAN1 prefers that FG 27-3-1 is reported per band considering UE implementation flexibility. Also, RAN1 recommends RAN4 to change FG 14-2 to per band [2]:

# 1. Overall Description: RAN1 would like to thank RAN2 for the LS R2-2208797(R1-2207812) on DL-PRS measurements with reduced samples capability. RAN1 discussed the question in the LS and thinks a unified design is desirable. However, RAN1 technically thinks FG 27-3-1 should be reported per band considering UE implementation flexibility. So RAN1 recommends RAN4 to change FG 14-2 to per band. # 2. Actions: To RAN2 and RAN4: RAN1 kindly asks RAN2 and RAN4 to take the above responses into account.

In the following section, UE feature support for DL-PRS measurements with reduced samples capability is discussed.

Discussion

Regarding the UE feature on the DL-PRS measurements with reduced samples, in our view, a unified design in the granularity of the UE capability is required. Therefore, the following proposal is made.

RAN4 to change FG 14-2 to per band applicability according to RAN1 request.

- References

- R2-2208797 (R1-2207812), “LS on DL-PRS measurements with reduced samples capability,” RAN2

- R1-2207940, “Reply LS on DL-PRS measurements with reduced samples capability,” RAN1