3GPP TSG-RAN WG4 Meeting # 105	R4-2219839

# Electronic Meeting, November 14 – 18, 2022

Source: 	Qualcomm Incorporated

Title: 	Updated Simulation Results for FR2-2 UE Demodulation PDSCH

Agenda item:	6.3.6.2.1

Document for:	Discussion

- Introduction

In RAN4#104-bis-e [2], requirement tables and partial configurations were agreed for PDSCH demodulation for UEs that support FR2-2[1]. In this paper, we share our alignment and impairment results for the definition of the proposed PDSCH requirements.

# PDSCH Alignment and Impairment Results

Based on the agreed simulation assumptions and the options provided in [3] for the requirements definition, we provide here the PDSCH SNR Alignment results for FR2-2 120kHz TDD test cases, at 70% of peak rate.

| SCS(kHz)/ CBW(MHz) | MCS | Rank | Propagation Channel | Antenna configuration | Tput (% of peak) | Num PRBs | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz/ 100MHz | 4 | 1 | TDLA30-650 | 2x2 ULA low | 70 | 66 | -2 | 0.5 |
|  | 13 | 1 | TDLA30-200 | 2x2 ULA low | 70 | 66 | 6.2 | 8.7 |
|  | 13 | 1 | TDLA30-650 | 2x2 ULA low | 30 | 66 | 0.1 | 2.6 |
|  | 17 | 1 | TDLD30-200 | 2x2 ULA low | 70 | 32 | 8.2 | 10.7 |
| 480kHz/ 400MHz | 4 | 1 | TDLA10-200 | 2x2 ULA low | 70 | 66 | -1.5 | 1 |
|  | 13 | 1 | TDLD10-200 | 2x2 ULA low | 70 | 20 | 5 | 7.5 |

- Conclusions

Alignment and Impairment SNR results for FR2-2 PDSCH demodulation requirements are included in this contribution.

- References

[1] RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, 3GPP TSG RAN Meeting #94-e, Electronic Meeting, December 06-17, 2021

[2] R4-2217391, “WF on FR2-2 UE demodulation requirements”, 3GPP TSG-RAN WG4 Meeting #104-bis-e, Electronic Meeting, October 10 – 19, 2022

[3] 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs (Release 18, v18.2.0)”;