3GPP TSG-RAN WG4 Meeting #105	R4-2219860

# Toulouse, France, 14th – 18th November 2022

Source: 		Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.892: BS requirements for APT 600 MHz NR band

Agenda Item: 	7.27.3

Document for:	Approval

# Introduction

This document provides the TP to TR 38.892 on BS requirements for APT 600 MHz NR band.

# TP to TR 38.892

## 7.2	BS requirements

Below are agreed changes (excluding system parameters) to 38.104 due to introduction of APT 600 MHz NR band:

##### 6.6.4.2.1	Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, n105, basic limits are specified in table 6.6.4.2.1-1.

6.6.4.2.2.1	Category B requirements (Option 1)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, n105, the basic limits are specified in table 6.6.4.2.2.1-1:

##### 6.6.5.2.3	Additional spurious emissions requirements

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105 |
|  | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105, since it is already covered by the requirement in clause 6.6.5.2.2. |
| NR Band n105 | 612 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or n105. |
|  | 663 – 703 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n105, since it is already covered by the requirement in clause 6.6.5.2.2. |

##### 6.6.5.2.4	Co-location with other base stations

| Type of co-located BS | Frequency range for | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  | co-location requirement | WA BS | MR BS | LA BS | bandwidth |  |
| NR Band n105 | 663 – 703 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |