3GPP TSG-RAN WG4 Meeting # 105 	R4-2220007

Toulouse, France, November 14 – November 18, 2022

Source:	T-Mobile USA

Title:	TP for TR38.899 for DL CA_n25-n71-n77 with PC2 2BUL

Agenda item:	7.19.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 3BDL/2BUL for c combinations with CA_n25-n71-n77 according to the request in [1].

The power class 2 MSD is calculated by calculating the interference power for PC3 MSD, increasing the interference by 3 dB, and then calculating the resulting MSD for PC2. The equations are included.

- Reference

[1] RP-222623, New WID: New WID: High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m 	(m<3) and high power on TDD band(s)

- Text Proposal

<Beginning of changes>

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.x	DL CA_n25-n71-n77, UL CA_n25A-n77A

### 6.x.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.x.3	REFSENS requirements

#### 6.x.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n25A-n77A, so there should be none for PC2.

#### 6.x.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.x.4	∆TIB and ∆RIB values

Void.

## 6.y	DL CA_n25-n71-n77, UL CA_n71A-n77A

### 6.y.1	Configurations

Table 6.y.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.y.2	Maximum output power

Table 6.y.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.y.3	REFSENS requirements

#### 6.y.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.y.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.y.3.1-2.

Table 6.y.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 19.5 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |

#### 6.y.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.y.4	∆TIB and ∆RIB values

Void