3GPP TSG-RAN WG4 Meeting # 105											                              	R4-2220110

Toulouse, France, November 14th – November 18th , 2022

Agenda item:			8.7.4

Source:	Moderator (Xiaomi)

Title:	Topic summary for [105][130] FR2_enh_req_Ph3_part2

Document for:	Information

Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

List of candidate target of discussions for this topic.

- 1st round: TBA

- 2nd round: TBA

The contributions for the following agenda items are summarised in this document:

8.7.2 UL 256QAM

Topic #1: EVM requirements for UL 256QAM

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2220036 | Ericsson Limited | Proposal 1: For 29 GHz AWGN channel with 3.5% EVM, the operating SNR for 256QAM MCS21 should be equal to 27.7 dB. Proposal 2: For 29 GHz TDL-D channel with 3.5% EVM, the operating SNR for 256QAM MCS21 should be equal to 30.4 dB. Proposal 3: Proposed method for limiting the MCS for 39 GHz consists of selecting the max MCS which satisfies the following criteria: performance degradation due to phase noise is less than XdB (e.g. X=1.5 dB since 256QAM is assumed) with the agreed PN compensation technique (CPE compensation), i.e. performance degradation is less than XdB between the cases of “No PN” and “PN with CPE compensation”. Proposal 4: If only CPE compensation method is used (with no ICI compensation) and having in mind the test implementation, it is reasonable to stick with a Rel-15 PTRS configuration of K=2, L=1 only. Proposal 5: 256 QAM for PC1 UEs in FR2-1 and 29 GHz carrier frequency is feasible. |
| R4-2218326 | Nokia, Nokia Shanghai Bell | Observation 1: For the simulation results at 48 GHz (n262): - 256QAM does not provide any throughput gain over 64QAM below 40 dB SNR. |
| R4-2218327 | Nokia, Nokia Shanghai Bell | Observation 1: For the simulation results in urban macro scenario: - The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. - ~15% of PC1 UE and ~25% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). Observation 2: For the simulation results in indoor scenario: - The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. - Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). Conclusion: - In all simulated scenarios, a significant percentage of PC1/PC2/PC5 UE can achieve the target SNR at BS side above which 256QAM provides throughput gain over 64QAM at both 29 GHz (n257) and 39 GHz (n260). |
| R4-2218328 | Nokia, Nokia Shanghai Bell | Proposal 1: To limit MCS with 256QAM for 39GHz to 21 and 22 in Table 5.1.3.1-2 in TS 38.214. Proposal 2: To adopt 28 dB at 29GHz and 25 dB at 39GHz as the target operating SNR. Proposal 3: To adopt option 2 to use a fixed PTRS configuration for all devices for the EVM test. Proposal 4: Opponents of using the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis should provide concrete alternative proposal to be discussed in RAN4 to progress the discussion in this topic. |
| R4-2218595 | ZTE Corporation | Observation 1: For 39GHz: UL 256QAM performance gain can be expected in the following cases: - AWGN, TDL-D and TDL-A channel when MCS21(256QAM)/MCS23(64QAM) are selected - AWGN, TDL-D and TDL-A channel when MCS22(256QAM)/MCS24(64QAM) are selected Observation 2: UL 256QAM performance gain can not be expected for 48GHz Proposal 1: To add Min peak EIRP in the system level simulation assumption. UE Min peak EIRP; n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm n260 PC1:38.0 dBm/PC3:20.6 dBm. Observation 3: PC1, PC2, PC3 and PC5 are feasible for UL 256QAM for 29GHz if the operating SNR is not higher than 28dB. |
| R4-2218681 | LG Electronics France | Observation 1: By applying Limited MCS, more practical operating SNR can be obtained. Observation 2: In 29GHz TDL-A, the throughput gain of 256QAM is shown in MCS index 20 and 21, and operating SNR can vary from 25.5dB to 30dB depending on the MCS index number. Observation 3: In 29GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20~23, and operating SNR can vary from 18.5dB to 27dB depending on the MCS index number. Observation 4: In 39GHz TDL-A, no throughput gain of 256QAM is shown in MCS index 20~23. Observation 5: In 39GHz TDL-D, the throughput gain of 256QAM is shown in MCS index 20 ~22, and operating SNR can vary from 21dB to 30dB depending on the MCS index number. Observation 6: If the operating SNR is considered based on TDL-A, SNR of 25.5 dB or more need to be set for operation at 29GHz, however it is not practical. And N/A is shown at 39GHz, so it is also not practical. Observation 7: If the operating SNR is considered based on TDL-D with limited MCS, an operating SNR value of 18.5dB to 27dB at 29GHz may be considered, and an operating SNR value of 21dB to 30dB at 39GHz may be considered. Proposal 1: Consider limited MCS not only in the 39GHz band but also in the 29GHz band. Proposal 2: Consider operating SNR as [18.5~23]dB with limited MCS of 20~22 at 29GHz. Proposal 3: Consider operating SNR as [21~24]dB with limited MCS of 20~21 at 39GHz. |
| R4-2218753 | Sony | Observation 1 For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz. Observation 2 For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required. |
| R4-2218869 | vivo | Observation 1: - For PC1, 20% of UE’s SINR can be larger than 25 dB. - For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB. Observation 2: - For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM. - For the Uma scenario, the performance gain of all PC UEs is tiny. Observation 3: Even though limiting the MCS can lower the operating SNR, the system performance gain will also be restricted which makes the UL 256QAM meaningless. Proposal 1: Conclude the UL 256QAM for PC2/PC3/PC5 is not feasible in FR2-1, and focus on PC1 only in the future discussion. Proposal 2: No need to limit the max MCS for UL 256QAM, it depends on NW whether the higher MCS can be used. |
| R4-2218940 | MediaTek Korea Inc. | Observation 1: The phase noise impact on the FR2 is much higher compared with FR1. Low phase noise implementation is challenging for FR2. The EVM budget differing from Table 1 should be used for FR2-1 application. Proposal 1: RAN4 to take the EVM budget shown in Table 2 for UL 256QAM MPR simulation for FR2-1. |
| R4-2219075 | Beijing Xiaomi Software Tech | Observation 1: From the simulation results figure 2.1-1 to figure 2.1-6, only under AWGN channel with MCS21 for UL 256QAM can get performance gain compared to 64QAM. From simulation results in last meeting, most companies can’t get the performance gain for 48GHz UL 256QAM compared to UL 64QAM under TDL-D and TDL-A channel, only one company can get the performance gain compared to UL 64QAM under TDL-D with MCS 21, only two companies can get the performance gain compared to UL 64QAM under AWGN with MCS 21. Observation 2: UL 256QAM for 48GHz is unfeasible. And proposed: Proposal 1: UL 256QAM isn’t considered for 48GHz in Rel-18. Proposal 2: For 29GHz, - The operating SNR using the average value 23dB with limited MCS 21. - The operating SNR using the average value 29dB with limited MCS 23. Proposal 3: For 39GHz, The operating SNR using the average value 27dB with limited MCS 21. Proposal 4: The minimum EIRP for UL 256 QAM for EVM test should be defined based on 400MHz channel bandwidth as for UL 16QAM and 64QAM. |
| R4-2219144 | Huawei, HiSilicon | In the contribution, we provide proposals for the simulation assumption and preliminary simulation results to study the gain and operating SNR for UL 256QAM. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 EVM evaluation by link level simulation

Sub-topic description: Discuss the EVM requirements for 29GHz, 39GHz and 48GHz based on link level simulation.

In last meeting and this meeting, many companies submitted the link level simulation results based on the simulation assumption agreed in RAN4 #104-e meeting. Below tables summarized simulation results from these two meetings, and the red font represents the simulation results submitted in this meeting.

For 29GHz:

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2215920 R4-22118681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2216251 R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 | Ericsson R4-2220036 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz | 29 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 100MHz | 50MHz | 100MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none |  | none |  | none | none | none/8 |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | 25.5 |  |  |  |  |  |  |
|  |  | MCS21 |  | 30 | 22.7/24 |  |  | 22.7 | 26.45/24.03 |  |
|  |  | MCS22 |  | NA |  |  |  |  |  |  |
|  |  | MCS23 |  | NA | 26.3/28.4 |  |  | 28 | 34.31/29.07 |  |
|  | TDL-D | MCS20 |  | 18.5 |  |  |  |  |  |  |
|  |  | MCS21 | 24-25 | 21 | 20.8/21.8 | 20.1 | 23 | 22.5 | 23.73/22.42 | 32.4/31.2 |
|  |  | MCS22 |  | 23 |  |  |  |  |  |  |
|  |  | MCS23 | 32 | 27 | 25.7/28 | 23 |  | 27.3 | 30.10/27.71 |  |
|  | AWGN | MCS21 |  | 17 | 19.5/19.7 |  | 20 | 19.5 | 21.71/20.8 | 28.7/28.1 |
|  |  | MCS23 |  | 21 | 22.2/23.8 |  | 25 | 23.5 | 27.66/27.47 |  |

For 39GHz:

| Parameter |  |  | Nokia R4-2215577 | LG Electronics R4-2218681 | vivo R4-2216128 | Huawei R4-2216245 R4-2219144 | Sony R4-2218753 | Xiaomi R4-2216350 | ZTE R4-2216426 R4-2218595 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz | 39 GHz |
| CBW |  |  | 50MHz | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| HARQ |  |  | none | none |  | none |  | none | none |
| Phase noise model |  |  | Option a): example1 (UE) + example1(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS20 |  | NA |  |  |  |  |  |
|  |  | MCS21 |  | NA | 29.2/NA |  |  | 26.7 | 32.35/26.09 |
|  |  | MCS22 |  | NA |  |  |  |  | 30.70 |
|  |  | MCS23 |  | NA | NA |  |  | NA | NA |
|  | TDL-D | MCS20 |  | 21 |  |  |  |  |  |
|  |  | MCS21 | 36 | 24 | 25/NA | 21 |  | 25.2 | 27.50/26 |
|  |  | MCS22 |  | 30 |  |  |  |  | 29.61 |
|  |  | MCS23 |  | NA | NA | 25.4 |  | NA | NA |
|  | AWGN | MCS21 |  |  | 21.8/28 |  | 21.5 | 21.5 | 21.64/22 |
|  |  | MCS22 |  |  |  |  |  |  | 27.55 |
|  |  | MCS23 |  |  | NA |  | 36 | NA | 27.64 |

For 48GHz

| Parameter |  |  | LG Electronics R4-2215920 | vivo R4-2216128 | Huawei R4-2216245 | Xiaomi R4-2216350 R4-2219075 | ZTE R4-2216426 R4-2219595 | Nokia R4-2218326 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz | 48 GHz |  |
| CBW |  |  | 100MHz | 50MHz/100MHz | 50MHz | 50MHz | 100MHz | 50MHz |  |
| SCS |  |  | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |  |
| Phase noise model |  |  | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option d): example1 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option b): example2 (UE) + example2(BS) | Option a): example1 (UE) + example1(BS) | Option d): example1 (UE) + example2(BS) |
| Tx EVM=Rx EVM |  |  | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% | 3.5% |
| Target SNR(dB) | TDL-A | MCS21 | NA | NA |  | NA | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |
|  | TDL-D | MCS21 | NA | NA | 22.6 | NA | NA | NA | NA |
|  |  | MCS23 | NA | NA |  | NA | NA | NA | NA |
|  | AWGN | MCS21 | 27.5 | NA |  | 25 | NA |  |  |
|  |  | MCS23 | NA | NA |  | NA | NA |  |  |

Open issues and candidate options before f2f meeting:

Issue 1-1-1: MCS limitation for 29GHz and 39GHz

- Proposals

  - Option 1: Consider limited MCS not only in the 39GHz band but also in the 29GHz band.(LGE)

  - Option 2: No need to limit the max MCS for UL 256QAM, it depends on NW whether the higher MCS can be used.(vivo)

  - Option 3: No need to limit the max MCS for UL 256QAM in the 29GHz band, consider limited MCS only in the 39GHz band (R4-2217729).

      - Proposed method for limiting the MCS for 39 GHz consists of selecting the max MCS which satisfies the following criteria: performance degradation due to phase noise is less than XdB (e.g. X=1.5 dB since 256QAM is assumed) with the agreed PN compensation technique (CPE compensation), i.e. performance degradation is less than XdB between the cases of “No PN” and “PN with CPE compensation”.(Ericsson)

- Recommended WF

  - TBA

Issue 1-1-2: Operating SNR for 29GHz

- Proposals

  - Option 1: Adopt 28 dB at 29GHz as the target operating SNR without MCS limitation.(Nokia)

  - Option 2: Consider operating SNR as [18.5~23]dB with limited MCS of 20~22 at 29GHz. (LGE)

  - Option 3: SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.(Sony)

  - Option 4: The operating SNR using the average value 23dB for MCS 21 and 29dB forMCS 23 for 29GHz based on the submitted results in last meeting .(Xiaomi).

  - Option 5: The operating SNR for 256QAM MCS21 should be equal to 30.4 dB.(Ericsson)

  - Option 6: Consider operating SNR as 28 dB with limited MCS 20~23 for 29GHz averaged based on the submitted results from two meetings.

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 25 | - | 32 |
| LGE | 18.5 | 21 | 23 | 27 |
| vivo | - | 21.8 | - | 28 |
| Huawei | - | 20.1 | - | 23 |
| Sony | - | - | - | - |
| Xiaomi | - | 22.5 | - | 27.3 |
| ZTE | - | 22.42 | - | 27.71 |
| Ericsson | - | 32.4 | - | - |
| Average | 18.5 | 27.27 | 23 | 27.5 |

- Recommended WF

  - Option 6: Consider operating SNR as 28 dB with limited MCS 20~23 for 29GHz averaged based on the submitted results from these two meetings.

Issue 1-1-3: Operating SNR for 39GHz

- Proposals

  - Option 1: Adopt 25 dB at 39GHz as the target operating SNR.9 (Nokia)

  - Option 2: Consider operating SNR as [21~24]dB with limited MCS of 20~21 at 39GHz. (LGE)

  - Option 3: SNR levels of 24 dB for MCS21 and 28dB for MCS23 are needed for 29GHz.(Sony)

  - Option 4: The operating SNR using the average value 27dB for MCS 21 for 29GHz based on the submitted results in last meeting .(Xiaomi)

  - Option 5: Consider operating SNR as 30 dB with limited MCS 20~22 for 29GHz averaged based on the submitted results from two meetings.

|  | Operating SNR (dB) with EVM 3.5% |  |  |  |
| --- | --- | --- | --- | --- |
|  | TDL-D, MCS20 | TDL-D, MCS21 | TDL-D, MCS22 | TDL-D, MCS23 |
| Nokia | - | 36 | - | - |
| LGE | 21 | 24 | 30 | NA |
| vivo | - | 25 | - | NA |
| Huawei | - | 21 | - | 25.4 |
| Sony | - | - | - | - |
| Xiaomi | - | 25.2 | - | NA |
| ZTE | - | 26 | 29.21 | NA |
| Average | 21 | 26.2 | 29.6 | 25.4 |

- Recommended WF

  - Option 5: Consider operating SNR as 30 dB with limited MCS 20~22 for 29GHz averaged based on the submitted results from two meetings.

Issue 1-1-4: EVM requirements for 48GHz

- Proposals

  - Option 1: UL 256QAM is unfeasible for 48GHz. (Nokia, ZTE, Xiaomi)

  - Option 2: Others

- Recommended WF

  - Option 1: UL 256QAM is unfeasible for 48GHz.

Sub-topic 1-2 EVM evaluation by system level simulation

Sub-topic description: Discuss UL 256QAM feasibility for 29GHz and 39GHz based on system level simulation.

.

Below tables summarized system level simulation results submitted by four companies in this meeting.

|  | Nokia (2218327) | ZTE (R4-2218595) | vivo (R4-228869) | Ericsson R4-2220036 |
| --- | --- | --- | --- | --- |
| Urban macro | The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. ~15% of PC1 UE and ~25% of PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). | 5% UE can achieve above 30dB SINR which show that PC1, PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase, for example, ~10% UE can achieve above 28dB SINR even for PC3. | For PC1, 20% of UE’s SINR can be larger than 25 dB. For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB. the performance gain of all PC UEs is tiny | For 29GHz, we can see that around 30% of UEs can achieve the operating SNR of 30 dB for FTP with 30% load and around 35% of UEs can achieve the operating SNR of 30 dB for FTP with 10% load. |
| Indoor | The target SNR at BS side (of 28 dB and 25 dB, respectively, at 29 GHz and 39 GHz) can be achieved by ~70% of PC1 UE and ~60% of PC2/5 UE. Almost none of PC1 UE and PC2/5 UE are transmitting at maximum output power (of 35 dBm and 23 dBm, respectively, for PC1 UE and PC2/5 UE). |  | For PC1, 20% of UE’s SINR can be larger than 25 dB. For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB. only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM. |  |

Open issues and candidate options before f2f meeting:

Issue 1-2-1: Modify the simulation assumptions

- Proposals

  - Option 1: Add Min peak EIRP in the system level simulation assumptions.(ZTE)

| UE Min peak EIRP | n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm n260 PC1:38.0 dBm/PC3:20.6 dBm |
| --- | --- |

  - Option 2: Add power control and traffic model in the system level simulation assumptions.(vivo)

| Power control | PC1: P0 = -60 / PC2&PC5: P0 = -70 / PC3: -76 alpha: Urban macro =0.8 / Indoor = 0.6 |
| --- | --- |
| Traffic model | Full Buffer |

  - Option 3: Add power control in the system level simulation assumptions.(Nokia)

| Power control | Power control parameters set to achieve the target SNR at BS side. |
| --- | --- |

- Recommended WF

  - TBA

Issue 1-2-2: UL 256QAM feasibility for 29GHz and 39GHz

- Proposals

  - Option 1: UL 256 QAM for PC1 UEs of 29 GHz is feasible. (Ericsson, ZTE, Nokia, vivo)

  - Option 2: UL 256QAM for PC2/PC3/PC5 is not feasible in FR2-1, and focus on PC1 only in the future discussion.(vivo)

- Recommended WF

  - TBA

Topic #2: MPR simulation

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218040 | Qualcomm Incorporated | Observation 1: The MPR simulation activity for UEs is gated by agreement on the PTRS correction method employed in the EVM calculator. Proposal 3: PTRS based correction method is agreed prior to MPR simulations |
| R4-2218328 | Nokia, Nokia Shanghai Bell | Proposal 4: Opponents of using the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis should provide concrete alternative proposal to be discussed in RAN4 to progress the discussion in this topic. |
| R4-2218940 | MediaTek Korea Inc. | Observation 1: The phase noise impact on the FR2 is much higher compared with FR1. Low phase noise implementation is challenging for FR2. The EVM budget differing from Table 1 should be used for FR2-1 application. Proposal 1: RAN4 to take the EVM budget shown in Table 2 for UL 256QAM MPR simulation for FR2-1. |
| R4-2219075 | Beijing Xiaomi Software Tech | Proposal 6: Antenna configuration and PA calibration point for MPR evaluation: - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation. - PA calibration point should follow current definition in Spec 38.101-2: - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition. - Calculate MPR as total backoff needed for 256QAM from this calibration point. Proposal 7: Emission requirements for MPR evaluation should follow the current Spec 38.101-2, otherwise specified: - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2) - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2) - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2) - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2) - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting. Proposal 8: EVM requirement for each contributor can use the FR1 assumption as the starting point. Tx EVM contributor; EVM; SNR(dB). PA; 1.85%; 34.7. Transmitter; 1.19%; 38.5. Phase noise; 1.78%; 35. IQ imbalance; 2.06%; 33.7. Total; 3.5%; 29.1. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions..

Sub-topic 2-1 EVM budget

Sub-topic description: Discuss the EVM budget for EVM simulation

Open issues and candidate options before f2f meeting:

Issue 2-1-1: Phase noise assumption on EVM budget

- Proposals

  - Option 1: Using the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis.

  - Option 2: Others (Opponents of option 1 should provide concrete alternative proposal)

- Recommended WF

  - TBA

Issue 2-1-2: EVM budget for MPR simulation

- Proposals

  - Option 1: Take the EVM budget shown as below Table for UL 256QAM MPR simulation for FR2-1.

| EVM Contributor | EVM(%) | SNR(dB) |
| --- | --- | --- |
| Transmitter | 1.32 | 37.59 |
| Phase Noise | 2.10 | 33.56 |
| IQ Imbalance | 1.45 | 36.77 |
| PA Non-linearity | 2.00 | 33.98 |
| Total | 3.50 | 29.12 |

  - Option 2: Use the FR1 assumption as the starting point for UL 256QAM MPR simulation for FR2-1.

| Tx EVM contributor | EVM | SNR(dB) |
| --- | --- | --- |
| PA | 1.85% | 34.7 |
| Transmitter | 1.19% | 38.5 |
| Phase noise | 1.78% | 35 |
| IQ imbalance | 2.06% | 33.7 |
| Total | 3.5% | 29.1 |

- Recommended WF

  - TBA

Sub-topic 2-2 General MPR simulation assumptions

Sub-topic description: Discuss the MPR simulation assumptions.

Open issues and candidate options before f2f meeting:

Issue 2-2-1: Antenna configuration and PA calibration point for MPR simulation

- Proposals

  - Option 1:

      - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation.

      - PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

  - Option 2: Others

- Recommended WF

  - TBA

Issue 2-2-2: Emission requirements for MPR simulation

- Proposals

  - Option 1: Follow the current Spec 38.101-2, otherwise specified

      - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2)

      - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2)

      - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2)

      - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2)

      - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting.

  - Option 2: Others

Issue 2-2-3: Priority of PTRS correction method and MPR simulations

- Proposals

  - Option 1: PTRS based correction method is agreed prior to MPR simulations.

  - Option 2: Others.

- Recommended WF

  - TBA

- Recommended WF

  - TBA

Topic #3: Minimum EIRP

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2219075 | Beijing Xiaomi Software Tech | Observation 3: PC2 UE needs 0.4dBm and 2.2 dBm minimum EIRP for EVM test for 256QAM with 400MHz channel bandwidth for 29GHz and 39GHz separately. And proposed: Proposal 4: The minimum EIRP for UL 256 QAM for EVM test should be defined based on 400MHz channel bandwidth as for UL 16QAM and 64QAM. Proposal 5: The minimum EIRP for UL 256 QAM for EVM test could be relaxed by 15.5 dB based on the minimum output power for different PCs: Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 Minimum EIRP requirements

Sub-topic description: Discuss the minimum EIRP requirements for EVM test

Open issues and candidate options before f2f meeting:

Issue 3-1-1: How to define the minimum EIRP requirements for EVM test

- Proposals

  - Option 1: Based on 400MHz channel bandwidth as for UL 16QAM and 64QAM.(Xiaomi)

  - Option 2: Introduce minimum UE EIRP scaling for 256QAM. (Apple’s proposal in RAN4 #104-e)

- Recommended WF

  - TBA

Issue 3-1-2: The minimum EIRP requirements for EVM test

- Proposals

  - Option 1: Relaxed by 15.5 dB based on the minimum output power for different PCs

| Parameter | Unit | Level for PC1 | Level for PC2 | Level for PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 256 QAM | dBm | 19.5 | 2.5 | 9.5 |

  - Option 2: Apple’s proposal in RAN4 #104-e (R4-2212370)

      - UE EIRP for PC1: 19.5dBm

      - UE EIRP for PC2, PC3, PC4: 2.5dBm

      - UE EIRP for PC5: 9.5dBm

|  |  | Level for PC2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| UE EIRP for UL 256 QAM | dBm | 2.5 | 2.5 | 5.5 | 8.5 |
| Operating conditions | Normal Conditions |  |  |  |  |
| NOTE 1: PTRS is configured for 256 QAM |  |  |  |  |  |

  - Option 3: Others

- Recommended WF

  - TBA

Topic #4: EVM test

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218040 | Qualcomm Incorporated | Observation 1: The MPR simulation activity for UEs is gated by agreement on the PTRS correction method employed in the EVM calculator. Proposal 1: For CP-OFDM, PTRS correction is implemented by de-rotation of each sub carrier in an OFDM symbol. The de-rotation angle is estimated as the frequency domain average of the phase rotation of all the PTRS tones in the allocation. Proposal 2: For DFT-s-OFDM: - PTRS correction is implemented by de-rotation of each time-domain symbol by the estimated instantaneous phase deviation. - The instantaneous phase deviation impacting a data symbol due to DUT phase noise is estimated by linearly interpolating between the phase deviations determined for the nearest neighbouring PTRS groups. The phase deviation for each PTRS group is determined as the time domain arithmetic mean phase deviation of all PTRS symbols in the group. Proposal 3: PTRS based correction method is agreed prior to MPR simulations Observation 2: The EVM benefit due to PTRS-based corrections depends on phase noise profile of the UE and modulation type (DFT-s or CP-OFDM). Observation 3: No single fixed PTRS configuration can be assumed to be beneficial for all UEs for all RB conditions and all MCSs. Proposal 4: For UL 256QAM in FR2, the PTRS configuration shall be aligned with the UE’s recommended PTRS configuration (IE PTRS-DensityRecommendationSetUL). Proposal 5: The EVM calculation signal flow including PTRS processing shall be included in the annex as normative content. |
| R4-2218328 | Nokia, Nokia Shanghai Bell | Proposal 3: To adopt option 2 to use a fixed PTRS configuration for all devices for the EVM test. |
| R4-2220036 | Ericsson Limited | Proposal 4: If only CPE compensation method is used (with no ICI compensation) and having in mind the test implementation, it is reasonable to stick with a Rel-15 PTRS configuration of K=2, L=1 only. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1 PTRS

Sub-topic description: Discuss the PTRS requirements for UL 256QAM

Open issues and candidate options before f2f meeting:

Issue 4-1-1: PTRS configuration

- Proposals

  - Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration. (IE PTRS-DensityRecommendationSetUL)

  - Option 2: Using a fixed PTRS configuration for all devices for the EVM test.

      - It is reasonable to stick with a Rel-15 PTRS configuration of K=2, L=1 only, If only CPE compensation method is used (with no ICI compensation) and having in mind the test implementation.

  - Option 3: Others.

- Recommended WF

  - TBA

Issue 4-1-2: PTRS correction methods

- Proposals

  - Option 1:

      - For CP-OFDM

  - PTRS correction is implemented by de-rotation of each sub carrier in an OFDM symbol. The de-rotation angle is estimated as the frequency domain average of the phase rotation of all the PTRS tones in the allocation.

      - For DFT-s-OFDM:

  - PTRS correction is implemented by de-rotation of each time-domain symbol by the estimated instantaneous phase deviation.

  - The instantaneous phase deviation impacting a data symbol due to DUT phase noise is estimated by linearly interpolating between the phase deviations determined for the nearest neighbouring PTRS groups. The phase deviation for each PTRS group is determined as the time domain arithmetic mean phase deviation of all PTRS symbols in the group.

- Recommended WF

  - TBA

Sub-topic 4-2 EVM calculation flow with PTRS

Sub-topic description: Discuss the EVM calculation flow with PTRS.

Open issues and candidate options before f2f meeting:

Issue 4-2: EVM calculation flow with PTRS

- Proposals

  - Option 1: The EVM calculation signal flow including PTRS processing shall be included in the annex as normative content

- Recommended WF

  - TBA

Topic #5: TP

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2219122 | Xiaomi | The TP captures the link level simulation results from different companies and system level simulation assumptions into TR 38.891. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1 TP to capture the agreement in last meeting

Sub-topic description:

The TP is to capture the agreement and simulation results in last meeting

Open issues and candidate options before f2f meeting:

Issue 5-1: Approved TP in R4-2219122

- Proposals

  - Option 1: Yes

  - Option 2: Modification is needed

- Recommended WF

  - TBA