3GPP TSG-RAN WG4 Meeting # 105												R4-2220147

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			6.3.6.1, 6.3.6.2

Source:	Qualcomm

Title:	Topic summary for [105][321] NR_exto71GHz_Demod_Part2

Document for:	Information

# Introduction

This email discussion summary collects the contributions for the following agenda items:

- 6.3	Extending current NR operation to 71GHz	 				[NR_ext_to_71GHz]

- 6.3.6	 Demodulation and CSI requirements 						[NR_ext_to_71GHz-Perf]

  - 6.3.6.1	General (incl. Channel models) 			[NR_ext_to_71GHz-Perf]

  - 6.3.6.2	UE Demodulation and CSI requirements 	[NR_ext_to_71GHz-Perf]

    - 6.3.6.2.1	PDSCH requirements 			[NR_ext_to_71GHz-Perf]

    - 6.3.6.2.2	PDCCH/PBCH requirements	[NR_ext_to_71GHz-Perf]

    - 6.3.6.2.3	CSI reporting requirements		[NR_ext_to_71GHz-Perf]

The target for the discussion is the following:

1st round:

- Conclude on the open issues;

- Collect and review the submitted impairment simulation results;

2nd round:

- Agree on tentative SNR requirements;

- Endorse the draft CRs for BigCR approval;

# Topic #1: General and PDSCH Demodulation Requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218179 | Apple | General aspects Proposal #1: Use the following table to determine testable SNR based on agreed PDSCH allocation: CBW (MHz); Num RBs; Test method. IFF. Single band UE; 100; 66; [9.8]. 32; [13.2]. 400; 66; [2.6]. 32; [6.6]. 20; [8.9]. PDSCH demod requirements Observation #1: The testable SNR is lower than the SNR after considering impairment margin for - 120KHz SCS MCS 17 with 66 PRBs and 480KHz SCS MCS 13 with 32 PRBs. Proposal #2: Define requirements with 120KHz with MCS17 for 32 PRB PDSCH allocation considering testable SNR Proposal #3: Define requirements with 480khz SCS with MCS 13 with 20 PRB allocation considering testable SNR Proposal #3: For PDSCH requirements with reduced PRB allocation, PDCCH CORESET BW is also reduced. Observation #2: With partial CORESET BW allocated to PDCCH in line with PDSCH, the same AL and/or CORESET duration as FR2-1 cannot be used for PDCCH for test cases with partial PDSCH allocation. Proposal #4: For PDSCH allocation of 32 PRB, PDCCH CORESET BW is 30 PRB duration is 1 symbol with AL=4. Observation #3: PDCCH performance with no decoding errors cannot be guaranteed with AL=2 with MCS 13 test case. Proposal #5: For PDSCH allocation of 20 PRB, PDCCH CORESET BW is 18 PRB and duration is 2 symbol with AL=4. PDSCH demod requirements for CA Observation #4: There is no benefit of introducing CA requirements with 120KHz/400MHz with same PRB allocation as that for 100MHz. Observation #5: There will be additional simulation effort needed if FR2-2 carrier has different configuration for CA and single carrier test cases. Proposal #6: Use the same BW configuration for FR2-2 carrier for CA requirements as single carrier requirements. Proposal #7: Introduce CA requirements for FR2-2 carrier where corresponding single carrier requirements are defined. Proposal #8: Use the following bandwidth and test setup for FR1+FR2-2 CA requirements - 100MHz/120KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is 66RBs - 400MHz/480KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). Proposal #9: Test cases and applicability rule for FR1+FR2-2 CA requirements: - Reuse all cases of single CC for CA and define applicability rules as follows: - If UE supports standalone operation, UE shall only be tested with single CC. - If UE don’t support standalone operation, UE shall only be tested with CA Proposal #10: Do not define new requirements for CA. Reuse single carrier requirements for CA. |
| R4-2218307 | Ericsson | Proposal 1: RAN4 defines the PDSCH demodulation requirements using the extended maximum testable SNR: CBW (MHz); Num RBs; Test method. IFF. Single band UE; 100; 66; [9.8]. [32]; [13.2]. 400; 66; [2.6]. [32]; [6.6]. [20]; [8.9]. Proposal 2: Define max MCS for PDSCH demodulation requirements for UE at 70% of the peak throughput as - 120 kHz/100 MHz: MCS 17 with full RB allocation (66 RBs) - 480 kHz/400 MHz: MCS 13 with partial RB allocation (32RBs) Proposal 3: Consider a margin of 3 dB for RF impairments when defining the PDSCH demodulation requirements for UE. Observation 1: PDSCH demodulation parameters at 70% of the peak throughput has been agreed as Assumptions; Value. Carrier Frequency [GHz]; 70 GHz. Subcarrier Spacing [KHz]; 120 KHz, 480 KHz. Waveform; CP-OFDM. CP Type; Normal CP. Channel Model; TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread). Antenna configuration; 2x2 ULA Low. Velocity; 3 km/h, 10 km/h. PA Model; None. gNB PN Model; None. UE PN Model; Up to UE implementation. Pre-loaded Tx EVM; 6%. Additive Rx EVM; 0%. I-Q Imbalance; None. Frequency Offset; 0 ppm. Channel Estimation; Realistic channel estimation. Transmission Rank; Rank 1. DMRS Configuration; 2 DMRS symbols at (2,11) symbol index. PTRS Configuration; For CP-OFDM: (K = 2, L = 1). MCS/TBS; From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM).. NOH; 6. Number of HARQ Processes; 8 for 120kHz and 16 for 480kHz SCS. Observation 2: Based on our simulation results in [3], the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation and considering an additional margin of 3 dB for RF impairments are - 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200 - 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200 Proposal 4: Referring to Observation 2 and the Table in Observation 1, consider PDSCH test cases at 70% of the peak throughput for 120 kHz and 480 kHz SCS as follow FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 70% Peak Throughput Test case; CBW / SCS (PRB); MCS and rank; TDD UL/DL pattern; Propagation condition; Antenna configuration; PN Compensation; SNR (dB) @ 70% of peak Throughput; Reference from TS38.101-4 7.2.2.2.1-6. 1-1; 100MHz / 120kHz (66); QPSK, 0.30 Rank 1; FR2.120-1; TDLA30-650; 2x2 ULA Low; CPE; [-2.9 + 3.0]; R.PDSCH.5-1.1 TDD. 1-2; 100MHz / 120kHz (66); 16QAM, 0.48 Rank 1; FR2.120-1; TDLA30-200; 2x2 ULA Low; CPE; [4.8 + 3.0]; R.PDSCH.5-2.1 TDD. 1-3; 100MHz / 120kHz (66); 64QAM, 0.43 Rank 1; FR2.120-1; TDLD30-200; 2x2 ULA Low; CPE; [4.7 + 3.0]; R.PDSCH.5-3.2 TDD. FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 70% Peak Throughput Test case; CBW / SCS (PRB); MCS and rank; TDD UL/DL pattern; Propagation condition; Antenna configuration; PN Compen- sation; SNR (dB) @ 70% of peak Throughput; Reference from TS38.101-4 7.2.2.2.1-6. 2-1; 400MHz / 480kHz (66); QPSK, 0.30 Rank 1; FR2.480-1; TDLA10-200; 2x2 ULA Low; CPE; [-2.9 + 3.0]; R.PDSCH.7-1.1 TDD. 2-2; 400MHz / 480kHz (32); 16QAM 0.48 Rank 1; FR2.480-1; TDLD10-200; 2x2 ULA Low; CPE; [1.0 + 3.0]; R.PDSCH.7-2.1 TDD. Observation 3: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup. Assumptions; Value. Carrier Frequency [GHz]; 70 GHz. Subcarrier Spacing [KHz]; 120 KHz. Waveform; CP-OFDM. CP Type; Normal CP. Channel Model; TDLA (30 ns delay spread). Antenna configuration; 2x2 ULA Low. Velocity; 3 km/h, 10 km/h. PA Model; None. gNB PN Model; None. UE PN Model; Up to UE implementation. Pre-loaded Tx EVM; 6%. Additive Rx EVM; 0%. I-Q Imbalance; None. Frequency Offset; 0 ppm. Channel Estimation; Realistic channel estimation. Transmission Rank; Rank 1. DMRS Configuration; 2 DMRS symbols at (2,11) symbol index. PTRS Configuration; For CP-OFDM: (K = 2, L = 1). MCS/TBS; From MCS Table 1 (TS38.214): MCS 13. NOH; 6. Number of HARQ Processes; 16. Proposal 5: Referring to the Table in Observation 3, consider PDSCH test cases at 30% of the peak throughput using the following test setup. FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 30% Peak Throughput Test number; CBW / SCS (PRB); MCS and rank; TDD UL/DL pattern; Propagation condition; Antenna configuration; PN Compen- sation; SNR (dB) @ 30% of peak Throughput; Reference from TS38.101-4 7.2.2.2.1-6. 3-1; 100MHz / 120kHz (66); 16QAM, 0.48 Rank 1; FR2.120-1; TDLA30-650; 2x2 ULA Low; CPE; [-1.3 + 3.0]; R.PDSCH.5-2.1 TDD. Observation 4: Consider CBW1 = 100 MHz and nx100 MHz for 120 kHz SCS, and CBW2 = 400 MHz and nx400 MHz for 480 kHz SCS. Proposal 6: Define PDSCH requirements for FR1+FR2-2 CA, with FR1 TDD 30 kHz SCS, for the following cases - 100MHz/120KHz - PDSCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). - CQI requirements: FR1(40MHz, single CC) + FR2-2(100MHz, single CC). The RB allocation for FR2-2 CC is 66RBs - 400MHz/480KHz - PDSCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). - CQI requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is 66RBs Proposal 7: Use same PDSCH requirements of single CC for CA, where - In case of SA, only single CC test cases will be considered. - In case of NSA, UE will be tested considering CA configurations. |
| R4-2219056 | Nokia, Nokia Shangai Bell | General and PDSCH Demodulation Requirements Maximum testable SNR - Requirements for 120kHz/100MHz will not include MCS20 and 400MHz CBW. - Define the Extended maximum Testable SNR for reduced allocation as: CBW (MHz)/SCS(kHz); Num RBs; Test method. IFF. Single band UE; 100/120; 66; [9.8]. 32; [13.2]. 400/480; 66; [2.6]. 32; [6.6]. 20; [8.9]. PDSCH Requirements RB allocation for 120kHz/100MHz for MCS 17 for 70% requirements Observation 2: Using full allocation for MCS17 will have a SNR below the feasible SNR of [9.8]dB - Use full RB allocation (66 RBs) for 120kHz/100Mhz SCS/CBW with MCS17 for 70% requirements. RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements Observation 3: Based on our simulation results, both options seem fine for us with a preference for 32 RBs to cover most bandwidth. - Define requirements for 480kHz/400MHz with MCS 13 using 32RBs. PDSCH Requirements for CA Bandwidth choice and test setup for FR1+FR2-2 CA requirements Observation 4: We see, that defining requirements for 100MHz/120kHz and 400MHz/480kHz for FR1+FR2-2 CA will be aligned with the already agreed requirements to be defined for FR2-2 single CC. - RAN4 to define requirements for FR1+FR2-2 CA with 100MHz/120kHz CBW/SCS and 400MHz/480kHz CBW/SCS. The configuration for FR1 is to be further discussed. Applicability rule for FR1+FR2-2 CA requirements Observation 5: Single CC requirements will be close to CA requirements, hence can be copied to CA (FR1+FR2-2) securing the same coverage for CA as for single CC. In addition, this will also reduce the simulation workload. - To secure same test coverage for single CC and CA UEs and reduce simulation workload, all test cases from single CC shall be copied to CA. Observation 6: Single CC and CA are two different features; hence no applicability rules should be defined between single CC and CA requirements. - Do not define applicability rules between single CC and CA. Configurations for FR1 CC for FR1+FR2-2 CA requirements Observation 7: As CA is related to high throughput, it will in our view be valid to select a configuration for FR1 which has the highest CBW to ensure enough resources for control. - RAN4 use FR1 configuration as 50MHz/15kHz CBW/SCS as baseline and discuss further the detailed configuration. Test cases for FR1+FR2-2 CA requirements - Reuse all test cases of single CC to define CA testcases. How to define FR1+FR2-2 CA requirements Observation 8: Performance loss should be negatable and compared to 2 single CC test, as such feasible. To save time directly use CC requirements. - As we see no expected performance loss and to save time, apply the requirements of single CC to CA. PDSCH Simulation Assumptions Whether to schedule PDSCH in slots that contain TRS symbols during PDSCH testing Observation 9: Current agreement is targeting simulation alignment only. Observation 10: Feedback from TE vendors, if any, might change companies view on current agreement. Whether the TE transmit TRS in the full CBW during PDSCH testing Observation 11: As it was agreed in RAN4#104-bis-e to not multiplex PDSCH with TRS symbols, we see it acceptable to tentatively agree to transmit TRS in unallocated PRB for simulation alignment. - If PDSCH is not scheduled in slots that contain TRS symbols, transmit RTS in unallocated PRBs. |
| R4-2219502 | Huawei | Proposal 1: Use table specified in option 2 as maximum DL testable SNR derivation. I.e. Use following table: CBW (MHz)/SCS(kHz); Num RBs; Test method. IFF. Single band UE; 100/120; 66; 9.8. 32; 13.2. 400/480; 66; 2.6. 32; 6.6. 20; 8.9. 16; 10. Observation 1: Target SNR is larger than maximum testable SNR in following cases: - 120kHz/100MHz, MCS17, TDLD-300 - 480 kHz/400MHz, MCS13. TDLD10-200 Proposal 2: Define FR2-2 PDSCH requirements for cases listed in Table 2-2. Observation 2: For case with 120 kHz/100MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 32RBs, it can be approximately equivalent to 41 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 12.1dB, which leaves enough margin between impairment SNR and maximum testable SNR. Observation 3: For case with 480kHz/400MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 20RBs, it can be approximately equivalent to 32 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 6.8 dB, which leads to the problem that impairment SNR is very closed to maximum testable SNR. Proposal 3: For case with 120 kHz/100MHz, MCS17, TDLD30-200 (32RBs allocation), TRS is full bandwidth transmitted in PDSCH scheduled slot. For case with 480kHz/400MHz, MCS13, TDLD10-200(20RBs allocation), FFS for following candidate options: - Option 1: Reduce the PDSCH RB allocation to 16RBs - Option 2: Not transmit PDSCH in TRS slot - Option 3: Schedule same RB size for TRS and PDSCH. Observation 4: The minimum bandwidth of FR2-2 CC is 400MHz for CA configuration “FR1(Single CC)+FR2-2(Single CC)” . Option 2 for 120kHz SCS conflict the RF agreements. Observation 5: CA_n48A-n263A is the simplest CA configuration among all FR1+FR2-2 CA configurations. Meanwhile, 40MHz+400MHz is the most typical CA configuration regarding to all bandwidth combination of CA_n48A-n263A which will be supported by every UE. Proposal 5: RAN 4 to choice following bandwidth configuration to define FR1+FR2-2 CA requirements: - FR1 single CC with 40MHz+FR2-2 single CCs with 400MHz Proposal 6: Considering following scheduling and feedback pattern for FR1+FR2-1 CA requirements: - For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1. - For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1. Observation 6: There is negligible difference of performance between CA and single CC. Proposal 7: Apply the requirements of single CC to CA. Proposed 8: Use parameters listed in Table 2-4 for Pcell CC for CA test. |
| R4-2219838 | Qualcomm | Proposal 1: Use the Maximum DL testable SNR in Table 1 in this contribution as reference for the definition of the requirements; Observation 1: Maximum testable SNRs are computed independently from SCS; Observation 2: PDSCH Requirement tables will list combinations of CBW, SCS and RB allocation size; Proposal 2: Do not include SCS details in the maximum DL testable SNR at this stage of the WI, because values are applicable to all SCS values that can be configured with the respective CBW. The applicability of reduced RB sizes to different SCSs should be included in the PDSCH requirements table. Proposal 3: For 120kHz/100MHz with MCS 17, use 32RB allocation; Proposal 3: For 480kHz/400MHz with MCS 13, use 20RB allocation; Proposal 3: For PDSCH Requirements with reduced RB allocation, define a test setup with TRS not multiplexed with PDSCH and TRS transmission on the full CBW (including unallocated RBs); Observation 3: Regarding FR1 + FR2-2 CA requirements, RAN4 will not specify 400MHz requirements for 120kHz SCS in this release. Observation 4: Regarding FR1 + FR2-2 CA requirements, there are no RF CA combinations specified for FR1 + FR2-2 CA with 100MHz CCBW for FR2-2. Proposal 3: RAN4 to not introduce dedicated FR1+FR2-2 CA demodulation requirements in Release 17, further discuss test setup to ensure testability of UEs that do not support standalone FR2-2 operations; |
| R4-2218308 | Ericsson | In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed. Observation 1: Referring to Table 1.1, and considering SCS/CBW = 120 KHz/100 MHz with full PRB allocation for MCS (4, 13 and 17) and CPE for PN compensation, we note that MCS 4, MCS 13 and MCS 17 are testable using full RB allocation under the agreed propagation environments and considering a margin of 3 dB for RF impairments. Observation 2: Referring to Table 2.1, and considering SCS/CBW = 480 KHz/400 MHz with full and partial PRB allocation for MCS 4 and 13, respectively, with CPE for PN compensation, we note that • MCS 4 is testable using full RB allocation under TDLA10-200 and considering a margin of 3 dB for RF impairments. • MCS 13 is testable using partial RB allocation of 32 under TDLD10-200 and considering a margin of 3 dB for RF impairments. Observation 3: As a general conclusion, the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation are • 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200 • 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200 Observation 4: Referring to Table 3.1, when considering performance metric at 30% of the peak throughput, for 120 KHz/100 MHz and 480 KHz/400 MHz using MCS 13, it has been noted that • 120 kHz/100 MHz: MCS 13 is testable under TDLA30-650 at 30% of peak throughput using full RB allocation and considering a margin of 3 dB for RF impairments. |
| R4-2219057 | Nokia | In this document we have provided our simulation results for PDSCH based on the agreed simulation assumptions and test cases |
| R4-2219503 | Huawei, HiSilicon | In this paper, we provide our simulation results for FR2-2 PDSCH. |
| R4-2219839 | Qualcomm | Alignment and Impairment SNR results for FR2-2 PDSCH demodulation requirements are included in this contribution. |

## Open issues summary

### Sub-topic 1-1: General

Issue 1-1-1: Extended maximum Testable SNR for reduced allocation

- Proposals

  - Option 1: (Apple, Ericsson)

|  | CBW (MHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 | [9.8] |
|  |  | [32] | [13.2] |
|  | 400 | 66 | [2.6] |
|  |  | [32] | [6.6] |
|  |  | [20] | [8.9] |

  - Option 2: (Nokia)

|  | CBW (MHz)/SCS(kHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100/120 | 66 | [9.8] |
|  |  | 32 | [13.2] |
|  | 400/480 | 66 | [2.6] |
|  |  | 32 | [6.6] |
|  |  | 20 | [8.9] |

  - Option 3 (Huawei):

|  | CBW (MHz)/SCS(kHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100/120 | 66 | 9.8 |
|  |  | 32 | 13.2 |
|  | 400/480 | 66 | 2.6 |
|  |  | 32 | 6.6 |
|  |  | 20 | 8.9 |
|  |  | 16 | 10 |

  - Option 4: Use table below and do not include SCS details (Qualcomm)

Table 1: Extended Maximum DL testable SNR preliminary extension for band n263

|  | CBW (MHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 | 9.8 |
|  |  | 32 | [13.2] |
|  | 400 | 66 | 2.6 |
|  |  | 20 | [8.9] |

- Recommended WF

  - TBA

### Sub-topic 1-2: PDSCH Requirements

Issue 1-2-1: RB allocation for 120kHz/100MHz for MCS 17 for 70% requirements

- Proposals

  - Option 1: 32RBs (Apple, Huawei, Qualcomm);

  - Option 2: 66RBs (Ericsson, Nokia);

- Recommended WF

  - TBA

Issue 1-2-2: RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements

- Proposals

  - Option 1: 20RBs (Apple, Huawei, Qualcomm);

  - Option 2: 32RBs (Ericsson, Nokia);

- Recommended WF

  - TBA

Issue 1-2-3: Whether to reduce PDCCH CORESET BW for PDSCH Requirements with reduced PRB Allocation

- Proposals

  - Option 1: Reduce PDCCH CORESET BW if larger than PDSCH RB allocation (Apple)

  - Option 2: Do not reduce PDCCH CORESET BW;

- Recommended WF

  - TBA

Issue 1-2-4: PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=32RBs;

- Proposals

  - Option 1: CORESET BW=30PRB, Duration = 1Sym, AL=4 (Apple);

  - Option 2: TBA;

- Recommended WF

  - TBA

Issue 1-2-5: PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs;

- Proposals

  - Option 1: CORESET BW=18PRB, Duration = 2Sym, AL=4 (Apple);

  - Option 2: TBA;

- Recommended WF

  - TBA

Issue 1-2-6: Margin for RF Impairments for the PDSCH Demodulation Requirements

- Proposals

  - Option 1: 3dB (Ericsson)

  - Option 2: Up to UE Implementation;

- Recommended WF

  - TBA

Issue 1-2-7: Whether the TE transmit TRS in the full CBW during PDSCH testing with Reduced RB allocation

- Proposals

  - Option 1: Do not multiplex PDSCH with TRS and transmit full CBW TRS (including unallocated PRBs) (Nokia, Qualcomm);

  - Option 2: Multiplex Full Band TRS + 32RB PDSCH for 120kHz, while for 480kHz: (Huawei);

    - Sub-Option 2a: Schedule Full BW TRS if PDSCH is not multiplexed with TRS;

    - Sub-Option 2b: Schedule Full BW TRS + 16RBs PDSCH;

    - Sub-Option 2c: Schedule TRS + PDSCH with same RB size;

- Recommended WF

  - TBA

### Sub-topic 1-3: PDSCH Requirements with CA

Issue 1-3-1: Scope of CA Requirements for FR1+FR2-2

- Proposals

  - Option 1: Reuse single carrier requirements (Apple, Ericsson, Huawei, Nokia);

    - Option 1b: Do not introduce CA requirements for cases not covered by Single carrier requirements (Apple);

  - Option 2: Reuse single carrier requirements and apply all single CC test cases to CA; (Nokia);

  - Option 3: Do not introduce CA requirements for FR1 + FR2-2. Further discuss test setup for UEs that do not support standalone FR2-2 operations (Qualcomm)

- Recommended WF

  - TBA

Issue 1-3-2: BW Configuration for FR2-2 Carrier for CA Requirements

- Proposals

  - Option 1: Reuse Single Carrier BW configuration (Apple);

  - Option 2: TBA

- Recommended WF

  - TBA

Issue 1-3-3: Configuration for FR1 Carrier for CA Requirements

- Proposals

  - Option 1: 50MHz/15kHz as baseline, discuss details (Nokia);

  - Option 2: Table below (Huawei);

- Table 2-4: Proposed test parameters for Pcell CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | 7D1S2U S=6D:4S:4U |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | See FR1.30-1 as defined in Annex A.1.2 |

- Recommended WF

  - TBA

Issue 1-3-4: Applicability rule for FR1+FR2-2 CA requirements

- Proposals

  - Option 1 (Previous WF, Apple, Ericsson)

    - Copy all cases of single CC to CA and define applicability rules as follows:

    - If UE support standalone operation, UE can only be tested cases with single CC.

    - If UE don’t support standalone operation, UE can only be tested cases with CA

  - Option 2: Do not define an applicability rule (Nokia);

- Recommended WF

  - TBA

Issue 1-3-5: Scheduling and feedback pattern for FR1+FR2-2 CA Requirements

- Proposals

  - Option 1 (Huawei): Considering following scheduling and feedback pattern for FR1+FR2-1 CA requirements:

- For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

- For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

- Recommended WF

  - TBA

Issue 1-3-6: Bandwidth choice and test setup for FR1+FR2-2 CA requirements for PDSCH

- Proposals

  - Option 1: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration (Huawei);

  - Option 2: FR1(40MHz, single CC) + FR2-2 (single CC, 100MHz for 120kHz SCS and 400MHz for 480kHz SCS). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration; (Apple, Ericsson);

- Recommended WF

  - TBA

Issue 1-3-7: Bandwidth choice and test setup for FR1+FR2-2 CA requirements for PDCCH

- Proposals

  - Option 1: FR1(40MHz, single CC) + FR2-2 (400MHz, single CC) (Huawei);

  - Option 2: FR1(40MHz, single CC) + FR2-2 (single CC, 100MHz for 120kHz SCS and 400MHz for 480kHz SCS); (Apple, Ericsson)

- Recommended WF

  - TBA

Issue 1-3-8: Bandwidth choice and test setup for FR1+FR2-2 CA requirements for CQI

- Proposals

  - Option 1: FR1(40MHz, single CC) + FR2-2(400MHz, single CC). The RB allocation for FR2-2 CC is 66RBs (Huawei);

  - Option 2: FR1(40MHz, single CC) + FR2-2 (single CC, 100MHz for 120kHz SCS and 400MHz for 480kHz SCS). The RB allocation for FR2-2 CC is 66RBs (Ericsson);

  - Option 3: FR1(40MHz, single CC) + FR2-2 (single CC, 100MHz for 120kHz SCS). The RB allocation for FR2-2 CC is 66RBs; No CA CQI reporting for 480KHz SCS. (Apple);

- Recommended WF

  - TBA

# Topic #2: PDCCH and PBCH Demodulation Requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218181 | Apple | Proposal #1 Introduce PBCH demod requirements with 120KHz SCS with TDLA30-200 propagation channel. |
| R4-2218309 | Ericsson | Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup. Test num; CBW (MHz) / SCS (kHz); Interleaver size; REG bundle size; CORE SET RB; CORE SET duration; Aggrega-tion level; Propagation condition; Antenna config; DCI format; Info Bit; SNRBB (dB) @ 1% Pm-dsg. 1; 100/ 120; 3; 2; 60; 1; 2; TDLA30-200; 1x2 ULA Low; 1_0; 40; [3.5 + 3.0]. 2; 100/ 120; 2; 6; 60; 1; 4; TDLA30-650; 1x2 ULA Low; 1_1; 56; [1.14 + 3.0]. 3; 100/ 120; 3; 2; 60; 1; 8; TDLA30-200; 2x2 ULA Low; 1_1; 56; [-2.3 + 3.0]. 4; 100/ 120; 3; 2; 60; 2; 16; TDLA30-650; 2x2 ULA Low; 1_0; 40; [-6.2 + 3.0]. 5; 400/ 480; 2; 6; 60; 1; 4; TDLD10-200; 1x2 ULA Low; 1_1; 56; [-1.5 + 3.0]. 6; 400/ 480; 2; 6; 60; 1; 8; TDLA10-200; 1x2 ULA Low; 1_1; 56; [-1.49 + 3.0]. 7; 400/ 480; 3; 2; 60; 2; 16; TDLA10-200; 2x2 ULA Low; 1_0; 40; [-6.35 + 3.0]. Proposal 2: As per our simulation results, TDLA10-200 is not applicable for 400 MHz/480 kHz with AL4. We can replace it by TDLD10-200, as shown in the Table below SCS 480 kHz/CBW 400 MHz Test case; Interleaver Size; REG bundle size; CORE SET RB; CORE SET duration; Aggre-gation level; Antenna config; DCI format; Infor Bit; Propagation condition; SNR (dB) @ 1% BLER. Test 1-5; 2; 6; 60; 1; 4; 1x2 ULA Low; 1_1; 56; TDLA10-200; 1.27. TDLD10-200; -1.5. Proposal 3: Define PBCH demodulation requirements with the following test setup Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; TDLA30-650; [-6.95 + 3.0]; Unknown. 1-2; 480kHz/400MHz; TDLA10-200; [-6.36 + 3.0]; Unknown. |
| R4-2219058 | Nokia | PBCH Requirements Table Observation 1: Based on our simulation results, we see it feasible to select propagation condition of TDLA30-650 to align with other requirement agreements on propagation conditions for FR2-2 with 120kHz/100MHz SCS/CBW. Proposal 1: Use TDLA30-650 for 120kHz/100MHz SCS/CBW requirements. |
| R4-2218310 | Ericsson | In this paper, we carried out simulations for PDCCH and PBCH in FR2-2. Here we propose some observations that need further discussions: Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [9.8] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters. Observation 2: For SCS 480 kHZ/CBW 400 MHz and referring to the max DL testable SNR at [2.6] dB, and considering an additional margin of 3 dB for RF impairments, we note that the proposed SNR levels are adequate to the defined parameters, except for AL4 under TDLA10-200. Observation 3: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions). Observation 4: The reference SNRs for 120 KHz and 480 KHz SCS, where the SS/PBCH index is unknown, have been provided. These values confirm their conformance with the max DL testable SNR levels set at [9.8] dB and [2.6] dB, respectively. |
| R4-2219059 | Nokia | In this document we have provided our simulation results for PDCCH and PBCH based on the agreed simulation assumptions and test cases |
| R4-2219504 | Huawei, HiSilicon | In this paper, we updated the simulation results for PDCCH. |
| R4-2219840 | Qualcomm | Alignment and Impairment SNR results for FR2-2 PDCCH and PBCH demodulation requirements are included in this contribution. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

### Sub-topic 2-1: PDCCH Demod Requirements

Issue 2-1-1: Channel Model for FR2-2 PDCCH Requirements Test #5 (480kHz SCS, AL4);

- Proposals

  - Option 1: TDLA10-200 (Previous WF);

  - Option 2: TDLD10-200 (Ericsson);

- Recommended WF

  - TBA

### Sub-topic 2-2: PBCH Demod Requirements

Issue 2-2-1: Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS);

- Proposals

  - Option 1: TDLA30-200 (Apple);

  - Option 2: TDLA30-650 (Ericsson, Nokia);

- Recommended WF

  - TBA

# Topic #2: CSI Performance Requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2218182 | Apple | Proposal #1: Introduce CQI reporting requirements with rank 1 for FR2-2, in line with PDSCH demod requirements. Observation #1: It would not be impractical to introduce CQI reporting requirements at SNR ≥ 10dB given testable SNR limitation. Observation #2: CQI corresponding to 64QAM is reported at SNR ≥ 10dB for rank 1 and at SNR ≥ 13 dB for rank 2. Proposal #2: Do not introduce requirements for SNR corresponding to CQI reported for 64QAM due to testable SNR limitation. Proposal #3: Introduce CQI reporting requirements in FR2-2 at SNR points 0/1 dB for QPSK and 6/7 dB for 16QAM with rank 1. |
| R4-2218311 | Ericsson | Observation 1: Following RAN1 and RAN4 PDSCH agreements where only Rank 1 has been considered, our view is to define CQI reporting with Rank 1 where 64QAM can be considered. Proposal 1: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 KHz/100 MHz with Rank 1. • Consider PN with CPE compensation only. • Tx EVM = 6% • Test points: o SNR = 8/9 dB for 64 QAM o SNR = 2/3 dB for 16QAM. o SNR = -2/-1 dB for QPSK. Observation 2: Some companies raised the interest to consider CQI reporting with Rank 2 following FR2-1. In the following Proposal we show that only 16QAM can be considered. Proposal 2: Define the CQI reporting definition test for 2Rx UE with CQI table 1 (64QAM), see Appendix A.1, by reusing the existing test setup and metrics for SCS 120 kHz/100 MHz with Rank 2. • Consider PN with CPE compensation only. • Tx EVM = 6%. • Test points: o SNR = 5/6 dB for 16QAM o SNR = -2/-1 dB for QPSK. Proposal 3: We prefer considering CQI reporting under static conditions and considering Rank 1 only for the following reason • Align with RAN1 conclusions • Consider 64QAM. |
| R4-2219061 | Nokia | Rank to use for the definition of CQI Requirements Observation 1: To our understanding, simulation results up till now has been provided with Rank 1. Proposal 1: Define CQI requirements using Rank 1. Whether to define CQI requirements including 64QAM Observation 2: Maximum feasible SNR for CQI requirements is 8/9dB. Observation 3: Using 8/9dB SNR will based on our simulations result in CQI Index of 7/8, which is in the 16QAM range, hence 64QAM is not feasible. Proposal 2: Define CQI requirements up to 16QAM SNR Test points for 64QAM Observation 4: Maximum feasible SNR for CQI requirements is 8/9dB. Observation 5: Using 8/9dB SNR will based on our simulations result in CQI Index of 7/8, which is in the 16QAM range, hence 64QAM is not feasible. Proposal 3: Do not define requirements for 64QAM. SNR Test points for 16QAM Observation 6: Maximum feasible SNR for CQI requirements is 8/9dB. Proposal 4: Define SNR test points for 16QAM as 8/9dB SNR Test points for QPSK Observation 7: Both -2/-1dB and 0/1dB will result in QPSK being selected, hence we have no preference for either. |
| R4-2219505 | Huawei | In the last meeting, we provide our views on remain issues on CQI reporting requirements for FR2-2. The proposals are: Proposal 1: Define FR2-2 CQI requirements for rank 2. Proposal 2: Set SNR value to -2/-1 dB for QPSK and 8/9 dB for 16QAM. |
| R4-2219842 | Qualcomm | Observation 1: For the purposes of CQI Requirements in FR2-2, consider Max SNR = 9.8dB; Proposal 1: For the definition of CQI Requirements, R4 should use Rank 2; Proposal 2: For the definition of CQI Requirements, R4 should not consider 64QAM; Proposal 3: For 16QAM CQI Requirements, we support using 8/9 dB Test Points; Proposal 4: For 16QAM CQI Requirements, we support using 0/1 dB Test Points; |
| R4-2218312 | Ericsson | Observation 1: The results show that the lowest SNR to report efficiently a CQI with Rank 1 related to • 64QAM is at SNR = 8/9 dB (CQI 10). • 16QAM is at SNR = 2/3 dB (CQI 7). • QPSK is at SNR = -5/-4 dB (CQI 3). Observation 2: The results show that the lowest SNR to report efficiently a CQI with Rank 2 related to • 16QAM is at SNR = 5/6 dB (CQI 7). • QPSK is at SNR = -5/-4 dB (CQI 3). |
| R4-2219062 | Nokia | In this document we have provided our simulation results for CQI. |
| R4-2219506 | Huawei, HiSilicon | In this paper we provide our simulation results for FR2-2 CQI requirements. |

## Open issues summary

### Sub-topic 3-1: CQI Requirements

Issue 3-1-1: Rank to use for CQI Reporting requirements;

- Proposals

  - Option 1: Rank 1 (Apple, Ericsson, Nokia);

  - Option 2: Rank 2 (Huawei, Qualcomm);

- Recommended WF

  - TBA

Issue 3-1-2: Whether to define CQI requirements including 64QAM

- Proposals

  - Option 1: No (Apple, Nokia, Qualcomm, Huawei);

  - Option 2: Yes (Ericsson);

- Recommended WF

  - TBA

Issue 3-1-3: SNR Test point for 64QAM, if agreed

- Proposals

  - Option 1: 8/9, Rank 1 (Ericsson);

- Recommended WF

  - TBA

Issue 3-1-4: SNR Test point for 16QAM

- Proposals

  - Option 1: 6/7dB, Rank 1 (Apple);

  - Option 2: 2/3dB, Rank 1 (Ericsson, preferred);

  - Option 3: 8/9dB, Rank 1 (Nokia);

  - Option 4: 8/9dB, Rank 2 (Huawei, Qualcomm);

  - Option 5: 5/6dB, Rank 2 if agreed; (Ericsson);

- Recommended WF

  - TBA

Issue 3-1-5: SNR Test point for QPSK

- Proposals

  - Option 1: 0/1dB, Rank 1 (Apple, Nokia);

  - Option 2: -2/-1dB, for both Rank 1 and 2 (Ericsson, Nokia, Huawei);

  - Option 3: 0/1dB, Rank 2 (Qualcomm);

- Recommended WF

  - TBA