3GPP TSG-RAN WG4 Meeting # 105												R4-2220187

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			6.3.6.1, 6.3.6.2

Source:	Qualcomm

Title:	Topic summary for [105][321] NR_exto71GHz_Demod_Part2

Document for:	Information

# Way Forward

Main technical topic overview. The structure can be done based on sub-agenda basis.

## General

Extended maximum Testable SNR for reduced allocation

The extended table below is agreed in RAN4#105, containing the Maximum DL testable SNR for band n263 used for the definition of the requirements.

[ ] are included to keep alignment with the Table 7.2.3-2 in 38.884, v 18.2.0 used as reference.

|  | CBW (MHz) | Num RBs / SCS (KHz) | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 / 120 | [9.8] |
|  |  | 32 / 120 | [13.2] |
|  | 400 | 66 / 480 | [2.6] |
|  |  | 32 / 480 | [6.6] |
|  |  | 20 / 480 | [8.9] |

## PDSCH Requirements for FR2-2 Single Carrier

RB allocation for 120kHz/100MHz for MCS 17 for 70% requirements

- Tentative Agreement from offline discussion:

Considering that most of the companies submitted alignment and impairment results for both options of RB allocation under consideration ([66, 32] RBs), companies agree to introduce both requirements, subject to the following applicability rule:

- For UE supporting FR2-2, the PDSCH Demod test for 120kHz/100MHz with MCS17 (64QAM, 0.43) with 32RBs allocation applies only if the correspondent requirement cannot be tested using 66 RBs due to testable SNR restrictions;

Issue 1-2-2: RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements

- Agreement: 20RBs;

Whether to reduce PDCCH CORESET BW for PDSCH Requirements with reduced PRB Allocation

- Agreement: Reduce PDCCH CORESET BW if larger than PDSCH RB allocation (Apple)

PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=32RBs;

- Proposals

  - Option 1: CORESET BW=30PRB, Duration = 1Sym, AL=4 (Apple);

Agreement: If PDSCH Requirements with PDSCH Allocation=32RBs introduced, option 1 shall be considered as starting point for further discussion.

PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs;

- Proposals

  - Option 1: CORESET BW=18PRB, Duration = 2Sym, AL=4 (Apple);

Agreement: If PDSCH Requirements with PDSCH Allocation=20RBs introduced, option 1 shall be considered as starting point for further discussion.

Margin for RF Impairments for the PDSCH Demodulation Requirements

- Agreement: Up to UE Implementation;

Whether the TE transmit TRS in the full CBW during PDSCH testing with Reduced RB allocation

- Tentative Agreement:

Do not multiplex PDSCH with TRS and transmit full CBW TRS (including unallocated PRBs) (Nokia, Qualcomm);

## PDSCH Requirements

Tentative PDSCH requirements definition

- Tentative Agreement from offline discussion:

Based on the simulation results collected in this meeting, the table below is extracted from R4-2218306.

For the highlighted requirement values, the alignment simulation results span was over 2.5 dB.

To define at least tentative requirements for these tests, a compromise has been reached by excluding the 2 lowest reported SNRs, considered as outliers, and choosing the lowest SNR reported among the 3 reported values left.

The span for non-highlighted requirements were averaged across all reported values.

| Duplex CBW/SCS | Antenna configuration | RB allocation | MCS and rank | Channel model | % of Peak Throughput | Tentative Requirement | Max Testable SNR (Info) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD 100MHz/120kHz | 2x2 Low | 66 | MCS4, Rank 1 | TDLA30-650 | 70 | 1.0 | 9.8 |
|  |  | 66 | MCS13, Rank 1 | TDLA30-200 | 70 | 8.9 | 9.8 |
|  |  | 66 | MCS13, Rank 1 | TDLA30-650 | 30 | 2.6 | 9.8 |
|  |  | 66 | MCS17, Rank 1 | TDLD30-200 | 70 | 11.4 | 9.8 |
|  |  | 32 | MCS17, Rank 1 | TDLD30-200 | 70 | 11.5 | 13.2 |
| TDD 400MHz/480kHz | 2x2 Low | 66 | MCS4, Rank 1 | TDLA10-200 | 70 | 0.9 | 2.6 |
|  |  | 20 | MCS13, Rank 1 | TDLD10-200 | 70 | 8.0 | 8.9 |

Simulation alignment for RAN4#106

- Tentative Agreement from offline discussion:

The companies have agreed to further check their simulations and submit the following results to the next meeting, to facilitate the alignment effort and to try to reduce the span in the results:

- Demod simulation results for alignment, excluding PN impairment modeling;

- Impairment results for the definition of the requirements, including RF and PN impairment;

On the basis of the compromise reached for the test cases highlighted for this section, the companies have also agreed on a way forward to eventually update the requirement in the next meeting based on updated results:

- If the updated alignment results span for the highlighted cases is below 2.5dB, the requirements will be updated based on the averaged results;

- If the updated alignment results span for the highlighted cases is still above 2.5dB,

  - if the results show no significant change with respect to results collected in RAN#105, the current compromise will be used for the definition of the requirements;

  - otherwise, companies reserve the option to revise the current compromise;

## PDSCH Requirements with FR1+ FR2-2 CA

Scope of CA Requirements for FR1+FR2-2

- Agreement:

Reuse single carrier requirements;

FFS: Whether to introduce CA requirements for cases not covered by Single carrier requirements;

Applicability rule for FR1+FR2-2 CA requirements

- Tentative Agreement from offline discussion:

Copy all cases of single CC to CA, and define applicability rules as follows:

- If the FR2-2 UE supports FR2-2 Single Carrier operations only, UE is tested for the Single Carrier requirements only;

- If the FR2-2 UE supports FR2-2 CA operations only, UE is tested for the CA requirements only;

- If the FR2-2 UE supports both Single Carrier and CA operations, UE is tested for the Single Carrier requirements and one test from the CA requirements;

  - The chosen CA requirement applicable to this case is the one for MCS 13;

Common understanding for information: Test setup for Single Carrier should consider FR2-2 UEs that do not support FR2-2 CA operations and do not support Initial Access on FR2-2 carriers (optional capabilities initalAccessSSB-120kHz-r17 and initialAccessSSB-480kHz-r17).

BW choice and test setup for FR1+FR2-2 Carrier for CA Requirements

- Tentative Agreement from offline discussion:

Configure FR1(40MHz, single CC) + FR2-2(400MHz, single CC);

To reuse 120kHz requirements for Single Carrier defined with CBW=100MHz, the correspondent CA test setup is as follows:

- Allocation for PDSCH/PDCCH follows the same RB size chosen for the single carrier requirements;

- Allocation is full CBW for other reference signals (e.g. TRS);

Configuration for FR1 Carrier for CA Requirements

- Agreement: Configure according to the table below;

- Table 2-4: Proposed test parameters for Pcell CC;

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | 7D1S2U S=6D:4S:4U |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | See FR1.30-1 as defined in Annex A.1.2 |

Scheduling and feedback pattern for FR1+FR2-2 CA Requirements

- Agreement:

Scheduling and feedback pattern for FR1+FR2-1 CA requirements:

- For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

- For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

Channel Model for FR2-2 PDCCH Requirements Test #5 (480kHz SCS, AL4);

- Agreement: Remove test #5 for PDCCH due to testable SNR limitation.

Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS);

- Agreement: TDLA30-650;

Rank to use for CQI Reporting requirements;

- Agreement: Rank 1;

SNR Test point for high modulation order

- Agreement: [7/8] dB, companies can further check;

SNR Test point for low modulation order

- Agreement: 0/1 dB;