3GPP TSG-RAN WG4 Meeting # 105	R4-2220278

Toulouse, France, November 14th – November 18th, 2022

Title: 	WF on IoT-NTN UE demodulation and CQI reporting requirements

Agenda Item:	9.5.9

Source: 	MediaTek

Document for:	Approval

Introduction

This document is to capture the all agreements and open issues for [105][328]IoT_NTN_Demod in RAN4#105.

Topic #1: IoT-NTN UE demodulation and CSI reporting requirements

Sub-topic 1-1: Test Scope

Issue 1-1-1: Requirements for different NTN scenarios

Agreement:

- Only consider LEO-600 scenario for LTE NTN IOT performance requirements definition.

- Adopt the following applicability rule for Cat-M1 UE for NTN.

| ntn-Connectivity-EPC-r17 | ntn-ScenarioSupport-r17 | Applicability |
| --- | --- | --- |
| Supported | GSO only | UE needs to pass TS36.101 Cat-M1 requirements only according to the UE capability |
| Supported | NGSO only | UE needs to pass TS36.101 Cat-M1 requirements and the additional NGSO test in TS36.102 according to the UE capability |
| Supported | Not specified (supporting both GSO and NGSO) | UE needs to pass TS36.101 Cat-M1 requirements and the additional NGSO test in TS36.102 according to the UE capability |
| Not supported | - | UE needs to pass TS36.101 Cat-M1 requirements only according to the UE capability |

- Adopt the following applicability rule for Cat-NB1/NB2 UE for NTN.

| ntn-Connectivity-EPC-r17 | ntn-ScenarioSupport-r17 | Applicability |
| --- | --- | --- |
| Supported | GSO only | UE needs to pass TS36.101 Cat-NB1/NB2 requirements only according to the UE capability |
| Supported | NGSO only | UE needs to pass TS36.101 Cat-NB1/NB2 requirements and the additional NGSO test in TS36.102 according to the UE capability |
| Supported | Not specified (supporting both GSO and NGSO) | UE needs to pass TS36.101 Cat-NB1/NB2 requirements and the additional NGSO test in TS36.102 according to the UE capability |
| Not supported | - | UE needs to pass TS36.101 Cat-NB1/NB2 requirements only according to the UE capability |

Issue 1-1-2: Operation mode for NB-IoT over NTN

Agreement:

- Consider only standalone deployment to define requirements for NB-IoT over NTN.

Issue 1-1-3: Duplex mode for Cat-M1 over NTN

Agreement:

- Define UE demodulation requirement for FDD only, both HD-FDD and FD-FDD

Issue 1-1-4: Channel bandwidth for Cat-M1 over NTN

Agreement:

- Define UE demodulation requirement for channel bandwidth 1.4MHz.

Issue 1-1-5: CSI reporting requirements

Agreement:

- Do not introduce new CSI requirements for both NB-IoT and Cat-M1 over NTN.

Issue 1-1-6: PBCH and PDCCH requirements

Agreement:

- Do not introduce new NPBCH and NPDCCH requirements for NB-IoT over NTN.

- Do not introduce new PBCH and MPDCCH requirements for Cat-M1 over NTN.

- Note: PDCCH performance can be implicitly verified under PDSCH requirements.

Sub-topic 1-2 General assumptions

Issue 1-2-1: Frequency and timing drift modelling

Agreement:

- Do not consider Doppler shift for feeder link for DL

- Do not consider the frequency Drift

- Do not define sampling offset model

- The maximum Doppler shift is residual frequency offset, i.e., 0.1ppm.

Issue 1-2-2: Propagation channel models

Agreement:

- Introduce requirements with NTN-TDLA100-x and NTN-TDLC5-y

- FFS on the doppler shift value

- This only applied for UE side. FFS for SAN side

Issue 1-2-3: Antenna configuration

Agreement:

- Define requirements with 1Tx1Rx for both Cat-M1 and NB-IoT over NTN.

Issue 1-2-4: K_offset

Agreement:

- Define requirements with K_offset = 8ms for both Cat-M1 and NB-IoT over NTN.

Issue 1-2-5: Number of HARQ processes

Agreement

- For Cat-M1 UE, define PDSCH demodulation requirements with HARQ retransmission with:

  - 8 HARQ processes for CE Mode A.

  - 2 HARQ processes for CE Mode B.

- For Cat-NB1/NB2 UE, define NPDSCH demodulation requirements with HARQ retransmissions with 1 HARQ process.

Sub-topic 3: Test cases for Cat-M1

Issue 1-3-1: Simulation assumption (for initial simulation purpose)

Interested companies are encouraged to provide the simulation results in RAN4#106. Other options are not precluded.

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Table 8.11.1.1.1.1-1 Test 1) | FDD 1.4MHz | 16QAM 1/2 (Reuse R.79 FDD) | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms |
| 2 (Table 8.11.1.1.2.1-1 Test 1) | FDD 1.4MHz | QPSK 1/3 (Reuse R.80 FDD) | TM1 | 8 | NTN-TDLA100-x Option 1: x=200Hz Option 2: x=10Hz | 1x1 | 8ms |
| 3 (Table 8.11.1.1.3.1-1 Test 1) | FDD 1.4MHz | QPSK 1/10 (Reuse R.81 FDD) | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms |

Issue 1-3-2: MPDCCH/PDSCH scheduling

FFS the how many MPDCCH/PDSCH subframes are dropped/punctured by PSS/SSS/PBCH/SI, when MPDDCH/PDSCH uses repetitions.

Sub-topic 4: Test cases for NB-IoT

Issue 1-4-1: Simulation assumption (for initial simulation purpose)

Interested companies are encouraged to provide the simulation results in RAN4#106. Other options are not precluded.

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Table 8.12.1.1.2-2 Test 1) | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-y Option 1: y=200Hz Option 2: y=30Hz | 1x1 | 8ms |
| 2 (Table 8.12.1.1.2-2 Test 2) | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 256 | NTN-TDLA100-10 | 1x1 | 8ms |