3GPP TSG-RAN4 Meeting #105	R4-2220295

Toulouse, FR, November 14 – November 18, 2022

Title: 	TP for TR 38.877: On definition of FR2-1 multi-band BS in clause 6.1

Source: 	CATT, Huawei

Agenda item:	8.4.2

Document for:	Approval

- Introduction

This contribution provides a text proposal for TS 38.877: On definition of FR2-1 multi-band BS in clause 6.1 [1, 2].

- Reference

[1] TS 38.877, 0.0.1

[2] R4-2217450, WF on definition of FR2-1 multi-band BS, CATT, RAN4#104-bis-e

- Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

6	Study on RF requirements

6.1	Definition of FR2 multi-band BS

The existing definitions of single-band RIB and multi-band RIB for FR1 defined in TS 38.104 are shown as below:

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band.

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

From definition of multi-band RIB for FR1, the transmitter or receiver of multi-band RIB dependent BS have the capability to process two or more carriers from the different bands in common active RF components simultaneously. For FR2-1, the existing multi-band RIB definition for FR1 can be reused.

As the same with FR1, the existing explanations on BS capable of supporting operation in multiple operating bands in 38.104 [xx] for FR1 are sufficient. The same descriptions can be applied to FR2-1, i.e.

BS type 1-O and BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

---------------------------------------------------End of Text proposal---------------------------------------------------------