3GPP TSG-RAN WG4 Meeting # 105	   R4-2220544

Toulouse, France, November 14th – November 18th, 2022

Title: 	WF for the study on expanded and improved NR positioning

Agenda Item:	8.19.5

Source: 	Intel Corporation

Document for:	Approval

# Study of accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers

## TAE for single RF chain architecture

Agreement:

- No need to discuss TAE for single RF chain architecture

  - TAE refers to the TAE requirements across carriers in BS specification

## Content to be included in LS to RAN1

Agreement:

- PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single chain Tx/Rx architectures at both the UE and gNB

- The assumption for a single-chain Tx architecture is that PRS/SRS resources to be aggregated are transmitted from a single Tx antenna

  - Single chain Tx architecture means the single RF chain with single antenna

# Study of accuracy improvement based on NR carrier phase measurements

## RAN4 study on RF requirement

Agreement:

- RAN4 will uphold its previous agreement on RF issues related to carrier phase measurement for positioning

  - Agreement from RAN4 #104Bis-e (R4-2217739): RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement.

- Carrier phase measurement discussions will be left for WI phase

# References

- R4-2220117, “Topic summary for [105][137] FS_NR_pos_UERF,” Moderator (Intel Corporation), RAN4 #105, November 2022