3GPP TSG RAN WG4 Meeting #105	                                                                               R4-2220585

Toulouse, France, 14th – 18th November, 2022

Title:	[Draft] Reply LS on NS_50 A-MPR

Response to:	LS R5-225650 on A-MPR regions for NS_50 (Power Class 2) from RAN5

Release:	Rel-17

Work Item:	NR_UE_PC2_n39-Core

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact person:

Name: Jin Wang

E-mail Address: jinwang@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to thank RAN5 for the LS on A-MPR regions for NS_50 (Power Class 2) in R5-225650. It is RAN4’s understanding that the A-MPR regions are defined in terms of frequency limits within a given channel bandwidth. If there is no valid RB allocation in a given region due to e.g., increased SCS, the specified A-MPR value is not applicable.

More specifically, RAN4 has made the following observations regarding the test points for SCS=60kHz.

Observation 1: For channel BW=10MHz and SCS=60kHz, no RB would be allocated to the A-MPR region with A-MPR value A4. Hence, no A-MPR is applicable for this case.

Observation 2: For channel BW=15MHz and SCS=60kHz, the test point may be defined using RB_start=5 and L_CRB=13 for CP-OFDM, and using RB_start=6 and L_CRB=12 for DFT-s-OFDM.

Observation 3: For channel BW=20MHz and SCS=60kHz, the test point may be defined using RB_start=7 and L_CRB=17 for CP-OFDM, and using RB_start=8 and L_CRB=16 for DFT-s-OFDM.

# 2	Actions

To TSG RAN WG5

ACTION: 	RAN4 respectfully asks RAN5 to take the above information into account when developing the test specifications for NS_50 A-MPR for PC2.

# 3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #106	27Feb -03 Mar 2023         Athens, GR

TSG-RAN WG4 Meeting #106-bis-e	17 - 26 April 2023             E-meeting