3GPP TSG RAN WG4 #106			R4-2300061

Meeting, Feb 27th – Mar 3rd, 2023

Source:	Kumu Networks, INC.

Title:	SBFD feasibility study and RF impact on BS aspects

Agenda Item:	9.19.2.2.1

Document for: 	Discussion

# Introduction

At RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss on the feasibility and RF impact of the subband non-overlapping full duplex feature for NR duplex evolution.

# SBFD feasibility study and RF impact on BS aspects

# Overview

In meeting #104 and #104-bis-e discussions, we discussed the benefits of using RF cancellation in handling 3rd order non-linearity in the low noise amplifier, and discuss the RSIC values achievable with frequency domain beam-forming, beam-nulling and RF cancellation with a set of randomly placed  uplink and downlink UEs.

In this contribution, we extend our results to show the performance achievable under 3GPP_38.901_Uma_LOS channel model using models provided by Quadriga [9]. The results continue to show high feasibility of using RF cancellation together with beam-forming and beam-nulling to achieve viable SBFD system.

To recap, our simulation model is as follows:

Figure 1 Simulation model

The simulation model shows signal flow in black and beam-forming / RF SIC algorithm optimization targets in red. More details on the optimization procedure can be found in [3].

# Self-interference modelling and Simulation Results

# Mitigation of LNA 3rd order non-linearity

As in previous contribution, we simulated the effect of worst case antenna isolation (65 dB) on the receiver front end LNA and the result is summarized as follows. Our simulation uses 4 DL users and 1 UL user with frequency domain beam-forming per 4 RB span. Both the UL and DL gNB-UE channel is based on 3GPP_38.901_Uma_LOS channel, with DUD channel occupancy (2x40 Mhz total downlink bandwidth and 20MHz uplink bandwidth). For detail of our setup, please reference our contribution (R1-2205815) [3] shared on RAN 1 meeting #110. Further hardware details can be found [4].

Using an antenna configuration of 32 TX and 32 RX, and a total 192 cancellers, the effect of residue self-interference on the receiver LNA is shown in Figure 2.

MEAN ISOLATION (WORST RX) = -66.35 dB

|  | MEAN ISOLATION (WORST RX) = -82.87 dB | MEAN ISOLATION (WORST RX)= -98.26 dB |
| --- | --- | --- |
| - Basic beam-forming | - Beam-forming + beam-nulling without canceller | - Beam-forming + beam-nulling with canceller |

Figure 2: 32+32 SBFD configuration: Combined self-interference blocker channels per RX (pre Rx LNA). Each line represents the TX beam-formed channel at one of the 32 receivers. RF cancellation before Rx LNA mitigates saturation of the LNA.

Figure 2 compares the blocker rejection performance at each receiver, with transmit beam-forming coefficients optimizing for four fixed downlink UE positions, at the relevant TX frequency 3.66 GHz – 3.70  GHz and 3.72 GHz – 3.76 GHz. While the basic beam-forming optimization without beam-nulling performs the worst at -66.35 dB (Figure 2(a)), adding beam-nulling into the optimization objective without a canceller improves the isolation to -82.87dB (Figure 2(b)), which is insufficient for receiver protection. With 53dBm transmit power, -82.87dB isolation results in a -29.87dBm blocker at the receiver, which would cause saturation on a LNA with IIP3 of -20 dBm as shown in Figure 3(a).

Adding RF cancellers and jointly optimizing with beam-nulling, as shown in Figure 2(c), the isolation improves to -98.26dB, which results in -45.26dBm blocker power at the receiver in the worst case. Receivers can accommodate this blocker without affecting the signal integrity at the receiver frequency as shown in Figure 3(b). Note that this is a conservative estimate taken from the worst case transmit residue at the input of the receiver LNA. The mean blocker power is about -49.9 dBm, about 5 dB better than the worst case.

dBm@100MHz BW

dBm@100MHz BW

LNA Saturation

No LNA Saturation

LNA input per RX Link @ iip3 = -20 dBm

LNA input per RX Link @ iip3 = -20 dBm

| (a) Beam-forming + beam-nulling without canceller. Stronger residual blocker causes LNA saturation and higher noise floor on the RX band | (b)Beam-forming + beam-nulling with canceller. Weaker residual blocker, so no LNA saturation caused |
| --- | --- |

Figure 3 RX LNA spectral plot showing the worst case and best case residue for 32+32 Beam-forming + beam-nulling with canceller for combined self-interference ACL channels per RX. The worst case residue is low enough that no saturation is observed at the output of the LNA with IIP3 of -20 dBm when beam-nulling and RF cancellation is jointly performed. The purple line represent the worst case signal passing through an ideal LNA while the red signal represent the worst case signal passing through a LNA with IIP3 of -20 dBm.

| MEAN ISOLATION (WORST TX) = -60.47 dB | MEAN ISOLATION (WORST TX) = -77.28 dB | MEAN ISOLATION (WORST TX) = -103.31 |
| --- | --- | --- |
| - Basic beam-forming | - Beam-forming + beam-nulling without canceller | - Beam-forming + beam-nulling with canceller |

Figure 4: 32+32 SBFD configuration: Combined self-interference ACL channels per TX. Each line represents the RX beam-formed channel from one of the 32 transmitters.

Figure 4 shows the ACL rejection performance at the RX frequency, 3.7 – 3.72 GHz, from each transmitter with receiver beam-forming optimizing for a fixed UL UE location. Like the blocker channel results, beam-nulling alone cannot provide the needed isolation, as shown in Figure 4(b), because -77.28 dB isolation would result in 53dBm TX Pwr – 45dB ACLR – 77.28 dB isolation = -69.28dBm on-channel noise at the receiver which degrades the RX sensitivity significantly.

However, the joint optimization with cancellers achieves -103.31dB isolation (Figure 4(c)), which results in -95.31dBm on-channel noise. Given the noise floor for 100MHz is typically around -89 dBm (-94dBm thermal noise floor plus 5 dB receiver noise figure), the ACL residual impact on receiver sensitivity would be 0.91 dB. Note that this is for the RSI contribution of the worst case transmit element. The mean isolation is -108.68, which is about 5 dB better than the worst case.

dBm@100MHz BW

Combine Thermal Noise Floor (All Rx Links) -74 dBm dBm

-87.2 dBm

-84.5 dBm

Figure 5 Overall self-Interference Residue in the RX Band after Rx Beam-nulling and RF cancellation.

Figure 5 shows the overall self-interference residue after receive beam-nulling and RF cancellation in the receive band when a 16-QAM signal is transmitted.

As a reference, we first calculate the total noise power at the receiver without any interference present. The receiver combines 32 chains, so the noise from all chains also gets combined. The noise power at the receiver over 100MHz assuming 5dB noise figure at every receive chain is:

-89 dBm + 15.02 dB = -74.02 dBm.

As Figure 5 shows, the overall signal residue, when no receive non-linearity is present, is about -87.2 dBm. This is 8.07 dB higher than the self-interference residue from the worst case transmitter (rather than ~15dB higher), as the other 31 remaining transmitters are not contributing as much.  When accounting for IIP3 effects of all the receiver links, the combined signal residue is -84.5 dBm. This is still significantly lower than the -74.02dBm total noise power. The receiver degradation is thus merely 0.6 dB.

Observation 1:

Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

Proposal 1:

RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue.

Proposal 2:

Overall RSIC reporting should include pre-LNA RF mitigation as a separate capability. For example, total RSIC can be given by:

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-RFIC)+dB(αSI-digtial)+…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beam-nulling or beam isolation.

      - αSI-RFIC denotes the pre-LNA RF interference cancellation capability.

      - αSI-digtial denotes the digital cancellation capability.

Observation 2:

Using 3GPP_38.901_Uma_LOS channel, we show that the worst case rx sensitivity per Rx link is 0.9 dB and the mean case rx sensitivity per Rx link is 0.6 dB.

Proposal 3:

We support using overall Rx sensitivity degradation of 1 dB for SBFD system simulation and evaluation.

# Impact on transmit beam-forming gain for Downlink UEs

We also studied the impact of self-interference cancellation on downlink gNB -UE beam-forming gain when operating SBFD in 3GPP_38.901_Uma_LOS channel.

Figure 6 gNB-UE downlink beam-forming gain CDF comparing “beam-forming only” and “RF Cancellation and beam-nulling”.

The “beam-forming only” gNB -UE downlink gain serves as a reference for the achievable beam-forming gain when operating in 3GPP_38.901_Uma_LOS channel environment. With the joint cancellation using “RF cancellation and beam-nulling”, beam-forming gain degradation can be controlled within ~2 dB. These beam-forming results are also subject to UE placement and more intelligent UE scheduling can recover the remaining beam-forming loss as well.

Observation 3:

A small (~2 dB) degradation of beam-forming gain is observed with RF cancellation and beam-nulling working jointly to reduce self-interference residue when operating in 3GPP_38.901_Uma_LOS channel

Proposal 4:

When evaluating self-interference cancellation methodology, it is useful to also report the achievable beam-forming gain.

Based on the above-described simulation results, we report the following table for RSIC capability for SBFD gNB operation:

| FR1 (or FR2-1) |  |  | Kumu Networks, Inc. |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (High) | Wide Area BS (Mean) | - |
| BS TX Power dBPTX = ① dBm |  |  | 53 dBm | 53 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc | 45 dB | 45 |  |
|  |  | Frequency isolation techniques used | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc | 65 dB | 65 |  |
|  |  | Spatial isolation techniques used | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  | 17.87 dB | 21.3 |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. (Note 1) dBPSI_leakage |  | -57 dBm | -57 dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc | 15.39 dB | 16.6 |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc | 26.03 dB | 26.2 |  |
|  |  | RF IC techniques and other tech. (before LNA) | Analog RF IC joint tune with beam-nulling RF SIC Complexity : 192 RF taps for 32 Tx and 32 Rx antennas |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA | 0.2 dB (assuming 15 dB Rx coupler) | 0.2 |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA (Note 1) ①-③-④-⑤ dBPSI_TXSB |  | <-45.26 dBm | -49.9 |  |
|  | Frequency isolation at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc | 0 dB | 0 |  |
|  |  | [Frequency isolation] techniques | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX |  |  |
|  | Rx IMD | Rx IIP3 capability (dBm) | -20 | -20 |  |
|  |  | Rx IM3 contribution (dBm) | -95.78 | -109.7 |  |
|  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | 5 dB (noise figure) | 5 |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized (Note 1, 2) dBPSI_Selectivity |  | -95.78 dBm | -109.7 |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  | 12.28 dB | 17.5 |  |
|  | RX sensitivity degradation caused by RX beam nulling |  | TBD (?) | TBD |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  | 0 dB | 0 |  |
| Overall RSIC capability dBRSIC (Note 1) ②+③+⑧+⑨ |  |  | 148.31 dB | 153.7 |  |
| Noise floor ⑩dBm |  |  | -89 dBm/100MHz | -89 dBm/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  | -95 dBm | -95 |  |
| Required RSIC budget (①-⑪dBc) |  |  | 148 dB | 148 |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

Observation 4:

While the worst case RSI value are useful to provide insight to how each receiver link performance is impacted by self-interference residue, overall mean value is also useful to provide more typical performance expectation. In our simulation, the worst case RSIC capability is 148 dB and the mean case RSIC capability is 153 dB.

Proposal 5:

When reporting simulation results, it would be useful to clarify if it is the worst case or typical case being reported.

.

# Conclusion

In this contribution, we discussed on the evaluation of NR duplex operation. In particular, our simulation shows the worst case RSIC capability is 148 dB and the mean case RSIC capability is 153 dB.

Based on the discussion we made the following observations and proposals.

Observation 1:

Using 3GPP_38.901_Uma_LOS channel, we show that having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

Observation 2:

Using 3GPP_38.901_Uma_LOS channel, we show that the worst case rx sensitivity per Rx link is 0.9 dB and the mean case rx sensitivity per Rx link is 0.6 dB.

Observation 3:

A small (~2 dB) degradation of beam-forming gain is observed with RF cancellation and beam-nulling working jointly to reduce self-interference residue when operating in 3GPP_38.901_Uma_LOS channel

Observation 4:

While the worst case RSI value are useful to provide insight to how each receiver link performance is impacted by self-interference residue, overall mean value is also useful to provide more typical performance expectation. In our simulation, the worst case RSIC capability is 148 dB and the mean case RSIC capability is 153 dB.

Proposal 1:

RF cancellation should be used in SBFD to mitigate self-interference pre Rx LNA in terms of minimizing non-linearity effects and overall self-interference residue.

Proposal 2:

Overall RSIC reporting should include pre-LNA RF mitigation as a separate capability. For example, total RSIC can be given by:

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-RFIC)+dB(αSI-digtial)+…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (n,m) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beam-nulling or beam isolation.

      - αSI-RFIC denotes the pre-LNA RF interference cancellation capability.

      - αSI-digtial denotes the digital cancellation capability.

Proposal 3:

We support using overall Rx sensitivity degradation of 1 dB for SBFD system simulation and evaluation.

Proposal 4:

When evaluating self-interference cancellation methodology, it is useful to also report the achievable beam-forming gain.

Proposal 5:

When reporting simulation results, it would be useful to clarify if it is the worst case or typical case being reported.

# References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, TS38.830 v17.0.0, Dec. 2020.

- 3GPP R1-2205815, Kumu Networks, “RF cancellation techniques for Subband non-overlapping full duplex”, RAN1 #110, Aug 2022

- “K4051 Canceller Reference Board”. https://kumunetworks.com/wp-content/uploads/2020/04/K4051-Canceller-Product-Brief.pdf

- 3GPP R4-2215346, Qualcomm, “SBFD feasibility and impact on RF requirements: BS aspects”, Oct 2022.

- 3GPP R4-2216409, Intel, “Discussion on Total Achievable Self-Interference Cancellation”, Oct 2022.

- 3GPP R4-2216404, Ericsson, “SBFD gNB Radio and antenna considerations”, Oct 2022.

- 3GPP R4-2217464, Samsung, “WF on SBFD feasibility study and RF impact: BS aspects”, Oct 2022.

- https://quadriga-channel-model.de/

Appendix

# Simulation Parameters

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Carrier Frequency | 4 GHz |
|  | Duplex Type | SBFD |
|  | SBFD pattern | 40:20:40 SBFD (DUD) |
|  | Channel bandwidth | 40:20:40 MHz (DUD) for SBFD |
|  | Guard Band | 0 MHz |
|  | Available resource blocks | 108:52:108 (DUD) for SBFD |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 5 (4 DL and 1 UL) |
|  | Channel model | gNB-UE: 3GPP_38.901_Uma_LOS |
| BS | Array Configuration | 32x32 |
|  | Max gNB Tx Power | 53 dBm (same antenna gain) |
|  | (dv, dh) | (0.8λ, 0.5λ) |
|  | Antenna element gain | 7 dBi |
|  | Beam-forming method | Frequency domain |
|  | Antenna Isolation | 65 dB |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 10 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
|  | UE antenna | 1TX 1RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
| UE | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution | Random cluster |
|  | Traffic split | 40:20:40 DL:UL:DL |