3GPP TSG-RAN WG4 Meeting #106	R4-2300158

# Athens, Greece,  Feb. 27 - Mar.3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (Draft CR) Support of 5BDL and 2BUL of CA_n1A-n3A-n41A-n79A-n257A/G/H/I, CA_n3A-n28A-n41A-n79A-n257A/G/H/I, CA_n3A-n41A-n77A-n79A-n257A/G/H/I, CA_n28A-n41A-n77A-n79A-n257A/G/H/I |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SoftBank Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_yBDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Four 5BDL/ 2BUL combos in the title are newly introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The relevant information is added to CA configurations. (This paper is intended to skip a TP since there is no new technical study needed.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The combinations remain unsupported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.1(Table 5.2A.1-4), 5.5A.1 (Table 5.5A.1-4) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-3 ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected Portions Skipped]

Table 5.2A.1-4: Band combinations for inter-band CA between FR1 and FR2 (five bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n3-n8-n77-n257 | n1, n3, n8, n77, n257 |
| CA_n1-n3-n28-n41-n257 | n1, n3, n28, n41, n257 |
| CA_n1-n3-n28-n77-n257 | n1, n3, n28, n77, n257 |
| CA_n1-n3-n28-n79-n257 | n1, n3, n28, n79, n257 |
| CA_n1-n3-n41-n77-n257 | n1, n3, n41, n77, n257 |
| CA_n1-n3-n41-n79-n257 | n1, n3, n41, n79, n257 |
| CA_n1-n3-n77-n79-n257 | n1, n3, n77, n79, n257 |
| CA_n1-n28-n41-n77-n257 | n1, n28, n41, n77, n257 |
| CA_n1-n28-n41-n79-n257 | n1, n28, n41, n79, n257 |
| CA_n1-n28-n77-n79-n257 | n1, n28, n77, n79, n257 |
| CA_n1-n41-n77-n79-n257 | n1, n41, n77, n79, n257 |
| CA_n3-n28-n41-n77-n257 | n3, n28, n41, n77, n257 |
| CA_n3-n28-n41-n79-n257 | n3, n28, n41, n79, n257 |
| CA_n3-n41-n77-n79-n257 | n3, n41, n77, n79, n257 |
| CA_n28-n41-n77-n79-n257 | n28, n41, n77, n79, n257 |

[Unaffected Portions Skipped]

[Unaffected Portions Skipped]

Table 5.5A.1-4: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (five bands)

| NR CA configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n77A-n257A | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257G | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257H | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257I | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257J | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257K | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257L | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n1A-n3A-n8A-n77A-n257M | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257A | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257G | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257H | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257I | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257J | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257K | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257L | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n1A-n3A-n8A-n77(2A)-n257M | - | n1 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n8 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n1A-n3A-n28A-n41A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n28A-n41A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n1A-n257G CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n3A-n257G CA_n28A-n41A CA_n28A-n257A CA_n28A-n257G CA_n41A-n257A CA_n41A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n28A-n41A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n41A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n28A-n41A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n41A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n3A-n28A-n77A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  |  |  |  |  |
| CA_n1A-n3A-n28A-n77A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n28A-n77A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n28A-n77A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n3A-n28A-n79A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n28A-n79A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n28A-n79A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n79A-n257G CA_n79A-n257A CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n28A-n79A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n3A-n41A-n77A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n41A-n77A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n77A-n257A CA_n77A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n41A-n77A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n41A-n77A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n3A-n41A-n79A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n41A-n79A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n41A-n79A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n41A-n79A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n3A-n77A-n79A-n257A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n3A-n77A-n79A-n257G | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n3A-n77A-n79A-n257H | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n3A-n77A-n79A-n257I | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n3 |  | 5, 10, 15, 20, 25, 30 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n28A-n41A-n77A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n28A-n41A-n77A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n77A-n257A CA_n77A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n28A-n41A-n77A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n28A-n41A-n77A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n28A-n41A-n79A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n28A-n41A-n79A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n28A-n41A-n79A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n28A-n41A-n79A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n28A-n77A-n79A-n257A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n28A-n77A-n79A-n257G | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n28A-n77A-n79A-n257H | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n28A-n77A-n79A-n257I | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n1A-n41A-n77A-n79A-n257A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n1A-n41A-n77A-n79A-n257G | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n1A-n41A-n77A-n79A-n257H | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n1A-n41A-n77A-n79A-n257I | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 |  | 5, 10, 15, 20 | 0 |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n3A-n28A-n41A-n77A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |  |
| CA_n3A-n28A-n41A-n77A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n77A-n257A CA_n77A-n257G | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257G |  |  |  |
| CA_n3A-n28A-n41A-n77A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257H |  |  |  |
| CA_n3A-n28A-n41A-n77A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n77A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n3 |  | 5, 10, 15, 20, 25, 30, 40 | 0 |  |  |
|  |  | n28 |  | 5, 10, 15, 20, 30 |  |  |  |
|  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  | n257 |  | CA_n257I |  |  |  |
| CA_n3A-n28A-n41A-n79A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n3A-n28A-n41A-n79A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n79A-n257A CA_n79A-n257G | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n3A-n28A-n41A-n79A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n3A-n28A-n41A-n79A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n3A-n28A-n77A-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n3A-n28A-n77A-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n3A-n28A-n77A-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n3A-n28A-n77A-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n3A-n28A-n77(2A)-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n3A-n28A-n77(2A)-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n3A-n28A-n77(2A)-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n3A-n28A-n77(2A)-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |
|  |  | n79 | 40, 50, 80, 100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n3A-n41A-n77A-n79A-n257A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n3A-n41A-n77A-n79A-n257G | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n3A-n41A-n77A-n79A-n257H | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n3A-n41A-n77A-n79A-n257I | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n3A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77a-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n3 | 5, 10, 15, 20, 25, 30 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n28A-n41A-n77A-n79A-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n28A-n41A-n77A-n79A-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n28A-n41A-n77A-n79A-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n28A-n41A-n77A-n79A-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n41A-n257G CA_n41A-n257H CA_n41A-n257I CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n28 | 5, 10, 15, 20 |  |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. |  |  |  |  |  |  |

[Unaffected Portions Skipped]