3GPP TSG-RAN WG4 Meeting # 106		  	R4-2300234

Athens, Greece, 27 February –03 March, 2023

Title:	[draft] LS on MUSIM gap priority

Response to:

Release:	Release - 18

Work Item:	NR_DualTxRx_MUSIM-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 has discussed collisions between gaps in R18 MUSIM WI and agreed to use priority-based solution to address collision between MUSIM gaps and legacy gaps. Priority-based solution was introduced in R17 concurrent gaps design. With different priority, when gaps collide with each other, the gap with lower priority would be dropped. However, in current MUSIM gap configuration there is no priority information. Some optimization is expected on current RRC design of MUSIM gap request and configuration.

RAN4 kindly asks RAN2 to take the following RAN4 agreements into consideration and update corresponding RRC design for MUSIM in R18:

- Agreements

  - Introduction of priority for each periodic MUSIM gap. The priority level of MUSIM shall be configured in a way to be comparable to priority of other MGs.

  - No need to introduce priority for aperiodic MUSIM gap.

  - UE shall indicate the expected priority info when requesting periodic MUSIM gap. Nevertheless, it should be up to network on priority configuration.

2. Actions:

To RAN2 group.

ACTION: 	RAN4 kindly asks RAN2 to take above RAN4 agreements into consideration and update corresponding RRC design in R18.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #106-bis-e	17 Apr – 26 Apr 2023			Electronic Meeting

TSG RAN WG4 Meeting #107		              22 May – 26 Apr 2023			Incheon, Korea