3GPP TSG RAN WG4 Meeting #106                         R4- 2300296

Athens, Greece, February 27 – March 3, 2022

Title:	Discussion on LTM L1-RSRP measurement requirements

Source:	China Telecom

Agenda item:			9.23.3.2

Document for:	Discussion

- Introduction

In the RAN4 #105 meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues on L1-RSRP measurement requirements are further discussed.

- Discussion

Issue 2-1-1: Whether L1 measurement configured after receiving L3 measurement report on that cell

In last meeting, companies had different views on whether L1 measurement configured after receiving L3 measurement report on that cell. In detail, the way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (CTC, Apple, MTK, Xiaomi, Huawei): Network shall configure L1 measurement on a neighbor cell after receiving L3 measurement report on that cell - Option 2 (CATT, Ericsson): L3 measurement report is not the prerequisite of L1 measurement configuration on a neighbor cell. - Option 3 (CMCC): wait for RAN1/2 progress - Option 4 (vivo): Pending on whether intermediate results from L3 measurements is used in L1-RSRP reporting for both serving cell and candidate cell

In legacy L3 HO, after network receives the L3 measurement report, NW transmits the cell switch command which indicates the target cell to UE. Then UE performs target cell search and appropriate beam selection.

In MIMO case, cells in FR2 have large numbers of beam, which is challenge for UE to measure so many beams in the case of L1-RSRP measurement. So take the capabilities of UE into account, L3 measurement is important to indicate the scope of L1-RSRP measurement.

In addition, for FR1, in our understanding, the Tx beams and Rx beam are same for L1-RSRP measurement and L3 measurement. In this case, the intermedia result of L3 measurement could be reused as L1-RSRP measurement result, so need not to configure L1-RSRP measurement on a neighbor cell after receiving L3 measurement report on that cell.

Proposal 1: L3 measure is important to indicate the candidate cell(s) for L1 measurement before UE performs L1 measurement, except

- For FR1, the Tx beams and Rx beam are same for L1-RSRP measurement and L3 measurement, we could reuse the intermedia result of L3 measurement as L1-RSRP measurement result.

Issue 2-1-3: How to get L1-RSRP measurement results

In last meeting, companies had different views on how to get L1-RSRP measurement results. The Way forward during the last meeting is duplicated as below [1]:

< Wayforward >: FFS the following options - Option 1 (vivo, Ericsson): For both intra-frequency and inter-frequency L1 measurement, RAN4 to discuss using L3 measurement framework as baseline for LTM HO - RAN4 to assume same RX beam (rough beam) for L3 measurement and L1 measurement - RAN4 to reuse intermediate results of L3 measurement for L1 measurement. - Option 2 (Huawei): SSB based inter-frequency L1-RSRP measurement can be simultaneously performed with inter-frequency L3 measurement on the same frequency layer in FR1.

When UE performs L3 measurement, it firstly processes L1-RSRP measurement, then gets the result of L3 via filter. So we think the intermediate results of L3 measurement could be reused for L1 measurement when the Tx beams and Rx beams for L1-RSRP measurement and L3 measurement are same.

Otherwise, redo the L1-RSRP measurement.

Proposal 2: The intermediate results of L3 measurement could be reused for L1 measurement when the Tx beams and Rx beams for L1-RSRP measurement and L3 measurement are same. Otherwise, require additional L1-RSRP measurement.

- Summary

In this paper, we provide our views on L1-RSRP measurement requirements. From this discussion we have derived the following observations and proposals:

Proposal 1: L3 measure is important to indicate the candidate cell(s) for L1 measurement before UE performs L1 measurement, except

- For FR1, the Tx beams and Rx beam are same for L1-RSRP measurement and L3 measurement, we could reuse the intermedia result of L3 measurement as L1-RSRP measurement result.

Proposal 2: The intermediate results of L3 measurement could be reused for L1 measurement when the Tx beams and Rx beams for L1-RSRP measurement and L3 measurement are same. Otherwise, require additional L1-RSRP measurement.

- References

[1] R4-2220403, WF on L1/L2 inter-cell mobility, MediaTek Inc.