3GPP TSG-RAN4 Meeting #106	R4-2300344

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

Agenda item:	8.31.3

Source:	Apple Inc.

Title:	NR-U MPR for Wideband and A-MPR for LPI

WI/SI:	NR_unlic_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI, RAN#96 meeting approved a new WI [2] with the main motivation of introducing PC3 and enabling LPI and VLP in countries not covered by previous efforts

In this paper we present the simulation results for PC3 MPR for 27dB and 30dB ACLR. Also, the results values for VLP and SP for US and South Korea are provided.

# 2	Discussion

Simulations were done for unlicensed access in band n96 with PC3 and PC5. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform. The detailed list of the setup is provided in the following:

- Power Class: PC5, PC3

- Calibration: QPSK DFT-s-OFDM 100RB with 1dB backoff

- ACLR for PC3: 30dB

- ACLR for PC5: 27dB

- SEM: NR-U

- Image rejection: -28dBc

- Carrier leakage rejection: -28dBc

The PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum in-band power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the     NR-U SEM mask. Simulations were conducted for full carrier and interlace operation.

##### 2.1 PC3 MPR for Wideband

During RAN4#105 it was discussed that wideband cases need to be checked in addition to the existing analysis on standard channel bandwidths. As some concerns were raised that the power back-off need might be increased it was agreed to check several wideband cases. Table 1 lists 18 wideband cases which are considered for this contribution. The results are shown in Figure 1.

Table1: Signal Setup for MPR

| ID | Waveform | Allocation |
| --- | --- | --- |
| 1 | DFT-s-OFDM | 10 |
| 2 | DFT-s-OFDM | 100 |
| 3 | DFT-s-OFDM | 1000 |
| 4 | DFT-s-OFDM | 10000 |
| 5 | CP-OFDM | 10 |
| 6 | CP-OFDM | 100 |
| 7 | CP-OFDM | 1000 |
| 8 | CP-OFDM | 10000 |
| 9 | DFT-s-OFDM | 110 |
| 10 | DFT-s-OFDM | 1100 |
| 11 | DFT-s-OFDM | 11000 |
| 12 | CP-OFDM | 110 |
| 13 | CP-OFDM | 1100 |
| 14 | CP-OFDM | 11000 |
| 15 | DFT-s-OFDM | 1110 |
| 16 | DFT-s-OFDM | 11100 |
| 17 | CP-OFDM | 1110 |
| 18 | CP-OFDM | 11100 |

Figure 1: Required power backoff for PC3 and QPSK with 27dB ACLR

If observing Figure 1 it can be observed that the simulation indicates that certain cases require increased power back-off compared to the proposed MPR. This is due to the image falling into the wideband SEM mask. Together with spectral growth from the wanted signal the emissions at the image location can exceed the SEM limit. This effect requires increased back-off until emission comply to emission limits. Figure 2 provides exaggerated example sketches displaying the general issues for 1100 and 11000.

| Example for 1100 | Example for 11000 |
| --- | --- |

Figure 2: Approximated and exaggerated example sketches for the cases 1100 and 11000 to showcase the general issue.

Table 2: Maximum power reduction (MPR) for shared spectrum access UE power class 3

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.0 |
|  | QPSK | ≤ 2.0 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 3.0 |
|  | 64 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |

Proposal 1: Further discussion MPR impact of wideband configurations.

##### 2.2 PC3 A-MPR Setup

The signal setup for A-MPR simulations is provided in the table below. Full and interlaced allocations are simulated for all modulations and the channel bandwidth sizes 20MHz. 40MHz, 60MHz, 80MHz and 100MHz. Wideband operation covers the cases where the lowest 20MHz sub-bands are allocated while the other sub-bands are empty e.i. the sub-band configurations are 10, 100, 1000, 10000, 110, 1100, 11000, 1110 and 11100. Image versions are used when exploring performance at upper band edge.

Table 3: Signal Setup

| ID | Modulation | Waveform | Allocation |
| --- | --- | --- | --- |
| 1 | Pi/2 BPSK | DFT-s-OFDM | Full |
| 2 | Pi/2 BPSK | DFT-s-OFDM | Interlaced |
| 3 | Pi/2 BPSK | DFT-s-OFDM | Wideband |
| 4 | Pi/2 BPSK | DFT-s-OFDM | Wideband Interlaced |
| 5 | QPSK | DFT-s-OFDM | Full |
| 6 | QPSK | CP-OFDM | Full |
| 7 | QPSK | DFT-s-OFDM | Interlaced |
| 8 | QPSK | CP-OFDM | Interlaced |
| 9 | QPSK | DFT-s-OFDM | Wideband |
| 10 | QPSK | CP-OFDM | Wideband |
| 11 | QPSK | DFT-s-OFDM | Wideband Interlaced |
| 12 | QPSK | CP-OFDM | Wideband Interlaced |
| 13 | 16QAM | DFT-s-OFDM | Full |
| 14 | 16QAM | CP-OFDM | Full |
| 15 | 16QAM | DFT-s-OFDM | Interlaced |
| 16 | 16QAM | CP-OFDM | Interlaced |
| 17 | 16QAM | DFT-s-OFDM | Wideband |
| 18 | 16QAM | CP-OFDM | Wideband |
| 19 | 16QAM | DFT-s-OFDM | Wideband Interlaced |
| 20 | 16QAM | CP-OFDM | Wideband Interlaced |
| 21 | 64QAM | DFT-s-OFDM | Full |
| 22 | 64QAM | CP-OFDM | Full |
| 23 | 64QAM | DFT-s-OFDM | Interlaced |
| 24 | 64QAM | CP-OFDM | Interlaced |
| 25 | 64QAM | DFT-s-OFDM | Wideband |
| 26 | 64QAM | CP-OFDM | Wideband |
| 27 | 64QAM | DFT-s-OFDM | Wideband Interlaced |
| 28 | 64QAM | CP-OFDM | Wideband Interlaced |
| 29 | 256QAM | DFT-s-OFDM | Full |
| 30 | 256QAM | CP-OFDM | Full |
| 31 | 256QAM | DFT-s-OFDM | Interlaced |
| 32 | 256QAM | CP-OFDM | Interlaced |
| 33 | 256QAM | DFT-s-OFDM | Wideband |
| 34 | 256QAM | CP-OFDM | Wideband |
| 35 | 256QAM | DFT-s-OFDM | Wideband Interlaced |
| 36 | 256QAM | CP-OFDM | Wideband Interlaced |

##### 2.3 Introduction of 100MHz CBW to NS_53

The WF [3] from RAN4#105 recommended to agree the proposed NS_53 PC5 values for 100MHz channels and asked companies to crosscheck:

- Recommended WF - Agree on pi/2 BPSK values - with the same value as QPSK (DFT-s-OFDM precoder), These values are in []. - Other values - If other companies have time to check the values for comparison, it would be highly appreciated.

Below the simulations results for 100MHz CBW are provided. The results only cover the -27dBm/MHz out-of-band requirements at lower channel edge. Regarding the results it can be concluded that PC5 MPR provide enough power back-off allowance to achieve compliance with the emission requirements.

Figure 5: NS_53 simulation results for 100MHz CBW

Observation 1: For 100MHz CBW the power back-off need can be covered by PC5 MPR with respect to -27dBm/MHz out-of-band requirement.

##### 2.4 PC5 A-MPR for Japan LPI

In case of Japan LPI several special requirements need to be considered. ACLR is re-defined to -25dBc for the adjacent channel and requires -40dBc for the channel after. Additional emission requirement for lower band edge is defined as -27dBm/MHz. For upper edge specific regulatory requirements are defined dependent on channel size. These requirements are considered for simulations at upper and lower band edge.

Figure 6: Japan LPI simulation results for lower edge channels

Figure 7: Japan LPI simulation results for upper edge channels

Table 6: Proposed 1Tx PC5 A-MPR for Japan LPI (Lower Band Edge)

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table PC5 MPR | ≤ 2.0 | ≤ 3.0 |
|  | QPSK |  | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |

Table 7: Proposed 1Tx PC5 A-MPR for Japan LPI (Upper Band Edge)

| Pre-coding | Modulation | RB Allocation (Note 3) |  |
| --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | ≤ 2.0 | ≤ 3.0 |
|  | QPSK | ≤ 2.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 2.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to [6415 MHz], 40 MHz channels centered at the nearest NR-ARFCN corresponding to [6405 MHz] and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6385 MHz]. NOTE 3: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 4: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |

Proposal 2: Consider tables 6 and 7 when defining A-MPR for Japan LPI.

##### 2.5 PC3 A-MPR for EU LPI (NS_58)

Below the simulation results for EU LPI are shown with PC3. The maximum in-band PSD is -10dBm/MHz. This requirement is not limiting for most scenarios. However, for interlaced setup with 20MHz CBW the minimum required power backoff is expected to be at least 3dB. The simulations showed that the out-of-band emission limits were more limiting which leads to the proposal in Table 4.

Figure 8: EU LPI simulation results for edge channels

Table 8.  Proposed A-MPR for PC3

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 4.5 |
|  | QPSK | ≤ 2.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 2.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 3.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. |  |  |  |

Proposal 3: Consider table 8 when specifying PC3 A-MPR for NS_58.

# Conclusions

In this contribution we presented the wideband MPR results for PC3 and results for LPI. The following proposals and observations are made:

Proposal 1: Further discussion MPR impact of wideband configurations.

Observation 1: For 100MHz CBW the power back-off need can be covered by PC5 MPR with respect to -27dBm/MHz out-of-band requirement.

Proposal 2: Consider tables 6 and 7 when defining A-MPR for Japan LPI.

Proposal 3: Consider table 8 when specifying PC3 A-MPR for NS_58.

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2220495, “Way forward for NR-U PC3 MPR/A-MPR”, Charter Communications, Inc., 3GPP TSG-RAN WG4 Meeting #105