3GPP TSG-RAN4 Meeting #106	R4-2300629

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1321 | rev | - | Current version: | 15.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of configuration for carrier aggregation RMCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Annex A.3 defines the DL RMCs to be used for testing the Tx/Rx requirements. When these RMCs were defined, this was done with single carrier applications in mind and the RMCs were to be reused for CA as well. Since in NR we need to consider different SCS and duplex mode configurations in CA, the k1 and HARQ process values defined applicable for single carrier cannot be reused in all CA cases. To correctly schedule the ACK/NACK feedback on the PCC, the k1 values and number of HARQ processes need to be defined for different CA scenarios, similar to what is defined in 38.101-4 for demodulation CA TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add k1 and HARQ configurations for carrier aggregation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration for CA requirements remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

## A.3.1	General

Unless otherwise stated, Tables A.3.2.2-1, A.3.2.2-2, A.3.2.2-3, A.3.3.2-1, A.3.3.2-2 and A.3.3.2-3 are applicable for measurements of the Receiver Characteristics (clause 7) with the exception of clauses 7.4 (Maximum input level).

Unless otherwise stated, Tables A.3.2.3-1, A.3.2.3-2, A.3.2.3-3, A.3.3.3-1, A.3.3.3-2 and A.3.3.3-3 are applicable for clauses 7.4 (Maximum input level) and for UE not supporting PDSCH 256QAM,

Unless otherwise stated, Tables A.3.2.4-1, A.3.2.4-2, A.3.2.4-3, A.3.3.4-1, A.3.3.4-2 and A.3.3.4-3 are applicable for clauses 7.4 (Maximum input level) and for UE supporting PDSCH 256QAM,

Unless otherwise stated, Tables A.3.2.2-1, A.3.2.2-2, A.3.2.2-3, A.3.3.2-1, A.3.3.2-2 and A.3.3.2-3 also apply for the modulated interferer used in Clauses 7.5, 7.6 and 7.8 with test specific bandwidths.

In case of carrier aggregation scenarios, the k1 values and number of HARQ processes of the Reference Measurement Channels specified in Annex A.3 shall be adapted as specified in table A.3.1-2 and A.3.2-3

Table A.3.1-1: Common reference channel parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| CORESET frequency domain allocation |  |  | Full BW |
| CORESET time domain allocation |  |  | 2 OFDM symbols at the begin of each slot |
| PDSCH mapping type |  |  | Type A |
| PDSCH start symbol index (S) |  |  | 2 |
| Number of consecutive PDSCH symbols (L) |  |  | 12 |
| PDSCH PRB bundling |  | PRBs | 2 |
| Dynamic PRB bundling |  |  | false |
| Overhead value for TBS determination |  |  | 0 |
| First DMRS position for Type A PDSCH mapping |  |  | 2 |
| DMRS type |  |  | Type 1 |
| Number of additional DMRS |  |  | 2 |
| FDM between DMRS and PDSCH |  |  | Disable |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 0 for CSI-RS resource 1 and 2 1 for CSI-RS resource 3 and 4 30 kHz SCS: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 60 kHz SCS: 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |

Table A.3.1-2: Carrier aggregation test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + | FDD PCell | {2} | N/A |
| FDD 15 kHz CA |  |  |  |
| FDD 15 kHz + | 15kHz PCell | {2} | {2} |
| FDD 30 kHz CA | 30kHz PCell | {2} | {2} |
| FDD 15 kHz + | FDD PCell | {2} | {2} |
| TDD 15 kHz CA | TDD PCell | {4,3,2} | {4,3,2,6,5} |
| FDD 15 kHz + | FDD PCell | {2} | {2} |
| TDD 30 kHz CA | TDD PCell | {8,7,6,5,4,3,12} | {7,5,4,12,9} |
| TDD 15 kHz + | TDD PCell | {4,3,2} | N/A |
| TDD 15 kHz CA |  |  |  |
| TDD 15 kHz + | 15kHz PCell | {4,3,2} | {4,4,3,3,2,2,6} |
| TDD 30 kHz CA | 30kHz PCell | {8,7,6,5,4,3,2} | {7,5,4} |
| FDD 30 kHz + | FDD PCell | {2} | N/A |
| FDD 30 kHz CA |  |  |  |
| FDD 30 kHz + | FDD PCell | {2} | {2} |
| TDD 15 kHz CA | TDD PCell | {4,3,2} | {4,4,3,3,7,7,6,6,5,5} |
| FDD 30 kHz + | FDD PCell | {2} | {2} |
| TDD 30 kHz CA | TDD PCell | {8,7,6,5,4,3,12} | {8,7,6,5,4,3,12,12,10,10} |
| TDD 30 kHz + | TDD PCell | {8,7,6,5,4,3,2} | N/A |
| TDD 30 kHz CA |  |  |  |

Table A.3.1-3: Carrier Aggregation test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + | FDD PCell | 4 | N/A |
| FDD 15 kHz CA |  |  |  |
| FDD 15 kHz + | 15kHz PCell | 8 | 8 |
| FDD 30 kHz CA | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + | FDD PCell | 4 | 8 |
| TDD 15 kHz CA | TDD PCell | 8 | 8 |
| FDD 15 kHz + | FDD PCell | 4 | 8 |
| TDD 30 kHz CA | TDD PCell | 10 | 8 |
| TDD 15 kHz + | TDD PCell | 8 | N/A |
| TDD 15 kHz CA |  |  |  |
| TDD 15 kHz + | 15kHz PCell | 8 | 12 |
| TDD 30 kHz CA | 30kHz PCell | 8 | 8 |
| FDD 30 kHz + | FDD PCell | 8 | N/A |
| FDD 30 kHz CA |  |  |  |
| FDD 30 kHz + | FDD PCell | 8 | 8 |
| TDD 15 kHz CA | TDD PCell | 8 | 12 |
| FDD 30 kHz + | FDD PCell | 8 | 8 |
| TDD 30 kHz CA | TDD PCell | 8 | 16 |
| TDD 30 kHz + | TDD PCell | 8 | N/A |
| TDD 30 kHz CA |  |  |  |

#### < End of changes >